PCF Validator

Syntax Rule Book  v2.0

Validated Against Hexagon PCF Reference Guide v15

Reference: Alias Limited PCF Component Reference Guide + Official PCF Reference Guide v15

Document Version Status Badges

| Status Badge | Meaning |
| --- | --- |
| ✓ CONFIRMED | Rule validated against official Hexagon/Alias documentation. No change from v1.0 logic required. |
| ⚠ CORRECTED | Rule existed in v1.0 but contained a factual error. Corrected against official source. |
| ★ NEW | Rule not present in v1.0. Added based on official documentation research. |
| ◆ ALTERNATIVE | Ambiguous area where multiple valid patterns exist. Provide sample PCF to finalise. |

# 1.  Purpose, Scope, and Validation Basis

This document is the PCF Validator Syntax Rule Book v2.0. It defines all rules a PCF Validator program must enforce when checking Piping Component Files (PCF) before submission to Hexagon ISOGEN for isometric drawing generation. Every rule is stated in a form that translates directly to program logic: each rule has a unique ID, a processing pass, a severity level, unambiguous pseudocode, and a templated error message with {placeholder} tokens for actual file values.

Validation basis: This v2.0 has been validated against two official sources: (1) the Hexagon PCF Reference Guide Version 15 (docs.hexagonppm.com), and (2) the Alias Limited PCF Component Reference Guide Version 2 (the foundational specification by the format's creators). Discrepancies found between v1.0 rules and official documentation are marked ⚠ CORRECTED.

Critical corrections from v1.0: K-003 (CO-ORDS token count is context-dependent, not always 4), C-006 (OLET is NOT a break-point-only — it is a real component block with CENTRE-POINT+BRANCH1-POINT both having 4 values), K-002 (OLET CENTRE-POINT has 4 values; BEND/TEE CENTRE-POINT has 3 values), C-001 (component keyword list missing 20+ valid types).

# 2.  Program-Friendly Architecture

## 2.1  Precise Error Message Contract

Every error message template in this rule book uses the following contract. The validator program MUST substitute all {placeholder} tokens before displaying to the user:

| Placeholder | Meaning |
| --- | --- |
| {lineNo} | The exact 1-indexed line number in the file where the error was found or first triggered |
| {col} | Column number (1-indexed) for character-level errors |
| {kw} | The keyword token that caused the error |
| {comp} | The component type keyword of the enclosing block (e.g. PIPE, BEND, TEE) |
| {n} | A count value (e.g. number of tokens found) |
| {val} | An actual value found in the file |
| {found} | The actual text found on the line |
| {units} | The unit string from UNITS-CO-ORDS |
| {tol} | The connectivity tolerance for the declared unit system |
| {x},{y},{z} | Coordinate values from a parsed point |
| {gap} | Distance computed between two points |
| {missing} | Comma-separated list of missing attribute names |
| {ep},{cp},{b1p} | Count of END-POINTs, CENTRE-POINTs, BRANCH1-POINTs found |
| {first} | Line number of the first occurrence (for duplicate checks) |

## 2.2  Processing Passes (Ordered Execution)

Rules are executed in pass order. A FATAL in any pass stops subsequent passes from running on the affected block/file. Passes 1-3 are file-wide; Passes 4-8 operate per component block.

| Pass | Name | Rules | Scope | What It Checks | Gate Condition |
| --- | --- | --- | --- | --- | --- |
| 1 | File Scan | F-001 – F-004 | Raw bytes | Empty file, TABs, non-ASCII, line endings | FATAL F-001 stops all processing |
| 2 | Lexical Scan | L-001 – L-004 | Tokenised lines | Column positions, indentation, unknown keywords | None |
| 3 | Header Parse | H-001 – H-009 | Lines before first component kw | Mandatory header keywords, values, duplicates | FATAL H-001/H-003/H-005/H-007 stop component parsing |
| 4 | Block Splitting | C-001 | Column-1 keywords | Identify component type keywords, split file into blocks | FATAL C-001 on a block skips that block |
| 5 | Block Structure | C-002 – C-013 | Per component block | Keypoint counts, correct coord sets per component type | Errors logged; topology pass still attempted |
| 6 | Coordinate Parse | K-001 – K-006 | Coord lines in blocks | Token counts, numeric validity, OD sanity | Errors prevent topology pass for affected block |
| 7 | Angle & Units | A-001, A-002, U-001, U-002 | ANGLE lines, UNITS header | Angle encoding, unit system consistency | U-001 is high priority; logs regardless |
| 8 | Topology | T-001, T-002 | All parsed end-points | Connectivity graph, gap checking, dead ends | Requires Pass 6 success |
| 9 | Attributes | R-001 – R-008, S-001 | Attribute lines | Unknown attrs, SKEY, BEND-RADIUS, CATEGORY, ADDITIONAL-ITEM, REDUCER ODs | Can run in parallel with Pass 6 |
| 10 | Alternative | VALV-*, BRANCH-* | Specific component types | Valve port counts, olet geometry, branch selection | Depends on Pass 5 |
| 11 | Completeness | X-001, X-002 | Full file | Component count checks | Always runs last |

## 2.3  Internal Data Model (Required by Program)

PCFFile

  .raw_lines[]           # Original text, 1-indexed

  .header                # HeaderSection

      .isogen_fls        # string: FLS filename

      .units_bore        # "MM" | "INCH" | "MM-HUNDREDTHS" | "METER"

      .units_coords      # "MM" | "INCH" | "MM-HUNDREDTHS" | "METER"

      .pipeline_ref      # string: pipeline tag

  .components[]          # list[ComponentBlock]

      .type              # string: "PIPE" | "BEND" | "VALVE" etc.

      .start_line        # int: line number of component keyword

      .end_line          # int: line number of last attribute in block

      .attrs{}           # dict: kw → list[{values:[], line_no:int}]

      .end_points[]      # list[CoordPoint]

      .centre_points[]   # list[CoordPoint]

      .branch_points[]   # list[CoordPoint]

      .co_ords[]         # list[CoordPoint]

  .standalone_entries[]  # BOLT, END-CONNECTION-*, MESSAGE-*, ADDITIONAL-ITEM

  .issues[]              # list[ValidationIssue]

      .rule_id           # "K-001", "VALV-RULE1", etc.

      .severity          # "FATAL"|"ERROR"|"WARNING"|"INFO"

      .line_no           # int

      .col_no            # int | None

      .message           # string (fully interpolated)

      .comp_type         # string: which component block triggered this



CoordPoint

  .x, .y, .z            # float

  .od                    # float | None  (None for 3-value CO-ORDS)

  .end_treatment         # str | None  ("BW","FL","SW",... optional 5th token)
  .line_no               # int

# 3.  Rule Summary (All Rules)

| Rule ID | Cat | Rule Name | Sev | Status |
| --- | --- | --- | --- | --- |
| F-001 | File | File Must Not Be Empty | F | ✓ CONFIRMED |
| F-002 | File | No TAB Characters Permitted Anywhere | F | ✓ CONFIRMED |
| F-003 | File | Non-ASCII Characters | W | ✓ CONFIRMED |
| F-004 | File | CRLF Line Endings Recommended on Windows | I | ✓ CONFIRMED |
| L-001 | Line | Component/Header Keyword Must Start at Column 1 | E | ✓ CONFIRMED |
| L-002 | Line | Attribute Keyword Must Be Indented (Not at Column 1) | E | ✓ CONFIRMED |
| L-003 | Line | Indentation Should Be Exactly 4 Spaces | I | ✓ CONFIRMED |
| L-004 | Line | Unrecognised Column-1 Keyword | W | ★ NEW |
| H-001 | Header | ISOGEN-FILES Must Be the First Content Line | F | ✓ CONFIRMED |
| H-002 | Header | ISOGEN-FILES Must Have a Filename Value | F | ✓ CONFIRMED |
| H-003 | Header | UNITS-BORE Must Be Present in Header | F | ✓ CONFIRMED |
| H-004 | Header | UNITS-BORE Value Must Be Valid | F | ✓ CONFIRMED |
| H-005 | Header | UNITS-CO-ORDS Must Be Present in Header | F | ✓ CONFIRMED |
| H-006 | Header | UNITS-CO-ORDS Value Must Be Valid | F | ✓ CONFIRMED |
| H-007 | Header | PIPELINE-REFERENCE Must Be Present | F | ✓ CONFIRMED |
| H-008 | Header | PIPELINE-REFERENCE Must Have a Value | E | ✓ CONFIRMED |
| H-009 | Header | Duplicate Mandatory Header Keywords | W | ✓ CONFIRMED |
| C-001 | Component | Component Keyword Must Be in Recognised List | F | ⚠ CORRECTED |
| C-002 | Component | Standard 2-EP Component Must Have Exactly Two END-POINTs | E | ✓ CONFIRMED |
| C-003 | Component | BEND Requires 2 END-POINTs, 1 CENTRE-POINT | E | ✓ CONFIRMED |
| C-004 | Component | TEE Requires 2 END-POINTs, 1 CENTRE-POINT, 1 BRANCH1-POINT | E | ✓ CONFIRMED |
| C-005 | Component | TEE-SET-ON Requires 2 END-POINTs, 1 CENTRE-POINT, 1 BRANCH1-POINT (same as TEE) | E | ★ NEW |
| C-006 | Component | OLET Requires CENTRE-POINT and BRANCH1-POINT (Both With 4 Values) | E | ⚠ CORRECTED |
| C-007 | Component | SUPPORT Must Use CO-ORDS, Not END-POINT | E | ✓ CONFIRMED |
| C-008 | Component | SUPPORT Should Have SKEY Attribute | W | ✓ CONFIRMED |
| C-009 | Component | BEND Should Have SKEY, ANGLE, and BEND-RADIUS | W | ✓ CONFIRMED |
| C-010 | Component | Attribute Lines Must Not Appear Outside a Component Block | E | ✓ CONFIRMED |
| C-011 | Component | FLANGE-BLIND Requires Exactly One END-POINT | E | ★ NEW |
| C-012 | Component | WELD Requires Exactly Two Identical END-POINTs | E | ★ NEW |
| C-013 | Component | BOLT Standalone Entry Must Have CO-ORDS with 3 Values | E | ★ NEW |
| VALV-RULE1 | Valve | VALVE (Standard) — 2 END-POINTs Required | E | ◆ ALTERNATIVE |
| VALV-RULE2 | Valve | VALVE-3WAY — 3 END-POINTs Required | E | ◆ ALTERNATIVE |
| VALV-RULE3 | Valve | VALVE-4WAY — 4 END-POINTs Required | E | ◆ ALTERNATIVE |
| VALV-RULE4 | Valve | VALVE SPINDLE-DIRECTION Attribute Recommended | I | ◆ ALTERNATIVE |
| BRANCH-RULE1 | Branch | Branch-off: TEE vs TEE-SET-ON vs OLET — Usage Guidance | I | ◆ ALTERNATIVE |
| BRANCH-RULE2 | Branch | OLET CENTRE-POINT Must Lie on Main Pipe Centreline | W | ◆ ALTERNATIVE |
| BRANCH-RULE3 | Branch | OLET Welds Should Be Present When OLET Used | W | ◆ ALTERNATIVE |
| K-001 | Coordinate | END-POINT Must Have Exactly 4 Numeric Values | E | ✓ CONFIRMED |
| K-002 | Coordinate | CENTRE-POINT for BEND/TEE/TEE-SET-ON Must Have 3 Numeric Values Only | E | ✓ CONFIRMED |
| K-003a | Coordinate | CO-ORDS for SUPPORT Must Have 4 Numeric Values | E | ⚠ CORRECTED |
| K-003b | Coordinate | CO-ORDS for BOLT/END-CONNECTION/END-POSITION Must Have 3 Numeric Values | E | ★ NEW |
| K-003c | Coordinate | OLET/ELBOLET CENTRE-POINT Has 4 Numeric Values (Includes OD) | ? | ★ NEW |
| K-004 | Coordinate | BRANCH1-POINT Must Have 4 Numeric Values | E | ✓ CONFIRMED |
| K-005 | Coordinate | All Coordinate Values Must Parse as Floating-Point Numbers | E | ✓ CONFIRMED |
| K-006 | Coordinate | OD Value Must Be Positive | W | ✓ CONFIRMED |
| A-001 | Angle | ANGLE Value Is in Hundredths of Degrees | E | ✓ CONFIRMED |
| A-002 | Angle | Declared ANGLE Should Match Geometry | W | ✓ CONFIRMED |
| U-001 | Units | Bore and Coordinate Units Must Be in Same System | E | ✓ CONFIRMED |
| U-002 | Units | OD Values Should Be Consistent With Declared Bore Units | W | ✓ CONFIRMED |
| T-001 | Topology | All Component End-Points Must Connect Within Tolerance | E | ✓ CONFIRMED |
| T-002 | Topology | Dead-End Points Should Be Intentionally Terminated | W | ✓ CONFIRMED |
| R-001 | Attribute | Unrecognised Attribute Keyword | W | ⚠ CORRECTED |
| R-002 | Attribute | SKEY Value Must Not Be Empty | W | ✓ CONFIRMED |
| R-003 | Attribute | SKEY Value Format Recommendation | I | ✓ CONFIRMED |
| R-004 | Attribute | BEND-RADIUS Must Be Positive | W | ✓ CONFIRMED |
| R-005 | Attribute | Duplicate Single-Instance Attribute in Same Block | W | ✓ CONFIRMED |
| R-006 | Attribute | CENTRE-POINT for BEND/TEE Must Not Have 4 Values | E | ✓ CONFIRMED |
| R-007 | Attribute | CATEGORY Attribute Must Have a Valid Value | ? | ★ NEW |
| R-008 | Attribute | ADDITIONAL-ITEM Attribute Column Position | ? | ★ NEW |
| S-001 | Specific | REDUCER Must Have Two Different OD Values | E | ✓ CONFIRMED |
| X-001 | Completeness | File Has No Component Blocks | I | ✓ CONFIRMED |
| X-002 | Completeness | Pipeline Has Only One Component | W | ✓ CONFIRMED |

# 4.  Detailed Rule Definitions

## File Rules

| F-001 File Must Not Be Empty FATAL ✓ CONFIRMED |  |
| --- | --- |
| Applies To | Entire file |
| Trigger | After stripping blank lines and comment lines (lines starting with semicolon), zero content lines remain. |
| Check Logic | content_lines = [l for l in file.lines if l.strip() and not l.strip().startswith(";")] IF len(content_lines) == 0: FATAL |
| Error Message Template | F-001: FILE IS EMPTY. No content found after stripping blank lines. Provide at least ISOGEN-FILES, UNITS-BORE, UNITS-CO-ORDS, PIPELINE-REFERENCE and one component block. |
| Correction Hint | Add required header and at least one component block. |
| ❌ Bad (Fails) | (empty or only blank/comment lines) |
| ✅ Good (Passes) | ISOGEN-FILES ISOGEN.FLS UNITS-BORE MM ... |

| F-002 No TAB Characters Permitted Anywhere FATAL ✓ CONFIRMED |  |
| --- | --- |
| Applies To | Every byte in file |
| Trigger | Any line contains a TAB character (0x09). |
| Check Logic | FOR lineNo, line IN enumerate(raw_lines, 1): IF "\t" IN line: FATAL at lineNo |
| Error Message Template | F-002: Line {lineNo} — TAB CHARACTER found at column {col}. PCF uses spaces only. TABs cause ISOGEN to silently skip keywords. Replace all TABs with spaces. |
| Correction Hint | Use editor "Convert Tabs to Spaces". Search for 0x09 byte. |
| ❌ Bad (Fails) | PIPE END-POINT 100.0 200.0 300.0 50.0 |
| ✅ Good (Passes) | PIPE END-POINT 100.0 200.0 300.0 50.0 |

| F-003 Non-ASCII Characters WARNING ✓ CONFIRMED |  |
| --- | --- |
| Applies To | All characters |
| Trigger | Any character code > 127. |
| Check Logic | FOR lineNo, line IN enumerate(raw_lines, 1): FOR col, ch IN enumerate(line, 1): IF ord(ch) > 127: WARN at lineNo, col |
| Error Message Template | F-003: Line {lineNo}, Column {col} — NON-ASCII CHARACTER "{ch}" (code {code}). PCF is plain ASCII. Smart quotes, em-dashes, accented letters may cause parse failures. |
| Correction Hint | Replace with plain ASCII equivalents. |
| ❌ Bad (Fails) | PIPELINE-REFERENCE SYS‑1 |
| ✅ Good (Passes) | PIPELINE-REFERENCE SYS-1 |

| F-004 CRLF Line Endings Recommended on Windows INFO ✓ CONFIRMED |  |
| --- | --- |
| Applies To | File encoding |
| Trigger | File uses LF-only endings and environment is Windows. |
| Check Logic | IF b"\r\n" NOT IN file_bytes AND b"\n" IN file_bytes: INFO |
| Error Message Template | F-004: FILE uses LF-only (Unix) line endings. On Windows ISOGEN expects CRLF. Convert if you encounter loading issues. |
| Correction Hint | Convert using Notepad++ → Edit → EOL Conversion → Windows (CRLF). |

## Line Rules

| L-001 Component/Header Keyword Must Start at Column 1 ERROR ✓ CONFIRMED |  |
| --- | --- |
| Applies To | All non-blank, non-comment lines |
| Trigger | A known component-type keyword or file-header keyword appears with leading spaces. |
| Check Logic | FOR lineNo, line IN enumerate(lines, 1): stripped = line.lstrip() kw = stripped.split()[0] if stripped.split() else "" IF kw IN ALL_COMPONENT_KEYWORDS OR kw IN HEADER_KEYWORDS: IF line[0] == " ": ERROR at lineNo |
| Error Message Template | L-001: Line {lineNo} — KEYWORD "{kw}" has leading spaces. Component and header keywords MUST start at column 1 (no indentation). Found {n} leading spaces. |
| Correction Hint | Delete all leading spaces from this line. |
| ❌ Bad (Fails) | PIPE END-POINT 100.0 200.0 300.0 50.0 |
| ✅ Good (Passes) | PIPE END-POINT 100.0 200.0 300.0 50.0 |

| L-002 Attribute Keyword Must Be Indented (Not at Column 1) ERROR ✓ CONFIRMED |  |
| --- | --- |
| Applies To | All attribute-level keywords |
| Trigger | A known attribute keyword appears at column 1. |
| Check Logic | FOR lineNo, line IN enumerate(lines, 1): IF line[0] not in (" ",";","\n"): kw = line.split()[0] IF kw IN ATTRIBUTE_KEYWORDS: ERROR at lineNo |
| Error Message Template | L-002: Line {lineNo} — ATTRIBUTE "{kw}" appears at column 1. Attribute keywords must be indented (minimum 4 spaces). Did you mean to add a component type keyword above this line? |
| Correction Hint | Indent attribute by 4+ spaces, OR add missing component type keyword above. |
| ❌ Bad (Fails) | END-POINT 17989.4 -96400.0 101968.0 406.4 |
| ✅ Good (Passes) | END-POINT 17989.4 -96400.0 101968.0 406.4 |

| L-003 Indentation Should Be Exactly 4 Spaces INFO ✓ CONFIRMED |  |
| --- | --- |
| Applies To | All attribute lines |
| Trigger | Attribute line has fewer than 4 or more than 8 leading spaces. |
| Check Logic | FOR each attribute_line: indent = len(line) - len(line.lstrip()) IF indent < 4 OR indent > 8: INFO |
| Error Message Template | L-003: Line {lineNo} — Attribute "{kw}" has {n}-space indent. Standard is 4 spaces. Non-standard indentation is accepted by ISOGEN but reduces readability. |
| Correction Hint | Use exactly 4 spaces for all attribute lines. |

| L-004 Unrecognised Column-1 Keyword WARNING ★ NEW |  |
| --- | --- |
| Applies To | Lines where first non-space char is at column 1 and it is not blank or comment |
| Trigger | A word at column 1 is not in the recognised component keyword list AND not in the header keyword list AND not a known standalone-entry keyword. |
| Check Logic | KNOWN_COL1 = ALL_COMPONENT_KEYWORDS ¦ HEADER_KEYWORDS ¦ STANDALONE_KEYWORDS IF line[0] not in (" ",";","\n"): kw = line.split()[0] IF kw NOT IN KNOWN_COL1: WARN at lineNo |
| Error Message Template | L-004: Line {lineNo} — UNRECOGNISED COLUMN-1 KEYWORD "{kw}". This keyword is unknown. Possible typo or unsupported component type. ISOGEN may silently skip this block. |
| Correction Hint | Check spelling against Appendix A component keyword list. If intentional, verify your ISOGEN version supports this keyword. |

## Header Rules

| H-001 ISOGEN-FILES Must Be the First Content Line FATAL ✓ CONFIRMED |  |
| --- | --- |
| Applies To | Line 1 (ignoring blank/comment lines) |
| Trigger | First non-blank/non-comment line does not start with "ISOGEN-FILES". |
| Check Logic | first = next(l for l in lines if l.strip() and not l.strip().startswith(";")) IF NOT first.strip().startswith("ISOGEN-FILES"): FATAL |
| Error Message Template | H-001: Line {lineNo} — ISOGEN-FILES IS MISSING OR NOT FIRST. First content line must be: ISOGEN-FILES <filename.fls>. Found: "{found}" instead. ISOGEN cannot locate its configuration without this. |
| Correction Hint | Make ISOGEN-FILES the absolute first content line of the file. |
| ❌ Bad (Fails) | UNITS-BORE MM ISOGEN-FILES ISOGEN.FLS |
| ✅ Good (Passes) | ISOGEN-FILES ISOGEN.FLS UNITS-BORE MM |

| H-002 ISOGEN-FILES Must Have a Filename Value FATAL ✓ CONFIRMED |  |
| --- | --- |
| Applies To | ISOGEN-FILES line |
| Trigger | ISOGEN-FILES keyword present but no second token follows it. |
| Check Logic | tokens = line.split() IF len(tokens) < 2: FATAL |
| Error Message Template | H-002: Line {lineNo} — ISOGEN-FILES HAS NO FILENAME. Expected: ISOGEN-FILES ISOGEN.FLS (or your .fls filename). The FLS file tells ISOGEN where all its control files are located. |
| Correction Hint | Add the FLS filename: ISOGEN-FILES ISOGEN.FLS |
| ❌ Bad (Fails) | ISOGEN-FILES |
| ✅ Good (Passes) | ISOGEN-FILES ISOGEN.FLS |

| H-003 UNITS-BORE Must Be Present in Header FATAL ✓ CONFIRMED |  |
| --- | --- |
| Applies To | Header section (all lines before first component keyword) |
| Trigger | "UNITS-BORE" not found anywhere in header section. |
| Check Logic | IF "UNITS-BORE" NOT IN header_keywords: FATAL |
| Error Message Template | H-003: UNITS-BORE IS MISSING. This mandatory header keyword is absent. Without it ISOGEN cannot determine bore units (ISOGEN error 52 / 47). |
| Correction Hint | Add to header: UNITS-BORE MM (or INCH) |
| ❌ Bad (Fails) | ISOGEN-FILES ISOGEN.FLS UNITS-CO-ORDS MM |
| ✅ Good (Passes) | ISOGEN-FILES ISOGEN.FLS UNITS-BORE MM UNITS-CO-ORDS MM |

| H-004 UNITS-BORE Value Must Be Valid FATAL ✓ CONFIRMED |  |
| --- | --- |
| Applies To | UNITS-BORE line |
| Trigger | Value token after UNITS-BORE is not one of the four permitted strings. |
| Check Logic | VALID = {"MM","INCH","MM-HUNDREDTHS","METER"} IF tokens[1].upper() NOT IN VALID: FATAL |
| Error Message Template | H-004: Line {lineNo} — UNITS-BORE VALUE "{val}" IS INVALID. Permitted values: MM, INCH, MM-HUNDREDTHS, METER. |
| Correction Hint | Change to one of: UNITS-BORE MM UNITS-BORE INCH |
| ❌ Bad (Fails) | UNITS-BORE MILLIMETERS |
| ✅ Good (Passes) | UNITS-BORE MM |

| H-005 UNITS-CO-ORDS Must Be Present in Header FATAL ✓ CONFIRMED |  |
| --- | --- |
| Applies To | Header section |
| Trigger | "UNITS-CO-ORDS" not found in header section. |
| Check Logic | IF "UNITS-CO-ORDS" NOT IN header_keywords: FATAL |
| Error Message Template | H-005: UNITS-CO-ORDS IS MISSING. Without this ISOGEN cannot interpret coordinate values in END-POINT, CENTRE-POINT, CO-ORDS lines. |
| Correction Hint | Add: UNITS-CO-ORDS MM (or INCH) |
| ❌ Bad (Fails) | ISOGEN-FILES ISOGEN.FLS UNITS-BORE MM |
| ✅ Good (Passes) | ISOGEN-FILES ISOGEN.FLS UNITS-BORE MM UNITS-CO-ORDS MM |

| H-006 UNITS-CO-ORDS Value Must Be Valid FATAL ✓ CONFIRMED |  |
| --- | --- |
| Applies To | UNITS-CO-ORDS line |
| Trigger | Value is not MM, INCH, MM-HUNDREDTHS, or METER. |
| Check Logic | VALID = {"MM","INCH","MM-HUNDREDTHS","METER"} IF tokens[1].upper() NOT IN VALID: FATAL |
| Error Message Template | H-006: Line {lineNo} — UNITS-CO-ORDS VALUE "{val}" IS INVALID. Permitted: MM, INCH, MM-HUNDREDTHS, METER. |
| Correction Hint | Use: UNITS-CO-ORDS MM UNITS-CO-ORDS INCH |
| ❌ Bad (Fails) | UNITS-CO-ORDS MILLIMETERS |
| ✅ Good (Passes) | UNITS-CO-ORDS MM |

| H-007 PIPELINE-REFERENCE Must Be Present FATAL ✓ CONFIRMED |  |
| --- | --- |
| Applies To | All lines before first component block |
| Trigger | "PIPELINE-REFERENCE" not found before first component keyword. |
| Check Logic | pre_component = lines_before_first_component_keyword() IF "PIPELINE-REFERENCE" NOT IN {l.split()[0] for l in pre_component}: FATAL |
| Error Message Template | H-007: PIPELINE-REFERENCE IS MISSING. Every PCF must identify the pipeline it represents. Without this ISOGEN cannot label the isometric. |
| Correction Hint | Add: PIPELINE-REFERENCE 6\"-W-100 (use your line number tag) |
| ❌ Bad (Fails) | ISOGEN-FILES ISOGEN.FLS UNITS-BORE MM UNITS-CO-ORDS MM PIPE END-POINT... |
| ✅ Good (Passes) | ISOGEN-FILES ISOGEN.FLS UNITS-BORE MM UNITS-CO-ORDS MM PIPELINE-REFERENCE 6\"-W-100 PIPE END-POINT... |

| H-008 PIPELINE-REFERENCE Must Have a Value ERROR ✓ CONFIRMED |  |
| --- | --- |
| Applies To | PIPELINE-REFERENCE line |
| Trigger | Line has keyword but no second token. |
| Check Logic | IF len(tokens) < 2: ERROR |
| Error Message Template | H-008: Line {lineNo} — PIPELINE-REFERENCE HAS NO VALUE. Provide the pipeline tag/number. |
| Correction Hint | Example: PIPELINE-REFERENCE 6\"-W-100-CS1A |
| ❌ Bad (Fails) | PIPELINE-REFERENCE |
| ✅ Good (Passes) | PIPELINE-REFERENCE 6\"-W-100 |

| H-009 Duplicate Mandatory Header Keywords WARNING ✓ CONFIRMED |  |
| --- | --- |
| Applies To | Header section |
| Trigger | Any of ISOGEN-FILES, UNITS-BORE, UNITS-CO-ORDS, PIPELINE-REFERENCE appears more than once. |
| Check Logic | FOR kw IN MANDATORY_HEADER_KWS: IF count_in_header(kw) > 1: WARN |
| Error Message Template | H-009: Line {lineNo} — DUPLICATE HEADER KEYWORD "{kw}" (first at line {first}). ISOGEN uses the LAST occurrence. Verify which value is correct. |
| Correction Hint | Remove the duplicate. Keep only the intended value. |
| ❌ Bad (Fails) | UNITS-BORE MM UNITS-BORE INCH |
| ✅ Good (Passes) | UNITS-BORE MM |

## Component Rules

| C-001 Component Keyword Must Be in Recognised List FATAL ⚠ CORRECTED |  |
| --- | --- |
| Applies To | All column-1 words that are not header keywords or standalone-entry keywords |
| Trigger | A column-1 keyword is not in the validated component keyword list (v2.0 — expanded from official PCF Component Reference Guide). |
| Check Logic | VALID_COMPONENTS = { "PIPE","PIPE-FIXED","ELBOW","ELBOW-TEED","TEE","TEE-SET-ON", "BEND","BEND-TEED","REDUCER-CONCENTRIC","REDUCER-ECCENTRIC", "FLANGE","FLANGE-BLIND","GASKET","SUPPORT","VALVE","VALVE-3WAY", "VALVE-4WAY","VALVE-ANGLE","CAP","STUB-IN","OLET","ELBOLET", "INSTRUMENT","INSTRUMENT-3WAY","INSTRUMENT-4WAY","INSTRUMENT-ANGLE", "INSTRUMENT-DIAL","INSTRUMENT-OFFSET","INSTRUMENT-RETURN", "COUPLING","UNION","CROSS","Y-PIECE-FITTING","WELD","BOLT", "NOZZLE","LAPJOINT-RING","FILTER","FILTER-ANGLE","FILTER-OFFSET", "FILTER-RETURN","TRAP","TRAP-OFFSET","TRAP-RETURN","REINFORCEMENT-PAD", "SAFETY-DISC","MISC-COMPONENT","MISC-COMPONENT-RETURN", "MULTI-PORT-COMPONENT","DUCT","SPOOL-BREAK","JOINT","VENT-DRAIN", "SPECIAL-ITEM-1","SPECIAL-ITEM-2","SPECIAL-ITEM-3","SPECIAL-ITEM-4", "SPECIAL-ITEM-5","SPOOL-STATUS" } IF kw NOT IN VALID_COMPONENTS: FATAL |
| Error Message Template | C-001: Line {lineNo} — "{kw}" IS NOT A RECOGNISED PCF COMPONENT KEYWORD. ISOGEN will reject or silently skip this block. Check spelling. See Appendix A for the full list. |
| Correction Hint | Replace with the correct keyword from the PCF component keyword list. |
| v1.0 Change | v1.0 list was missing: BOLT, ELBOLET, ELBOW-TEED, BEND-TEED, FLANGE-BLIND, VALVE-3WAY, VALVE-4WAY, VALVE-ANGLE, INSTRUMENT variants, FILTER variants, TRAP variants, REINFORCEMENT-PAD, SAFETY-DISC, MISC-COMPONENT, LAPJOINT-RING, Y-PIECE-FITTING, MULTI-PORT-COMPONENT. |
| ❌ Bad (Fails) | PIPE-FITTING END-POINT ... |
| ✅ Good (Passes) | ELBOW END-POINT ... |

| C-002 Standard 2-EP Component Must Have Exactly Two END-POINTs ERROR ✓ CONFIRMED |  |
| --- | --- |
| Applies To | PIPE, PIPE-FIXED, ELBOW, FLANGE, GASKET, VALVE, VALVE-ANGLE, CAP, COUPLING, UNION, REDUCER-CONCENTRIC, REDUCER-ECCENTRIC, WELD, LAPJOINT-RING |
| Trigger | Block has a component type from the 2-EP list but does not have exactly 2 END-POINT lines. |
| Check Logic | TWO_EP = {"PIPE","PIPE-FIXED","ELBOW","FLANGE","GASKET","VALVE","VALVE-ANGLE","CAP","COUPLING","UNION","REDUCER-CONCENTRIC","REDUCER-ECCENTRIC","WELD","LAPJOINT-RING","STUB-IN","CROSS","FILTER","TRAP","SAFETY-DISC","MISC-COMPONENT"} IF comp_type IN TWO_EP AND count("END-POINT",block) != 2: ERROR |
| Error Message Template | C-002: Line {lineNo} — {comp} HAS {n} END-POINT(s); EXACTLY 2 REQUIRED. Each END-POINT needs 4 values: X Y Z OD. |
| Correction Hint | Add or remove END-POINT lines so exactly 2 are present. |
| ❌ Bad (Fails) | PIPE END-POINT 100.0 200.0 300.0 50.0 |
| ✅ Good (Passes) | PIPE END-POINT 100.0 200.0 300.0 50.0 END-POINT 500.0 200.0 300.0 50.0 |

| C-003 BEND Requires 2 END-POINTs, 1 CENTRE-POINT ERROR ✓ CONFIRMED |  |
| --- | --- |
| Applies To | BEND and ELBOW blocks (unless ELBOW-TEED — see C-005) |
| Trigger | BEND/ELBOW does not have exactly 2 END-POINT and exactly 1 CENTRE-POINT. |
| Check Logic | IF comp_type IN {"BEND","ELBOW"}: IF count("END-POINT") != 2 OR count("CENTRE-POINT") != 1: ERROR |
| Error Message Template | C-003: Line {lineNo} — {comp} REQUIRES 2 END-POINTs AND 1 CENTRE-POINT. Found {ep} END-POINT(s) and {cp} CENTRE-POINT(s). CENTRE-POINT locates the arc centre and has 3 values only (no OD). |
| Correction Hint | Add END-POINT (4 values each) and CENTRE-POINT (3 values, no OD). |
| ❌ Bad (Fails) | BEND END-POINT 100.0 200.0 300.0 50.0 SKEY BEBW |
| ✅ Good (Passes) | BEND END-POINT 100.0 200.0 300.0 50.0 END-POINT 100.0 400.0 300.0 50.0 CENTRE-POINT 100.0 300.0 300.0 SKEY BEBW ANGLE 9000 BEND-RADIUS 609.6000 |

| C-004 TEE Requires 2 END-POINTs, 1 CENTRE-POINT, 1 BRANCH1-POINT ERROR ✓ CONFIRMED |  |
| --- | --- |
| Applies To | TEE component blocks |
| Trigger | TEE missing any required keypoint. |
| Check Logic | IF comp_type == "TEE": IF count("END-POINT")!=2 OR count("CENTRE-POINT")!=1 OR count("BRANCH1-POINT")!=1: ERROR |
| Error Message Template | C-004: Line {lineNo} — TEE REQUIRES 2 END-POINTs + 1 CENTRE-POINT + 1 BRANCH1-POINT. Found EP={ep}, CP={cp}, B1P={b1p}. CENTRE-POINT = 3 values (no OD). END-POINT & BRANCH1-POINT = 4 values each. |
| Correction Hint | Add all missing keypoints. Note: CENTRE-POINT has NO OD value. |
| ❌ Bad (Fails) | TEE END-POINT 100.0 200.0 300.0 50.0 END-POINT 300.0 200.0 300.0 50.0 SKEY TEBW |
| ✅ Good (Passes) | TEE END-POINT 100.0 200.0 300.0 50.0 END-POINT 300.0 200.0 300.0 50.0 CENTRE-POINT 200.0 200.0 300.0 BRANCH1-POINT 200.0 200.0 400.0 40.0 SKEY TEBW |

| C-005 TEE-SET-ON Requires 2 END-POINTs, 1 CENTRE-POINT, 1 BRANCH1-POINT (same as TEE) ERROR ★ NEW |  |
| --- | --- |
| Applies To | TEE-SET-ON blocks |
| Trigger | TEE-SET-ON missing required keypoints. |
| Check Logic | IF comp_type == "TEE-SET-ON": IF count("END-POINT")!=2 OR count("CENTRE-POINT")!=1 OR count("BRANCH1-POINT")!=1: ERROR |
| Error Message Template | C-005: Line {lineNo} — TEE-SET-ON REQUIRES 2 END-POINTs + 1 CENTRE-POINT + 1 BRANCH1-POINT. Found EP={ep}, CP={cp}, B1P={b1p}. TEE-SET-ON is a welded branch connection; structure identical to TEE. |
| Correction Hint | Provide all four keypoints. CENTRE-POINT = 3 values, BRANCH1-POINT = 4 values. |
| ❌ Bad (Fails) | TEE-SET-ON END-POINT 100.0 200.0 300.0 50.0 |
| ✅ Good (Passes) | TEE-SET-ON END-POINT 100.0 200.0 300.0 50.0 END-POINT 300.0 200.0 300.0 50.0 CENTRE-POINT 200.0 200.0 300.0 BRANCH1-POINT 200.0 200.0 400.0 40.0 SKEY TSML |

| C-006 OLET Requires CENTRE-POINT and BRANCH1-POINT (Both With 4 Values) ERROR ⚠ CORRECTED |  |
| --- | --- |
| Applies To | OLET and ELBOLET component blocks |
| Trigger | OLET block does not have exactly 1 CENTRE-POINT and 1 BRANCH1-POINT. |
| Check Logic | IF comp_type IN {"OLET","ELBOLET"}: IF count("CENTRE-POINT")!=1 OR count("BRANCH1-POINT")!=1: ERROR IF count("END-POINT") > 0: WARN (END-POINT not expected on OLET) |
| Error Message Template | C-006: Line {lineNo} — {comp} REQUIRES 1 CENTRE-POINT AND 1 BRANCH1-POINT (both with 4 values: X Y Z OD). OLET has NO END-POINT lines. Found CP={cp}, B1P={b1p}, EP={ep}. |
| Correction Hint | Provide CENTRE-POINT (4 values) at the main pipe centreline, BRANCH1-POINT (4 values) at the branch outlet. Remove any END-POINT lines. |
| v1.0 Change | CRITICAL CORRECTION: v1.0 incorrectly stated OLET = break point only with no PCF block. OLET IS a real component block with CENTRE-POINT + BRANCH1-POINT, both having 4 values including OD. The CENTRE-POINT for OLET includes OD (unlike TEE/BEND CENTRE-POINT which has 3 values only). |
| ❌ Bad (Fails) | OLET END-POINT 330500.0 6400.0 -2097.0 3.0 SKEY SKSW |
| ✅ Good (Passes) | OLET CENTRE-POINT 330500.0 6400.0 -2097.0 3.0 BRANCH1-POINT 330500.0 6647.6 -2097.0 3.0 SKEY SKSW CATEGORY FABRICATION |

| C-007 SUPPORT Must Use CO-ORDS, Not END-POINT ERROR ✓ CONFIRMED |  |
| --- | --- |
| Applies To | SUPPORT component blocks |
| Trigger | SUPPORT block contains END-POINT or has no CO-ORDS. |
| Check Logic | IF comp_type == "SUPPORT": IF count("CO-ORDS") == 0: ERROR IF count("END-POINT") > 0: ERROR |
| Error Message Template | C-007: Line {lineNo} — SUPPORT MUST USE CO-ORDS, NOT END-POINT. CO-ORDS takes 4 values: X Y Z OD. Using END-POINT on a SUPPORT is the most common PCF error in practice. |
| Correction Hint | Replace END-POINT with CO-ORDS. Verify 4 values are present. |
| ❌ Bad (Fails) | SUPPORT END-POINT 17989.4 -96400.0 101968.0 406.4 SKEY ANCH |
| ✅ Good (Passes) | SUPPORT CO-ORDS 17989.4 -96400.0 101968.0 406.4 SKEY ANCH |

| C-008 SUPPORT Should Have SKEY Attribute WARNING ✓ CONFIRMED |  |
| --- | --- |
| Applies To | SUPPORT blocks |
| Trigger | SUPPORT block has no SKEY. |
| Check Logic | IF comp_type=="SUPPORT" AND "SKEY" NOT IN block_attrs: WARN |
| Error Message Template | C-008: Line {lineNo} — SUPPORT HAS NO SKEY. ISOGEN will use a default support symbol. Recommended: SKEY ANCH for anchor. Verify intended symbol. |
| Correction Hint | Add SKEY with appropriate value (e.g. ANCH, GUID, SPRG). |

| C-009 BEND Should Have SKEY, ANGLE, and BEND-RADIUS WARNING ✓ CONFIRMED |  |
| --- | --- |
| Applies To | BEND blocks |
| Trigger | BEND block missing SKEY, ANGLE, or BEND-RADIUS. |
| Check Logic | missing=[] IF "SKEY" NOT IN block: missing.append("SKEY") IF "ANGLE" NOT IN block: missing.append("ANGLE") IF "BEND-RADIUS" NOT IN block: missing.append("BEND-RADIUS") IF missing: WARN |
| Error Message Template | C-009: Line {lineNo} — BEND IS MISSING: {missing}. Without ANGLE the bend degrees will be wrong. Without BEND-RADIUS the centerline dimensions are wrong. |
| Correction Hint | Add: SKEY BEBW ANGLE 9000 (=90°) BEND-RADIUS <mm> |

| C-010 Attribute Lines Must Not Appear Outside a Component Block ERROR ✓ CONFIRMED |  |
| --- | --- |
| Applies To | Lines between last header keyword and first component keyword |
| Trigger | An attribute keyword appears before any component keyword. |
| Check Logic | IF state=="header" AND line.lstrip().split()[0] IN ATTRIBUTE_KEYWORDS: ERROR |
| Error Message Template | C-010: Line {lineNo} — ATTRIBUTE "{kw}" FOUND OUTSIDE ANY COMPONENT BLOCK. Attributes must follow a component type keyword. Check for a missing component keyword above this line. |
| Correction Hint | Add the missing component keyword (e.g. PIPE) on the line above. |

| C-011 FLANGE-BLIND Requires Exactly One END-POINT ERROR ★ NEW |  |
| --- | --- |
| Applies To | FLANGE-BLIND blocks |
| Trigger | FLANGE-BLIND has 0 or more than 1 END-POINT. |
| Check Logic | IF comp_type=="FLANGE-BLIND": IF count("END-POINT") != 1: ERROR |
| Error Message Template | C-011: Line {lineNo} — FLANGE-BLIND REQUIRES EXACTLY 1 END-POINT (it is a terminal/cap fitting). Found {n} END-POINT(s). |
| Correction Hint | Provide exactly 1 END-POINT (the face of the blind flange). |
| ❌ Bad (Fails) | FLANGE-BLIND END-POINT 100.0 200.0 300.0 50.0 END-POINT 110.0 200.0 300.0 50.0 |
| ✅ Good (Passes) | FLANGE-BLIND END-POINT 100.0 200.0 300.0 50.0 SKEY FLBL |

| C-012 WELD Requires Exactly Two Identical END-POINTs ERROR ★ NEW |  |
| --- | --- |
| Applies To | WELD component blocks |
| Trigger | WELD does not have exactly 2 END-POINT, or the two END-POINT coordinates differ by more than tolerance. |
| Check Logic | IF comp_type=="WELD": IF count("END-POINT") != 2: ERROR ep1, ep2 = parse_end_points(block) IF distance(ep1,ep2) > TOLERANCE: WARN |
| Error Message Template | C-012: Line {lineNo} — WELD REQUIRES 2 IDENTICAL END-POINTs AT THE SAME POSITION. A weld is a zero-length component. Found {n} END-POINT(s). {gap_msg} |
| Correction Hint | Provide 2 END-POINT lines with identical X Y Z coordinates (same position, same OD). |
| ❌ Bad (Fails) | WELD END-POINT 100.0 200.0 300.0 50.0 |
| ✅ Good (Passes) | WELD END-POINT 100.0 200.0 300.0 50.0 END-POINT 100.0 200.0 300.0 50.0 SKEY WW |

| C-013 BOLT Standalone Entry Must Have CO-ORDS with 3 Values ERROR ★ NEW |  |
| --- | --- |
| Applies To | BOLT component blocks (standalone bolt entry) |
| Trigger | BOLT block has no CO-ORDS, or CO-ORDS has wrong value count. |
| Check Logic | IF comp_type=="BOLT": IF count("CO-ORDS")==0: ERROR ELSE: vals = numeric_tokens_after("CO-ORDS") IF len(vals) != 3: ERROR |
| Error Message Template | C-013: Line {lineNo} — BOLT CO-ORDS REQUIRES EXACTLY 3 VALUES: E/W N/S ELEV (no OD). Found {n} values. Note: BOLT CO-ORDS is 3 values, unlike SUPPORT CO-ORDS which is 4. |
| Correction Hint | Provide 3 coordinate values: BOLT position E/W, N/S, and elevation. |
| v1.0 Change | NEW rule. BOLT is a standalone component type distinct from SUPPORT. |
| ❌ Bad (Fails) | BOLT CO-ORDS 100.0 200.0 300.0 50.0 BOLT-DIA 22 |
| ✅ Good (Passes) | BOLT CO-ORDS 100.0 200.0 300.0 BOLT-DIA 22 BOLT-QUANTITY 4 BOLT-LENGTH 150 |

## Valve Rules

| VALV-RULE1 VALVE (Standard) — 2 END-POINTs Required ERROR ◆ ALTERNATIVE |  |
| --- | --- |
| Applies To | VALVE component blocks |
| Trigger | VALVE has 0 or 1 or more than 2 END-POINT lines. |
| Check Logic | # Standard VALVE: inline component with 2 connection ends IF comp_type=="VALVE": IF count("END-POINT") != 2: ERROR |
| Error Message Template | VALV-RULE1: Line {lineNo} — VALVE REQUIRES EXACTLY 2 END-POINTs. Found {n}. Standard inline valve connects two pipe ends. |
| Correction Hint | Provide 2 END-POINT lines with X Y Z OD (4 values each). If this is a 3-way valve, use VALVE-3WAY instead. |
| ❌ Bad (Fails) | VALVE END-POINT 100.0 200.0 300.0 50.0 SKEY VBFL |
| ✅ Good (Passes) | VALVE END-POINT 100.0 200.0 300.0 50.0 END-POINT 300.0 200.0 300.0 50.0 SKEY VBFL SPINDLE-DIRECTION UP |

| VALV-RULE2 VALVE-3WAY — 3 END-POINTs Required ERROR ◆ ALTERNATIVE |  |
| --- | --- |
| Applies To | VALVE-3WAY blocks |
| Trigger | VALVE-3WAY does not have exactly 3 END-POINT lines. |
| Check Logic | IF comp_type=="VALVE-3WAY": IF count("END-POINT") != 3: ERROR |
| Error Message Template | VALV-RULE2: Line {lineNo} — VALVE-3WAY REQUIRES EXACTLY 3 END-POINTs (one per port). Found {n}. |
| Correction Hint | 3-way valves have 3 connection ports. Provide 3 END-POINT lines. |
| ❌ Bad (Fails) | VALVE-3WAY END-POINT 100.0 200.0 300.0 50.0 END-POINT 300.0 200.0 300.0 50.0 |
| ✅ Good (Passes) | VALVE-3WAY END-POINT 100.0 200.0 300.0 50.0 END-POINT 300.0 200.0 300.0 50.0 END-POINT 200.0 200.0 400.0 50.0 SKEY V3** |

| VALV-RULE3 VALVE-4WAY — 4 END-POINTs Required ERROR ◆ ALTERNATIVE |  |
| --- | --- |
| Applies To | VALVE-4WAY blocks |
| Trigger | VALVE-4WAY does not have exactly 4 END-POINT lines. |
| Check Logic | IF comp_type=="VALVE-4WAY": IF count("END-POINT") != 4: ERROR |
| Error Message Template | VALV-RULE3: Line {lineNo} — VALVE-4WAY REQUIRES EXACTLY 4 END-POINTs (one per port). Found {n}. |
| Correction Hint | 4-way valves have 4 connection ports. Provide 4 END-POINT lines. |

| VALV-RULE4 VALVE SPINDLE-DIRECTION Attribute Recommended INFO ◆ ALTERNATIVE |  |
| --- | --- |
| Applies To | VALVE, VALVE-ANGLE, VALVE-3WAY blocks |
| Trigger | VALVE block has no SPINDLE-DIRECTION attribute. |
| Check Logic | IF comp_type IN {"VALVE","VALVE-ANGLE","VALVE-3WAY"} AND "SPINDLE-DIRECTION" NOT IN block: INFO |
| Error Message Template | VALV-RULE4: Line {lineNo} — VALVE has no SPINDLE-DIRECTION. The spindle/handwheel symbol on the isometric may point in an incorrect default direction. Provide SPINDLE-DIRECTION UP, DOWN, NORTH, SOUTH, EAST, or WEST. |
| Correction Hint | Add: SPINDLE-DIRECTION UP (or the correct direction for this valve) |

## Branch Rules

| BRANCH-RULE1 Branch-off: TEE vs TEE-SET-ON vs OLET — Usage Guidance INFO ◆ ALTERNATIVE |  |
| --- | --- |
| Applies To | TEE, TEE-SET-ON, OLET, ELBOLET component blocks |
| Trigger | Informational check: these are the three branch-off methods in PCF. This rule documents which to use. |
| Check Logic | # INFORMATIONAL — no failure condition # TEE: equal or reducing tee fitting in the pipeline # TEE-SET-ON: set-on tee welded to pipe surface (structural connection) # OLET: self-reinforced branch fitting (weldolet, sockolet, etc.) # ELBOLET: 45-degree olet for elbow branch # All require SKEY to match the connection type |
| Error Message Template | BRANCH-RULE1: Advisory — Branch method check: TEE/TEE-SET-ON: use 2 EP + 1 CP + 1 B1P structure OLET/ELBOLET: use 1 CP + 1 B1P structure (both 4-value, NO END-POINT) |
| Correction Hint | Choose the correct component type for the physical fitting in the pipeline. |

| BRANCH-RULE2 OLET CENTRE-POINT Must Lie on Main Pipe Centreline WARNING ◆ ALTERNATIVE |  |
| --- | --- |
| Applies To | OLET and ELBOLET blocks |
| Trigger | OLET CENTRE-POINT coordinates do not lie on any adjacent PIPE component centreline (within tolerance). |
| Check Logic | FOR each OLET block: cp = parse_centre_point(block) FOR each PIPE block: IF point_on_line_segment(cp, pipe_ep1, pipe_ep2, TOLERANCE): break ELSE: WARN |
| Error Message Template | BRANCH-RULE2: Line {lineNo} — OLET CENTRE-POINT ({x},{y},{z}) does not appear to lie on any adjacent PIPE centreline within {tol} {units}. The CENTRE-POINT of an OLET must be on the main pipe centreline. |
| Correction Hint | Set CENTRE-POINT to the exact point on the main pipe centreline where the olet is welded. |

| BRANCH-RULE3 OLET Welds Should Be Present When OLET Used WARNING ◆ ALTERNATIVE |  |
| --- | --- |
| Applies To | PCF file — OLET block neighbourhoods |
| Trigger | OLET component is present but no adjacent WELD blocks are found (welds between main pipe and olet, and between olet and branch). |
| Check Logic | IF comp_type in {"OLET","ELBOLET"}: nearby_welds = [w for w in weld_blocks if shares_coordinate(w, block)] IF len(nearby_welds) < 1: WARN |
| Error Message Template | BRANCH-RULE3: Line {lineNo} — OLET at ({x},{y},{z}) has no associated WELD blocks nearby. Olet connections typically require welds at CENTRE-POINT (main pipe weld) and BRANCH1-POINT (branch weld). Missing welds will omit weld callouts on isometric. |
| Correction Hint | Add WELD blocks at the olet connection points. Associate with olet using MASTER-COMPONENT-IDENTIFIER. |

## Coordinate Rules

| K-001 END-POINT Must Have Exactly 4 Numeric Values ERROR ✓ CONFIRMED |  |
| --- | --- |
| Applies To | All END-POINT lines inside any component block |
| Trigger | END-POINT line does not have exactly 4 numeric tokens after the keyword. |
| Check Logic | vals = numeric_tokens(line[1:]) IF len(vals) != 4: ERROR |
| Error Message Template | K-001: Line {lineNo} — END-POINT HAS {n} VALUE(S); EXACTLY 4 REQUIRED. Format: END-POINT X Y Z OD. The 4th value is the pipe outside diameter (bore). An end-treatment code (BW, FL, SW etc.) may optionally follow as a 5th non-numeric token. |
| Correction Hint | Provide all 4 numeric values: three coordinates and the bore OD. |
| ❌ Bad (Fails) | END-POINT 17989.4 -96400.0 101968.0 |
| ✅ Good (Passes) | END-POINT 17989.4000 -96400.0000 101968.0000 406.4000 |

| K-002 CENTRE-POINT for BEND/TEE/TEE-SET-ON Must Have 3 Numeric Values Only ERROR ✓ CONFIRMED |  |
| --- | --- |
| Applies To | CENTRE-POINT lines inside BEND, ELBOW, TEE, TEE-SET-ON, BEND-TEED, ELBOW-TEED blocks |
| Trigger | CENTRE-POINT in a BEND/TEE context has other than 3 numeric tokens. |
| Check Logic | IF parent_comp IN {"BEND","ELBOW","TEE","TEE-SET-ON","BEND-TEED","ELBOW-TEED"}: vals = numeric_tokens(line[1:]) IF len(vals) != 3: ERROR |
| Error Message Template | K-002: Line {lineNo} — CENTRE-POINT IN {comp} HAS {n} VALUE(S); EXACTLY 3 REQUIRED (X Y Z only — NO OD). Note: OLET CENTRE-POINT is different and uses 4 values (see K-003b). |
| Correction Hint | Remove the 4th value. CENTRE-POINT in bends/tees is a geometric arc centre — no bore dimension applies here. |
| v1.0 Change | Rule unchanged for BEND/TEE context. However: OLET/ELBOLET CENTRE-POINT uses 4 values. See K-003b. |
| ❌ Bad (Fails) | CENTRE-POINT 200.0 300.0 400.0 406.4 |
| ✅ Good (Passes) | CENTRE-POINT 200.0 300.0 400.0 |

| K-003a CO-ORDS for SUPPORT Must Have 4 Numeric Values ERROR ⚠ CORRECTED |  |
| --- | --- |
| Applies To | CO-ORDS lines inside SUPPORT blocks only |
| Trigger | CO-ORDS inside SUPPORT block does not have exactly 4 numeric tokens. |
| Check Logic | IF parent_comp == "SUPPORT": vals = numeric_tokens(line[1:]) IF len(vals) != 4: ERROR |
| Error Message Template | K-003a: Line {lineNo} — SUPPORT CO-ORDS HAS {n} VALUE(S); EXACTLY 4 REQUIRED: X Y Z OD. The 4th value is the bore/OD at the support location. |
| Correction Hint | Provide all 4 values for SUPPORT CO-ORDS. |
| v1.0 Change | Split from v1.0 K-003. CO-ORDS value count is CONTEXT-DEPENDENT. This rule covers SUPPORT only (4 values). |
| ❌ Bad (Fails) | CO-ORDS 17989.4 -96400.0 101968.0 |
| ✅ Good (Passes) | CO-ORDS 17989.4000 -96400.0000 101968.0000 406.4000 |

| K-003b CO-ORDS for BOLT/END-CONNECTION/END-POSITION Must Have 3 Numeric Values ERROR ★ NEW |  |
| --- | --- |
| Applies To | CO-ORDS lines inside BOLT, END-CONNECTION-PIPELINE, END-CONNECTION-EQUIPMENT, NOZZLE, END-POSITION-OPEN/CLOSED/VENT/DRAIN/NULL, SPOOL-IDENTIFIER, SYSTEM-SPLIT, MESSAGE-* standalone blocks |
| Trigger | CO-ORDS in a non-SUPPORT context does not have exactly 3 numeric tokens. |
| Check Logic | THREE_VAL_CONTEXTS = {"BOLT","END-CONNECTION-PIPELINE","END-CONNECTION-EQUIPMENT","NOZZLE","END-POSITION-OPEN","END-POSITION-CLOSED","END-POSITION-VENT","END-POSITION-DRAIN","END-POSITION-NULL","SPOOL-IDENTIFIER","SYSTEM-SPLIT","MESSAGE","MESSAGE-SQUARE","MESSAGE-ROUND","MESSAGE-CIRCLE","MESSAGE-TRIANGLE","MESSAGE-DIAMOND","MESSAGE-POINTED","MESSAGE-DOUBLE-CIRCLE","MESSAGE-ELLIPSE"} IF parent_comp IN THREE_VAL_CONTEXTS: vals = numeric_tokens(line[1:]) IF len(vals) != 3: ERROR |
| Error Message Template | K-003b: Line {lineNo} — {comp} CO-ORDS HAS {n} VALUE(S); EXACTLY 3 REQUIRED: E/W N/S ELEV (no OD for this component type). Note: SUPPORT CO-ORDS is different and uses 4 values. |
| Correction Hint | Provide exactly 3 coordinate values. This component type does not include bore in its CO-ORDS. |
| v1.0 Change | CRITICAL NEW RULE. v1.0 K-003 incorrectly stated all CO-ORDS = 4 values. Bolt entries and end-connection entries use 3-value CO-ORDS (E/W, N/S, ELEV). This distinction is from the official PCF Component Reference Guide. |
| ❌ Bad (Fails) | BOLT CO-ORDS 100.0 200.0 300.0 50.0 |
| ✅ Good (Passes) | BOLT CO-ORDS 100.0 200.0 300.0 |

| K-003c OLET/ELBOLET CENTRE-POINT Has 4 Numeric Values (Includes OD) NEW ★ NEW |  |
| --- | --- |
| Applies To | CENTRE-POINT lines inside OLET and ELBOLET blocks |
| Trigger | OLET CENTRE-POINT does not have exactly 4 numeric tokens. |
| Check Logic | IF parent_comp IN {"OLET","ELBOLET"}: vals = numeric_tokens(line[1:]) IF len(vals) != 4: ERROR |
| Error Message Template | K-003c: Line {lineNo} — OLET/ELBOLET CENTRE-POINT HAS {n} VALUE(S); EXACTLY 4 REQUIRED: X Y Z OD. Unlike BEND/TEE CENTRE-POINT, OLET CENTRE-POINT includes the bore OD as the 4th value. |
| Correction Hint | Add the OD (4th value) to the OLET CENTRE-POINT line. |
| v1.0 Change | CRITICAL NEW RULE. The official PCF documentation shows OLET CENTRE-POINT as: 330500.0000 6400.0000 -2097.0000 3.000 — 4 values including OD. This is different from BEND/TEE CENTRE-POINT which has only 3 values. |
| ❌ Bad (Fails) | OLET CENTRE-POINT 330500.0 6400.0 -2097.0 |
| ✅ Good (Passes) | OLET CENTRE-POINT 330500.0 6400.0 -2097.0 3.0 BRANCH1-POINT 330500.0 6647.6 -2097.0 3.0 |

| K-004 BRANCH1-POINT Must Have 4 Numeric Values ERROR ✓ CONFIRMED |  |
| --- | --- |
| Applies To | BRANCH1-POINT lines in TEE, TEE-SET-ON, OLET, ELBOLET blocks |
| Trigger | BRANCH1-POINT has other than 4 numeric tokens. |
| Check Logic | vals = numeric_tokens(line[1:]) IF len(vals) != 4: ERROR |
| Error Message Template | K-004: Line {lineNo} — BRANCH1-POINT HAS {n} VALUE(S); EXACTLY 4 REQUIRED: X Y Z OD. The 4th value is the branch bore OD. |
| Correction Hint | Provide all 4 values including branch bore. |
| ❌ Bad (Fails) | BRANCH1-POINT 200.0 200.0 400.0 |
| ✅ Good (Passes) | BRANCH1-POINT 200.0 200.0 400.0 40.0000 |

| K-005 All Coordinate Values Must Parse as Floating-Point Numbers ERROR ✓ CONFIRMED |  |
| --- | --- |
| Applies To | All tokens in coordinate value positions (END-POINT, CENTRE-POINT, CO-ORDS, BRANCH1-POINT) |
| Trigger | Any token in a coordinate value slot is not a valid floating-point number. |
| Check Logic | FOR each coord_line: FOR i, tok IN enumerate(value_tokens): TRY: float(tok) EXCEPT: ERROR at lineNo, token position i+2 |
| Error Message Template | K-005: Line {lineNo}, token {pos} — COORDINATE VALUE "{tok}" IS NOT A VALID NUMBER. All coordinate tokens must be parseable as floating-point. |
| Correction Hint | Replace "{tok}" with the correct numeric value. Remove any units labels (mm, in, etc.) from coordinate values. |
| ❌ Bad (Fails) | END-POINT 17989.4 N/A 101968.0 406.4 |
| ✅ Good (Passes) | END-POINT 17989.4000 -96400.0000 101968.0000 406.4000 |

| K-006 OD Value Must Be Positive WARNING ✓ CONFIRMED |  |
| --- | --- |
| Applies To | 4th value (OD) on END-POINT, BRANCH1-POINT, SUPPORT CO-ORDS, OLET CENTRE-POINT |
| Trigger | OD value ≤ 0. |
| Check Logic | od = float(tokens[-1]) IF od <= 0: WARN |
| Error Message Template | K-006: Line {lineNo} — OD VALUE IS {val}, which is zero or negative. A bore of zero is only valid for certain specialty closures. Verify this is intentional. |
| Correction Hint | Set OD to the actual outside diameter of the pipe at this point. |

## Angle Rules

| A-001 ANGLE Value Is in Hundredths of Degrees ERROR ✓ CONFIRMED |  |
| --- | --- |
| Applies To | ANGLE attribute lines in BEND and ELBOW blocks |
| Trigger | ANGLE value < 25 or > 18000, or is not a multiple of 25. |
| Check Logic | val = int(float(tokens[1])) IF val < 25 OR val > 18000 OR val % 25 != 0: ERROR |
| Error Message Template | A-001: Line {lineNo} — ANGLE VALUE {val} IS INVALID. ANGLE is in hundredths of degrees: 90° = 9000, 45° = 4500, 30° = 3000. Valid range: 25 to 18000. Multiples of 25 only. |
| Correction Hint | Multiply degree value by 100: 90° → 9000, 45° → 4500. |
| ❌ Bad (Fails) | ANGLE 90 |
| ✅ Good (Passes) | ANGLE 9000 |

| A-002 Declared ANGLE Should Match Geometry WARNING ✓ CONFIRMED |  |
| --- | --- |
| Applies To | BEND blocks with all keypoints present |
| Trigger | Computed geometric angle differs from declared ANGLE by more than 100 (i.e., ±1 degree). |
| Check Logic | vec1 = normalize(ep1_xyz - cp_xyz) vec2 = normalize(ep2_xyz - cp_xyz) geom = round(degrees(acos(clip(dot(vec1,vec2),-1,1))) * 100) IF abs(geom - declared) > 100: WARN |
| Error Message Template | A-002: Line {lineNo} — BEND ANGLE MISMATCH. Declared ANGLE = {declared} ({deg_d}°) but geometry computes {geom} ({deg_g}°). Tolerance: ±1°. This will cause incorrect dimensioning on isometric. |
| Correction Hint | Recalculate and correct the ANGLE value, or check that END-POINT and CENTRE-POINT coordinates are correct. |

## Units Rules

| U-001 Bore and Coordinate Units Must Be in Same System ERROR ✓ CONFIRMED |  |
| --- | --- |
| Applies To | UNITS-BORE and UNITS-CO-ORDS header values |
| Trigger | One is metric and the other is imperial. |
| Check Logic | METRIC={"MM","MM-HUNDREDTHS","METER"} IMPERIAL={"INCH"} IF (bore IN METRIC AND coord IN IMPERIAL) OR (bore IN IMPERIAL AND coord IN METRIC): ERROR |
| Error Message Template | U-001: UNIT SYSTEM MISMATCH — UNITS-BORE is "{bore}" (line {bl}) and UNITS-CO-ORDS is "{coord}" (line {cl}). Mixed metric/imperial will cause ISOGEN error 2001 (disconnection from scale mismatch). |
| Correction Hint | Use the same measurement system for both. Most common: UNITS-BORE MM + UNITS-CO-ORDS MM. |
| ❌ Bad (Fails) | UNITS-BORE INCH UNITS-CO-ORDS MM |
| ✅ Good (Passes) | UNITS-BORE MM UNITS-CO-ORDS MM |

| U-002 OD Values Should Be Consistent With Declared Bore Units WARNING ✓ CONFIRMED |  |
| --- | --- |
| Applies To | All OD (4th value) tokens in END-POINT, CO-ORDS (SUPPORT), BRANCH1-POINT |
| Trigger | OD magnitude appears inconsistent with declared UNITS-BORE. |
| Check Logic | IF UNITS_BORE=="MM" AND 0 < OD < 5.0: WARN IF UNITS_BORE=="INCH" AND OD > 100.0: WARN |
| Error Message Template | U-002: Line {lineNo} — OD VALUE {val} APPEARS INCONSISTENT WITH UNITS-BORE={units}. If units are MM, OD should typically be ≥ 5.0. If INCH, OD should be < 100. Possible unit mismatch. |
| Correction Hint | Verify OD values are in the units declared by UNITS-BORE. |

## Topology Rules

| T-001 All Component End-Points Must Connect Within Tolerance ERROR ✓ CONFIRMED |  |
| --- | --- |
| Applies To | All END-POINT coordinates across all component blocks |
| Trigger | Any END-POINT has no matching partner within the connectivity tolerance. |
| Check Logic | TOLS={"MM":8.0,"INCH":0.315,"MM-HUNDREDTHS":800,"METER":0.008} tol = TOLS[UNITS_CO_ORDS] Build graph of all end-points from all components FOR each ep: neighbours = [other for other in all_eps if distance(ep,other)<tol and other!=ep] IF len(neighbours)==0: ERROR |
| Error Message Template | T-001: DISCONNECTED END-POINT at ({x},{y},{z}) on {comp} block starting at line {lineNo}. Nearest other end-point is {gap} {units} away. Maximum allowed gap: {tol} {units}. This will cause ISOGEN error 2022. |
| Correction Hint | Align end-points exactly. A gap > 8mm (MM units) is treated as disconnected pipeline. |
| v1.0 Change | Rule confirmed. Tolerance table added: MM=8.0, INCH=0.315, MM-HUNDREDTHS=800, METER=0.008 |
| ❌ Bad (Fails) | (Two PIPE blocks whose shared end-point differs by 10mm — beyond tolerance) |
| ✅ Good (Passes) | (Shared end-point coordinates identical or within 8mm) |

| T-002 Dead-End Points Should Be Intentionally Terminated WARNING ✓ CONFIRMED |  |
| --- | --- |
| Applies To | All isolated end-points |
| Trigger | An end-point has no matching neighbour AND no terminal component nearby. |
| Check Logic | TERMINAL={"CAP","FLANGE-BLIND","VENT-DRAIN","END-POSITION-OPEN","END-POSITION-CLOSED","END-POSITION-VENT","END-POSITION-DRAIN","END-CONNECTION-PIPELINE","END-CONNECTION-EQUIPMENT","NOZZLE"} FOR each isolated_ep: IF no nearby TERMINAL component: WARN |
| Error Message Template | T-002: Line {lineNo} — UNCONNECTED END-POINT at ({x},{y},{z}) on {comp}. No terminal component or end-connection entry found nearby. Verify this is a deliberate open end. |
| Correction Hint | Add CAP, FLANGE-BLIND, or an END-POSITION-* / END-CONNECTION-* entry at the open end. |

## Attribute Rules

| R-001 Unrecognised Attribute Keyword WARNING ⚠ CORRECTED |  |
| --- | --- |
| Applies To | All indented keywords inside component blocks |
| Trigger | Indented keyword not in validated attribute keyword list. |
| Check Logic | VALID_ATTRS = {"END-POINT","CENTRE-POINT","BRANCH1-POINT","BRANCH2-POINT","CO-ORDS", "SKEY","ANGLE","BEND-RADIUS","BRANCH-TABLE","FABRICATION-ITEM","ERECTION-ITEM", "OFFSHORE-ITEM","GRADE","HEAT-NUMBER","ITEM-CODE","LONG-RADIUS","MATERIAL", "PIPING-SPEC","RECORD","SCHEDTHK","SHORT-RADIUS","SHOP-WELD","SUPPORT-DIRECTION", "SUPPORT-TYPE","WALL-THICKNESS","WEIGHT","WELD-NUMBER","PIPE-BORE","LENGTH", "DESCRIPTION","QUANTITY","ITEM-GROUP","SIZE","ADDITIONAL-ITEM","CATEGORY", "COMPONENT-IDENTIFIER","MASTER-COMPONENT-IDENTIFIER","MATERIAL-IDENTIFIER", "REPEAT-WELD-IDENTIFIER","UCI","LOCATION", "SPINDLE-DIRECTION","WINDOW-DIRECTION","CONNECTION-REFERENCE","PIPELINE-REFERENCE", "BOLT-DIA","BOLT-ITEM-CODE","BOLT-QUANTITY","BOLT-LENGTH","FABRICATION", "COMPONENT-ATTRIBUTE1",...,"COMPONENT-ATTRIBUTE10", "MESSAGE","MESSAGE-SQUARE","MESSAGE-ROUND","MESSAGE-CIRCLE","MESSAGE-TRIANGLE", "MESSAGE-DIAMOND","MESSAGE-POINTED","MESSAGE-DOUBLE-CIRCLE","MESSAGE-ELLIPSE", "TEXT","UNDIMENSIONED","END-CONNECTION-PIPELINE","END-CONNECTION-EQUIPMENT", "END-POSITION-OPEN","END-POSITION-CLOSED","END-POSITION-VENT","END-POSITION-DRAIN","END-POSITION-NULL"} IF attr NOT IN VALID_ATTRS: WARN |
| Error Message Template | R-001: Line {lineNo} — UNRECOGNISED ATTRIBUTE "{kw}". ISOGEN may silently ignore this. Check for a typo. If this is a COMPONENT-ATTRIBUTE, use COMPONENT-ATTRIBUTE1 through COMPONENT-ATTRIBUTE10. |
| Correction Hint | Correct the typo or use the appropriate standard attribute name. |
| v1.0 Change | Attribute list significantly expanded with: CATEGORY, COMPONENT-IDENTIFIER, LOCATION, SPINDLE-DIRECTION, WINDOW-DIRECTION, CONNECTION-REFERENCE, BOLT-* attributes, MESSAGE-* attributes, END-CONNECTION-* and END-POSITION-* types, ADDITIONAL-ITEM, UCI, FABRICATION, DESCRIPTION, QUANTITY, SIZE, LENGTH. |

| R-002 SKEY Value Must Not Be Empty WARNING ✓ CONFIRMED |  |
| --- | --- |
| Applies To | All SKEY lines |
| Trigger | SKEY keyword present but no value follows. |
| Check Logic | IF tokens[0]=="SKEY" AND len(tokens) < 2: WARN |
| Error Message Template | R-002: Line {lineNo} — SKEY HAS NO VALUE. ISOGEN will use a default symbol. Provide the appropriate symbol key (e.g. BEBW, TEBW, ANCH, FPFA). |
| Correction Hint | Add SKEY value. Refer to ISOGEN Symbol Key Library for valid values. |
| ❌ Bad (Fails) | SKEY |
| ✅ Good (Passes) | SKEY BEBW |

| R-003 SKEY Value Format Recommendation INFO ✓ CONFIRMED |  |
| --- | --- |
| Applies To | All SKEY lines |
| Trigger | SKEY value does not match the 4-character uppercase pattern. |
| Check Logic | IF NOT re.match(r"^[A-Z0-9]{2,6}(\*\*)?$", skey_val): INFO |
| Error Message Template | R-003: Line {lineNo} — SKEY VALUE "{val}" does not match typical 2–6 uppercase character format. Valid examples: BEBW, TEBW, FPFA, ANCH, VB**, BECL. Wildcards (**) are valid for "any end condition". |
| Correction Hint | Verify SKEY against ISOGEN symbol library. End-connection codes: BW=Butt Weld, FL=Flanged, SW=Socket Weld, SC=Screwed, CL=Clamped. |

| R-004 BEND-RADIUS Must Be Positive WARNING ✓ CONFIRMED |  |
| --- | --- |
| Applies To | BEND-RADIUS lines |
| Trigger | BEND-RADIUS value ≤ 0. |
| Check Logic | IF float(tokens[1]) <= 0: WARN |
| Error Message Template | R-004: Line {lineNo} — BEND-RADIUS VALUE {val} IS ZERO OR NEGATIVE. A physical pipe bend must have a positive centreline radius. |
| Correction Hint | Set BEND-RADIUS to the actual centreline bend radius in UNITS-CO-ORDS units. |

| R-005 Duplicate Single-Instance Attribute in Same Block WARNING ✓ CONFIRMED |  |
| --- | --- |
| Applies To | Each component block |
| Trigger | An attribute that should appear only once appears multiple times in the same block. |
| Check Logic | SINGLE = {"SKEY","ANGLE","BEND-RADIUS","CENTRE-POINT","PIPING-SPEC","MATERIAL","WEIGHT","SPINDLE-DIRECTION","CATEGORY"} FOR kw IN SINGLE: IF count(kw, block) > 1: WARN |
| Error Message Template | R-005: Line {lineNo} — DUPLICATE "{kw}" IN SAME {comp} BLOCK (first at line {first}, duplicate at line {lineNo}). ISOGEN uses the last value — earlier values are silently discarded. |
| Correction Hint | Remove the duplicate. Keep the correct value. |

| R-006 CENTRE-POINT for BEND/TEE Must Not Have 4 Values ERROR ✓ CONFIRMED |  |
| --- | --- |
| Applies To | CENTRE-POINT in BEND, ELBOW, TEE, TEE-SET-ON |
| Trigger | CENTRE-POINT in bend/tee context contains 4 or more numeric values. |
| Check Logic | IF parent_comp IN {"BEND","ELBOW","TEE","TEE-SET-ON"} AND len(numeric_vals) >= 4: ERROR |
| Error Message Template | R-006: Line {lineNo} — CENTRE-POINT IN {comp} HAS 4 VALUES BUT MUST HAVE EXACTLY 3 (X Y Z, NO OD). Adding an OD here will cause ISOGEN to misparse the geometry. |
| Correction Hint | Remove the 4th value from CENTRE-POINT. |
| ❌ Bad (Fails) | CENTRE-POINT 200.0 300.0 400.0 406.4 |
| ✅ Good (Passes) | CENTRE-POINT 200.0 300.0 400.0 |

| R-007 CATEGORY Attribute Must Have a Valid Value NEW ★ NEW |  |
| --- | --- |
| Applies To | CATEGORY lines inside any component block |
| Trigger | CATEGORY value is not one of the three recognised identifiers. |
| Check Logic | VALID_CATEGORY = {"FABRICATION","ERECTION","OFFSHORE"} IF tokens[0]=="CATEGORY" AND tokens[1].upper() NOT IN VALID_CATEGORY: WARN |
| Error Message Template | R-007: Line {lineNo} — CATEGORY VALUE "{val}" IS NOT RECOGNISED. Valid values: FABRICATION, ERECTION, OFFSHORE. This controls material list categorisation (shop vs field vs offshore work). |
| Correction Hint | Use CATEGORY FABRICATION (shop work), CATEGORY ERECTION (field work), or CATEGORY OFFSHORE. |
| ❌ Bad (Fails) | CATEGORY SHOP |
| ✅ Good (Passes) | CATEGORY FABRICATION |

| R-008 ADDITIONAL-ITEM Attribute Column Position NEW ★ NEW |  |
| --- | --- |
| Applies To | ADDITIONAL-ITEM entries (both associated and non-associated) |
| Trigger | ADDITIONAL-ITEM appears at wrong column position. |
| Check Logic | # ADDITIONAL-ITEM when standalone (non-associated) MUST be at column 1 # ADDITIONAL-ITEM when associated with a component MUST start at column 5 IF type=="standalone" AND line[0]==" ": ERROR IF type=="associated" AND line[0]!=" ": ERROR |
| Error Message Template | R-008: Line {lineNo} — ADDITIONAL-ITEM COLUMN POSITION ERROR. Standalone ADDITIONAL-ITEM must start at column 1. Associated (within component) ADDITIONAL-ITEM must start at column 5 (4 spaces indent). |
| Correction Hint | For non-associated extra materials: ADDITIONAL-ITEM at column 1. For items associated with a component: indent 4 spaces. |

## Specific Rules

| S-001 REDUCER Must Have Two Different OD Values ERROR ✓ CONFIRMED |  |
| --- | --- |
| Applies To | REDUCER-CONCENTRIC and REDUCER-ECCENTRIC blocks |
| Trigger | Both END-POINT lines have the same OD. |
| Check Logic | IF comp_type IN {"REDUCER-CONCENTRIC","REDUCER-ECCENTRIC"}: od1 = float(ep_lines[0].vals[3]) od2 = float(ep_lines[1].vals[3]) IF od1 == od2: ERROR |
| Error Message Template | S-001: Line {lineNo} — REDUCER HAS SAME OD ({val}) ON BOTH END-POINTs. A reducer must connect two DIFFERENT bore sizes. Check bore values. |
| Correction Hint | Provide correct OD values: larger bore on inlet END-POINT, smaller bore on outlet. |

## Completeness Rules

| X-001 File Has No Component Blocks INFO ✓ CONFIRMED |  |
| --- | --- |
| Applies To | File after header section |
| Trigger | Zero component blocks found. |
| Check Logic | IF len(component_blocks)==0: INFO |
| Error Message Template | X-001: FILE HAS VALID HEADER BUT NO COMPONENT BLOCKS. ISOGEN will produce an empty isometric. Add PIPE, BEND, TEE, VALVE and other component blocks. |
| Correction Hint | Add component blocks to represent the piping geometry. |

| X-002 Pipeline Has Only One Component WARNING ✓ CONFIRMED |  |
| --- | --- |
| Applies To | Count of component blocks |
| Trigger | Exactly one component block exists. |
| Check Logic | IF len(component_blocks)==1: WARN |
| Error Message Template | X-002: ONLY ONE COMPONENT BLOCK IN FILE. This usually indicates a truncated export. Verify no components are missing. |
| Correction Hint | Check source 3D model export. Complete pipelines typically have multiple components. |

# 5.  Alternative Rules — Ambiguous Components Awaiting Samples

The following components or situations have multiple possible valid PCF patterns. The rules are stated as alternatives (RULE1 = best known, RULE2 = possible alternative). Once you provide sample working PCF files, we can eliminate the alternatives and establish a single definitive rule.

### VALV — VALVE Variants

Alternative rules: VALV-RULE1 (2 END-POINTs — standard inline valve) | VALV-RULE2 (3 END-POINTs — VALVE-3WAY) | VALV-RULE3 (4 END-POINTs — VALVE-4WAY) | VALV-RULE4 (SPINDLE-DIRECTION recommendation)

Sample Request

Q: Please provide sample PCF blocks for: (a) a standard gate/ball valve, (b) a 3-way valve if used on your project, (c) an actuated valve. This will confirm whether SPINDLE-DIRECTION and CENTRE-POINT appear in your system's output.

### BRANCH — Branch Connections (TEE / TEE-SET-ON / OLET)

Alternative rules: BRANCH-RULE1 (usage guidance: TEE vs TEE-SET-ON vs OLET) | BRANCH-RULE2 (OLET CENTRE-POINT on main pipe centreline) | BRANCH-RULE3 (OLET associated WELD blocks)

Sample Request

Q: Please provide 3 sample PCF blocks for: (a) a weldolet branch, (b) a sockolet, (c) a set-on tee. This will confirm SKEY usage and whether WELD blocks appear in your exports.

### FLAN — FLANGE Variants (FLANGE, FLANGE-BLIND, LAPJOINT-RING)

Alternative rules: Standard FLANGE — 2 END-POINTs (confirmed) | FLANGE-BLIND — 1 END-POINT (confirmed from documentation) | LAPJOINT-RING — geometry unclear: may need 2 END-POINTs or CO-ORDS

Sample Request

Q: Please provide a sample PCF for a lapjoint flange assembly (stub end + lapjoint ring). This will confirm LAPJOINT-RING keypoints.

### INST — INSTRUMENT Variants

Alternative rules: INSTRUMENT — 2 END-POINTs (inline instrument like flow element) | INSTRUMENT-DIAL — may have SPINDLE-DIRECTION equivalent (WINDOW-DIRECTION) | INSTRUMENT-3WAY/4WAY — 3/4 END-POINTs analogous to VALVE-3WAY/4WAY

Sample Request

Q: Please provide sample PCF blocks for any instruments in your pipeline (pressure gauges, flow meters, temp elements). Confirm whether INSTRUMENT uses the same structure as VALVE.

### ENDCONN — End-Connection and End-Position Entries

Alternative rules: END-CONNECTION-PIPELINE — standalone block with CO-ORDS (3 values) + PIPELINE-REFERENCE | END-CONNECTION-EQUIPMENT — standalone block with CO-ORDS (3 values) + CONNECTION-REFERENCE | END-POSITION-* types — CO-ORDS (3 values) only

Sample Request

Q: Does your CAD system export END-CONNECTION-PIPELINE or END-CONNECTION-EQUIPMENT entries? If yes, please provide samples. These mark pipe-to-pipe and pipe-to-equipment terminations.

### MATL — MATERIALS Section at End of PCF

Alternative rules: Some PCF exports include a MATERIALS or ITEMS section at end of file with ITEM-CODE + DESCRIPTION pairs | This is NOT a component block — it has no END-POINT or SKEY

Sample Request

Q: Does your CAD system export a MATERIALS or ITEMS section at the end of the PCF? Please provide a sample. This section is sometimes seen and its exact format needs confirmation.

# Appendix A:  Iterative Improvement Methodology

This appendix captures the methodology for progressively refining the Rule Book using real PCF samples. It describes the process, what to look for in each sample, and the evaluation framework.

## A.1  The Iterative Loop

| Process Step | Description |
| --- | --- |
| Step 1: Receive Samples | User provides 3 PCF text samples. Ideally: 2 samples the same type (for pattern recognition), 1 sample different type (for coverage expansion). All samples should be confirmed working (accepted by ISOGEN). |
| Step 2: Pattern Recognition | Extract the structural pattern from each sample: - What component type keyword appears? - What attributes follow (in what order)? - How many END-POINTs / CENTRE-POINTs / etc.? - Do the coordinate value counts match expected rules? - What SKEY values appear? - Are any unexpected attributes present? |
| Step 3: Compare Against Current Rules | For each sample, run it mentally through every relevant rule. Record: - PASS: Rule fires correctly / does not fire (true positive / true negative) - FAIL: Rule fires incorrectly on valid PCF (false positive) → rule needs loosening - MISS: Rule does not fire on an error that exists (false negative) → rule needs tightening - GAP: No rule covers this pattern → new rule needed |
| Step 4: Update Rule Book | For each FAIL, MISS, or GAP: - Modify the pseudocode of the affected rule - Update the error message template - Resolve ALTERNATIVE rules to definitive rules where samples confirm the pattern - Add new rules if gaps found - Increment minor version (v2.1, v2.2, etc.) |
| Step 5: Document Changes | Append a change log entry: Rule ID, change type (modified/new/deleted), reason, sample that triggered change. |

## A.2  What To Request in Each Sample Round

Priority 1 — Confirm Alternative Rules (provide 3 of each):

| Priority Round | Target | What to Confirm |
| --- | --- | --- |
| Round 1 | VALVE samples | Standard gate/ball valve, check valve, control valve. Need to see SPINDLE-DIRECTION, CENTRE-POINT usage. |
| Round 2 | Branch connection samples | Weldolet, sockolet, elbow-o-let. Confirm OLET CENTRE-POINT has 4 values and WELD associations. |
| Round 3 | Flange assembly samples | Flanged valve (FLANGE+GASKET+VALVE), blind flange, lapjoint assembly. |
| Round 4 | Instrument samples | Inline instrument, dial instrument, pressure tapping. |
| Round 5 | Multi-run pipeline | A PCF with 3+ bends, branches, supports — tests topology rules T-001/T-002. |
| Round 6 | End connections | Pipe connecting to equipment nozzle, pipeline-to-pipeline continuation. |
| Round 7 | Special components | Y-piece, cross fitting, trap, filter. Confirm keypoint structure. |
| Round 8 | Complex assembly | A complete line with BOLT entries, MATERIALS section, ADDITIONAL-ITEM usage. |

## A.3  Pattern Recognition Checklist (Per Sample)

For every PCF sample received, answer all of the following:

1. What is the component type keyword? (exact spelling, column 1?)

2. Count of END-POINT lines in this block?

3. Count of CENTRE-POINT lines? How many values does each have? (3 or 4?)

4. Count of BRANCH1-POINT lines? How many values?

5. CO-ORDS present? How many values? (3 or 4?)

6. SKEY present? What value? Does it follow 4-char uppercase pattern?

7. ANGLE present? Is it in hundredths? (90°=9000)

8. BEND-RADIUS present? Is it positive?

9. CATEGORY present? What value? (FABRICATION/ERECTION/OFFSHORE)

10. PIPING-SPEC present? What value?

11. ITEM-CODE present?

12. SPINDLE-DIRECTION present? What direction?

13. Any COMPONENT-ATTRIBUTE{1-10} lines?

14. Any ADDITIONAL-ITEM lines associated with this component?

15. Any WELD blocks nearby that reference this component?

16. End-treatment codes on END-POINT (5th token like BW, FL, SW)?

17. Any attributes not in the known attribute list (Rule R-001)?

18. Does the block connect to adjacent blocks within 8mm tolerance?

## A.4  Rule Book Evaluation Framework

A rule book is only as good as its measurable quality. The following parameters should be tracked as the rule book evolves through sample iterations:

| Parameter | Definition | Target | How to Measure |
| --- | --- | --- | --- |
| Coverage Score | % of all known ISOGEN runtime errors (error code catalogue) that have at least one validator rule designed to catch them before ISOGEN execution. | Target: >85% | Track ISOGEN error codes 2022, 2024, 2001, 1001, 47, 52, 28, 2, 1010 vs rules that intercept them |
| False Positive Rate | % of valid PCF files (confirmed ISOGEN-accepted) that trigger at least one ERROR or FATAL rule incorrectly. | Target: <2% | Run each valid sample through all rules. Log any rules that fire on valid input. |
| False Negative Rate | % of known-bad PCF files (confirmed ISOGEN-rejected) where no ERROR or FATAL rule fires. | Target: <5% | Deliberately introduce known errors from ISOGEN error catalogue. Count misses. |
| Error Actionability Score | 1–5 scale: can the user fix the error using only the error message, without referring to this document? | Target: ≥4 average | User survey: give error message only, ask user to fix. Rate success. |
| Rule Precision | % of rules where the pseudocode is unambiguous enough to implement in under 60 minutes without additional specification. | Target: 100% | Developer peer review: mark rules needing clarification. |
| Component Coverage | % of the 50+ valid component types that have at least one specific validation rule. | Target: >80% | Map each component keyword to rules C-001 through S-001. |
| Line Number Accuracy | % of errors where the reported line number points to the exact problematic line (not a nearby line or the block header). | Target: >95% | Test with synthetic errors at known lines. Verify reported line matches. |
| Version Sensitivity | Rules that behave differently depending on ISOGEN version (v8.x vs v10.x vs v15). | Track count | Flag rules with version-specific behaviour. Minimum: identify all such rules. |
| Alternative Rule Resolution Rate | % of ALTERNATIVE rules resolved to definitive rules through sample validation. | Target: 100% by Round 8 | Count unresolved ALTERNATIVE rules after each sample round. |
| Sample Diversity Index | Number of unique CAD source systems from which sample PCFs have been provided. | Target: ≥3 systems | Log source system for each sample (SmartPlant 3D, CADWorx, AVEVA, Plant 3D, etc.). |

## A.5  Change Log Template

All changes to this rule book must be logged here. Each entry created when a sample round produces a rule change.

Version | Date | Rule ID | Change Type | Trigger | Description

v1.0    | 2026-02-13 | All    | Initial     | First issue from PCF research synthesis

v2.0    | 2026-02-13 | C-001  | CORRECTED   | Official Alias component guide | Added 20+ missing component keywords

v2.0    | 2026-02-13 | K-002  | CORRECTED   | Official Alias olet spec | OLET CENTRE-POINT = 4 values (includes OD)

v2.0    | 2026-02-13 | K-003  | CORRECTED   | Official Alias bolt spec | CO-ORDS is 3 values for BOLT, 4 for SUPPORT

v2.0    | 2026-02-13 | C-006  | CORRECTED   | Official Alias olet spec | OLET is NOT break-point-only; it is a real block

# Appendix B:  Complete Validated Component Keyword List

All column-1 keywords that begin a component block. Sources: Hexagon PCF Reference Guide v15 + Alias PCF Component Reference Guide v2 + CADMATIC PCF export documentation.

| Component Keyword | Description | Keypoints Required | Validation Status |
| --- | --- | --- | --- |
| PIPE | Standard pipe segment | 2 EP | Confirmed |
| PIPE-FIXED | Fixed-length pipe (flanged assembly) | 2 EP + SKEY | Confirmed |
| ELBOW | Elbow fitting | 2 EP + CP (3-val) | Confirmed |
| ELBOW-TEED | Elbow with integral tee outlet | 2 EP + CP + B1P | NEW — needs sample |
| BEND | Cold/hot bend | 2 EP + CP (3-val) + ANGLE + BR | Confirmed |
| BEND-TEED | Bend with tee outlet | 2 EP + CP + B1P | NEW — needs sample |
| TEE | Equal or reducing tee | 2 EP + CP (3-val) + B1P | Confirmed |
| TEE-SET-ON | Set-on welded tee | 2 EP + CP (3-val) + B1P | Confirmed |
| REDUCER-CONCENTRIC | Concentric reducer | 2 EP (diff OD) | Confirmed |
| REDUCER-ECCENTRIC | Eccentric reducer | 2 EP (diff OD) | Confirmed |
| FLANGE | Flange | 2 EP | Confirmed |
| FLANGE-BLIND | Blind flange (terminal) | 1 EP | Confirmed |
| GASKET | Gasket | 2 EP | Confirmed |
| SUPPORT | Pipe support | CO-ORDS (4-val) | Confirmed |
| VALVE | Inline valve (2-way) | 2 EP | Confirmed |
| VALVE-3WAY | 3-way valve | 3 EP | ALTERNATIVE — needs sample |
| VALVE-4WAY | 4-way valve | 4 EP | ALTERNATIVE — needs sample |
| VALVE-ANGLE | Angle body valve | 2 EP | Confirmed |
| CAP | Pipe cap (terminal) | 2 EP | Confirmed |
| OLET | Olet branch fitting | CP(4-val) + B1P(4-val) | Confirmed — CORRECTED |
| ELBOLET | Elbow olet | CP(4-val) + B1P(4-val) | NEW — confirmed pattern |
| STUB-IN | Stub-in branch | 2 EP | Needs sample |
| COUPLING | Coupling | 2 EP | Confirmed |
| UNION | Union | 2 EP | Confirmed |
| CROSS | Cross fitting | 4 EP | Needs sample |
| Y-PIECE-FITTING | Y-piece | 3 EP | Needs sample |
| INSTRUMENT | Inline instrument | 2 EP | Confirmed pattern |
| INSTRUMENT-3WAY | 3-way instrument | 3 EP | ALTERNATIVE |
| INSTRUMENT-4WAY | 4-way instrument | 4 EP | ALTERNATIVE |
| INSTRUMENT-ANGLE | Angle instrument | 2 EP | ALTERNATIVE |
| INSTRUMENT-DIAL | Dial instrument | 2 EP + WINDOW-DIRECTION | ALTERNATIVE |
| INSTRUMENT-OFFSET | Offset instrument | 2 EP | ALTERNATIVE |
| INSTRUMENT-RETURN | Return instrument | 2 EP | ALTERNATIVE |
| WELD | Weld callout | 2 identical EP | Confirmed |
| BOLT | Bolt set (standalone) | CO-ORDS (3-val) | Confirmed |
| NOZZLE | Equipment nozzle connection | CO-ORDS (3-val) + CONNECTION-REFERENCE | Confirmed |
| LAPJOINT-RING | Lapjoint flange ring | 2 EP | ALTERNATIVE — needs sample |
| FILTER | Inline filter | 2 EP | ALTERNATIVE |
| TRAP | Steam trap | 2 EP | ALTERNATIVE |
| REINFORCEMENT-PAD | Reinforcement pad | Varies | ALTERNATIVE — needs sample |
| SAFETY-DISC | Rupture/safety disc | 2 EP | ALTERNATIVE |
| MISC-COMPONENT | Miscellaneous component | 2 EP | ALTERNATIVE |
| MULTI-PORT-COMPONENT | Multi-port component | Varies | ALTERNATIVE |
| VENT-DRAIN | Vent/drain point | 2 EP or standalone | ALTERNATIVE |
| SPOOL-BREAK | Spool break marker | Standalone | Confirmed |
| JOINT | Joint/coupling | 2 EP | ALTERNATIVE |
| SPECIAL-ITEM-1 to 5 | User-defined special items | Varies | ALTERNATIVE |

# Appendix C:  Validated Attribute Keyword List

All indented attribute keywords grouped by function. Column 5+ (4-space indent) required unless noted.

### Geometry — Coordinates

END-POINT (4 values: X Y Z OD + optional end-treatment code)

CENTRE-POINT (3 values for BEND/TEE; 4 values for OLET)

BRANCH1-POINT (4 values: X Y Z OD)

BRANCH2-POINT (4 values: X Y Z OD)

CO-ORDS (4 values for SUPPORT; 3 values for all others)

### Geometry — Fitting

SKEY (symbol key, 2–6 chars, last 2 = end condition code)

ANGLE (hundredths of degrees: 90°=9000)

BEND-RADIUS (centreline bend radius in UNITS-CO-ORDS)

LONG-RADIUS (long radius value)

SHORT-RADIUS (short radius value)

SUPPORT-DIRECTION (direction of support)

SUPPORT-TYPE (type code)

SPINDLE-DIRECTION (valve spindle: UP/DOWN/NORTH/SOUTH/EAST/WEST)

WINDOW-DIRECTION (instrument display direction)

### Material & Specification

ITEM-CODE (material catalogue code)

DESCRIPTION (material description text)

PIPING-SPEC (piping specification code)

MATERIAL (material designation)

GRADE (material grade)

HEAT-NUMBER (heat/batch number)

SCHEDTHK (schedule/thickness code)

WALL-THICKNESS (numeric value)

WEIGHT (component weight in declared weight units)

LENGTH (component length)

SIZE (size descriptor)

QUANTITY (quantity count)

RECORD (reference record number)

### Fabrication Control

CATEGORY (FABRICATION | ERECTION | OFFSHORE)

FABRICATION-ITEM (marks item as fabrication scope)

ERECTION-ITEM (marks item as erection scope)

OFFSHORE-ITEM (marks item as offshore scope)

SHOP-WELD (shop weld identifier)

WELD-NUMBER (weld number/identifier)

### Bolt Data (within BOLT block or GASKET/NOZZLE associated)

BOLT-DIA (bolt diameter)

BOLT-ITEM-CODE (bolt catalogue code)

BOLT-QUANTITY (number of bolts)

BOLT-LENGTH (bolt length)

### Branch & Connection

BRANCH-TABLE (branch connection data)

CONNECTION-REFERENCE (equipment connection tag — used with END-CONNECTION-EQUIPMENT)

PIPELINE-REFERENCE (continuation pipeline — used with END-CONNECTION-PIPELINE)

### Component Identity

COMPONENT-IDENTIFIER (numeric ID for this component)

MASTER-COMPONENT-IDENTIFIER (links weld to its parent component)

MATERIAL-IDENTIFIER (material record reference)

REPEAT-WELD-IDENTIFIER (links repeat weld callouts)

UCI (unique component identifier — GUID format)

LOCATION (weld location: CP or B1P — used with WELD block)

### User-Defined

COMPONENT-ATTRIBUTE1 through COMPONENT-ATTRIBUTE10 (custom pipeline-level data)

ATTRIBUTE5, ATTRIBUTE7, ATTRIBUTE10, ATTRIBUTE11 (pipeline header attributes)

### Messages (must be at column 5 when inside component)

MESSAGE (plain text annotation)

MESSAGE-SQUARE, MESSAGE-ROUND, MESSAGE-CIRCLE, MESSAGE-POINTED (boxed messages)

MESSAGE-TRIANGLE (max 2 chars), MESSAGE-DIAMOND (max 3 chars)

MESSAGE-DOUBLE-CIRCLE, MESSAGE-ELLIPSE (ISOGEN v10+)

TEXT (text data for MESSAGE entries)

UNDIMENSIONED (suppress dimensioning of message position)

### Pipe Bore

PIPE-BORE (nominal bore specification)

### Additional Items (at column 5 when associated, column 1 when standalone)

ADDITIONAL-ITEM (extra material not from regular component data)

ITEM-GROUP (category: BOLT/FITT/FLAN/GASK/INST/MISC/PIPE/SUPP/VALV)

# Appendix D:  ISOGEN Runtime Error Code → Validator Rule Mapping

If a PCF passes all validator rules but ISOGEN still fails, the remaining error codes below indicate which rules need strengthening or which new rules are needed.

| ISOGEN Code | Error Description | Validator Rule(s) | Sev | Catch Status |
| --- | --- | --- | --- | --- |
| 2022 | Pipeline disconnected (gap >8mm) | T-001 | ERROR | Confirmed catch |
| 2024 | Serious pipeline disconnection | T-001 | FATAL-equiv | Confirmed catch |
| 2001 | Bore/coordinate unit mismatch | U-001 | ERROR | Confirmed catch |
| 1001 | UNITS-BORE/CO-ORDS inconsistency | U-001, H-004, H-006 | ERROR | Confirmed catch |
| 47 | Unable to extract bore units | H-003, H-004 | FATAL | Confirmed catch |
| 52 | Unable to extract bore units (alt) | H-003, H-004 | FATAL | Confirmed catch |
| 2 | Style/FLS folder missing | H-001, H-002 | FATAL | Partial — FLS existence not checkable |
| 1010 | Options file not found | H-001, H-002 | FATAL | Partial — only filename syntax checked |
| 28 | Invalid file extension | F-001 | INFO | Partial — validator checks content, not filename |
| N/A | SUPPORT END-POINT instead of CO-ORDS | C-007 | ERROR | Confirmed catch |
| N/A | ANGLE in degrees not hundredths | A-001 | ERROR | Confirmed catch |
| N/A | CENTRE-POINT with OD in BEND/TEE | R-006, K-002 | ERROR | Confirmed catch |
| N/A | TAB in file kills keyword recognition | F-002 | FATAL | Confirmed catch |
| N/A | OLET missing CENTRE-POINT or B1P | C-006 | ERROR | Confirmed catch (v2.0 new) |
| N/A | BOLT CO-ORDS with 4 values | K-003b | ERROR | Confirmed catch (v2.0 new) |
| N/A | REDUCER with same OD both ends | S-001 | ERROR | Confirmed catch |

# Appendix E:  SKEY End Connection Code Reference

The last two characters of every SKEY denote the end connection type. Use ** as wildcard for any end condition. Source: Alias PCF Component Reference Guide.

| Code (2-char) | Connection Type | Description |
| --- | --- | --- |
| BW | Butt Weld | Standard welded connection |
| FL | Flanged | Bolted flange connection |
| SW | Socket Weld | Socket welded fitting |
| SC | Screwed | Threaded connection |
| CP | Compression | Compression fitting |
| FA | Ball and Socket | Fixed length flanged systems |
| GF | Clamped | Grayloc/clamp type fittings |
| BS | Gland | Gland connection (fixed length) |
| GL | Glued | Adhesive bonded connection |
| PF | Push Fit | Push-fit systems |
| PL | Plain End | Plain end (cut pipe) |
| LN | Liner/Nut | Hygienic liner class piping |
| MP | Male Part | Male connection |
| LC | Liner Clamped | Clamped liner connection |
| LR | Reducing Liner | Reducing liner/nut |
| ** | Wildcard | Any end connection type (use in SKEY when end condition is variable) |

# Appendix F:  ITEM-GROUP Identifiers for ADDITIONAL-ITEM

When using ADDITIONAL-ITEM for extra materials, the ITEM-GROUP value controls which section of the material list the item appears in:

| ITEM-GROUP Value | Use On |
| --- | --- |
| BOLT | Bolts and fasteners |
| FITT | Fittings (elbows, olets, tees, reducers, etc.) |
| FLAN | Flanges |
| GASK | Gaskets |
| INST | Instruments |
| MISC | Miscellaneous components (default if no ITEM-GROUP) |
| PIPE | Pipe and tube |
| SUPP | Supports and hangers |
| VALV | Valves |

Appendix G:  PCF Attribute Mapping — Complete Reference

Source: Hexagon PCF Reference Guide v15 | Alias PCF Component Reference Guide v2 | PTC Creo ISOGEN Integration Docs | CADWorx I-Configure Mapping Reference | Real-world PCF samples

PCF attribute mapping defines how data written in a PCF file flows through to (a) the isometric drawing title block, (b) the material list (BOM), and (c) weld and spool control registers. ISOGEN uses three distinct attribute levels — Pipeline-level, Component-level, and Material-level — each with its own keyword family and its own mapping target. Understanding these three levels is essential for both writing a valid PCF and for configuring I-Configure to consume the data correctly.

G.1  Three-Level Attribute Architecture

| Attribute Level | Where in PCF | Keyword Family | Maps To | Source |
| --- | --- | --- | --- | --- |
| Pipeline-level | Header section (before first component keyword) | Named keywords + ATTRIBUTE0–ATTRIBUTE99 + MISC-SPEC1–MISC-SPEC5 | Isometric drawing title block fields, drawing filename, revision cloud control | Hexagon PCF Ref Guide v15 |
| Component-level | Inside each component block (indented) | COMPONENT-ATTRIBUTE1 through COMPONENT-ATTRIBUTE10 | Component annotation on isometric, custom material list columns | Alias PCF Component Ref Guide v2 (ISOGEN 8.9+) |
| Material-level | Inside component blocks OR in standalone MATERIALS section | ITEM-CODE, DESCRIPTION, MATERIAL-ATTRIBUTE1–n, WEIGHT, LENGTH | Material/BOM list on isometric, spool material registers | Hexagon PCF Ref Guide v15 |

Key insight for the validator: Pipeline-level attributes appear at column 1 in the header section (no indentation). Component-level and Material-level attributes appear indented (column 5+) inside component blocks. A pipeline-level keyword appearing indented inside a component block is an error. A component-level keyword appearing at column 1 is also an error.

G.2  Pipeline-Level Named Attributes (Header Section)

These are fixed-function keywords that appear in the PCF header section (between ISOGEN-FILES and the first component keyword). Each has a pre-defined meaning in ISOGEN; they are NOT user-configurable. A real-world sample (3"-LSA-3250, from the Hexagon PCF Reference Guide) demonstrates these in a complete header:

ISOGEN-FILES    ISOGEN.FLS

UNITS-BORE      INCH

UNITS-CO-ORDS   MM

UNITS-BOLT-DIA  INCH

UNITS-BOLT-LENGTH INCH

PIPELINE-REFERENCE  3"-LSA-3250

REVISION        2

AREA            AR-26A

PIPING-SPEC     SP1

NOMINAL-CLASS   150#

DATE-DMY        08/11/95

PIPELINE-TEMP   150.0

| Keyword | Value Type / Format | ISOGEN Function | Maps To On Isometric | Validator Rule |
| --- | --- | --- | --- | --- |
| ISOGEN-FILES | Filename (e.g. ISOGEN.FLS) | Locates ISOGEN configuration — the FLS file references all control files (style, symbol library, border) | Not shown on drawing — process control only | H-001, H-002 |
| UNITS-BORE | MM ¦ INCH ¦ MM-HUNDREDTHS ¦ METER | Declares the unit of the 4th value (OD/bore) in END-POINT, CO-ORDS, BRANCH1-POINT lines | Bore annotation on isometric pipe runs | H-003, H-004 |
| UNITS-CO-ORDS | MM ¦ INCH ¦ MM-HUNDREDTHS ¦ METER | Declares the unit of X Y Z coordinate values in all component keypoints | Dimension values, co-ordinate annotations on isometric | H-005, H-006 |
| UNITS-BOLT-DIA | MM ¦ INCH | Unit for BOLT-DIA attribute values. Independent of UNITS-BORE. Common: INCH for bolt diameters even in metric projects | Bolt diameter annotations in BOM | NEW: H-010 |
| UNITS-BOLT-LENGTH | MM ¦ INCH | Unit for BOLT-LENGTH attribute values. Independent of UNITS-CO-ORDS | Bolt length annotations in BOM | NEW: H-011 |
| PIPELINE-REFERENCE | Free text string (line tag) | Mandatory pipeline identifier. Used as drawing title, filename root, reference on continuation sheets | Drawing title / pipeline tag field in title block | H-007, H-008 |
| REVISION | Alphanumeric (e.g. 2, A, P1) | Drawing revision identifier. Controls ISOGEN revision cloud feature. Maps to I-Configure Revision Clouds setting | Revision field in title block; triggers revision cloud highlighting on reissue | NEW: H-012 |
| AREA | Text (e.g. AR-26A, UNIT-2) | Plant area or batch identifier for this pipeline. Used to group pipelines in batch processing and for drawing numbering schemes | Area/Batch field in title block or drawing number | NEW: H-013 |
| DATE-DMY | DD/MM/YY or DD/MM/YYYY | Issue date in Day-Month-Year format. ISOGEN automatically populates date field from this value. Companion: DATE-MDY (US format MM/DD/YY) | Date field in title block | NEW: H-014 |
| DATE-MDY | MM/DD/YY or MM/DD/YYYY | Issue date in US Month-Day-Year format. Use DATE-DMY or DATE-MDY — not both | Date field in title block (US format) | NEW: H-014 |
| PIPELINE-TEMP | Numeric (temperature in project units) | Operating temperature for the pipeline. Used in isometric notes and stress analysis exports | Design temperature note on isometric | INFO — no mandatory rule |
| NOMINAL-CLASS | Text (e.g. 150#, ANSI 300, CL600) | Pressure class / rating of the pipeline. Appears on isometric as a specification note | Pressure class note in title block or specification area | INFO — no mandatory rule |
| PIPING-SPEC | Text spec code (e.g. SP1, CS150) | Pipeline piping specification code. When placed in the header it sets a default spec for all components. Components may override with their own PIPING-SPEC attribute | Specification reference on isometric | INFO — pipeline-level default |
| FLOW | Integer (1 = left-to-right, 2 = right-to-left) | Sets the primary flow direction arrow on the isometric. 1 = flow arrow points left-to-right. 2 = right-to-left. ISOGEN uses this to draw the flow indicator on pipe runs | Flow direction arrow on isometric drawing | INFO — valid: 1 or 2 |
| WASTE | Numeric percentage (e.g. 10.0 = 10%) | Pipe wastage allowance percentage for cut allowances in pipe material quantities. Added to pipe lengths in material list calculations | Applied to pipe BOM quantities — does not appear as text on drawing | INFO — affects BOM only |
| PLANT-AREA | Integer or text code | Sub-area identifier within the plant. Used for batch processing segregation and material list sorting when multiple plant areas are present in one project | Material list area code; drawing sorting in batch runs | INFO — used in batch |
| REPLOT | Integer (number of replot copies) | Specifies number of copies to replot when reissuing a drawing. Used in batch processing workflows | Batch processing control — not drawn on isometric | INFO — batch only |
| SPOOL-PREFIX | Text prefix string | Prefix for spool identifiers on this pipeline. Applied to all SPOOL-BREAK / SPOOL-IDENTIFIER entries to form the full spool number (e.g. prefix "SP" + spool number "001" = "SP001") | Spool numbers on spool isometrics and spool schedule | INFO — spool control |
| WELD-PREFIX-FABRICATION | Text prefix string (e.g. "F") | Prefix applied to all fabrication weld numbers on this pipeline. ISOGEN auto-numbers welds; this prefix precedes the number | Weld number callouts on isometric (fabrication welds) | INFO — weld numbering |
| WELD-PREFIX-ERECTION | Text prefix string (e.g. "E") | Prefix for erection (field) weld numbers | Weld number callouts on isometric (erection welds) | INFO — weld numbering |
| WELD-PREFIX-GENERAL | Text prefix string | General weld prefix when not differentiated between fabrication/erection | General weld callouts | INFO |
| WELD-PREFIX-OFFSHORE | Text prefix string | Prefix for offshore weld numbers | Offshore weld callouts | INFO |
| MISC-SPEC1 to MISC-SPEC5 | Free text (any pipeline specification data) | 5 free-format pipeline specification fields. Mapped via I-Configure to any title block text field. Commonly used for: fluid service, insulation type, test pressure, corrosion allowance, design pressure | User-defined title block fields (configured in I-Configure border file) | INFO — user-defined |

G.3  User-Defined Pipeline Attributes: ATTRIBUTE0 to ATTRIBUTE99

ISOGEN provides 100 numbered pipeline-level attribute slots (ATTRIBUTE0 through ATTRIBUTE99) in the PCF header section. These are free-format user-defined fields that CAD systems map from their internal pipeline properties to the PCF. ISOGEN then routes these values to designated positions on the isometric drawing title block, as configured in I-Configure.

The mapping mechanism is: PCF header keyword → I-Configure ATEXT setting → position on drawing frame. This is a three-step chain: the PCF carries the data, I-Configure defines where on the drawing to place it, and the drawing border/backing sheet has a text placeholder at that position.

| Keyword | Where in PCF | Typical CAD-System Mapping | I-Configure Mapping Target | Notes |
| --- | --- | --- | --- | --- |
| ATTRIBUTE0 to ATTRIBUTE99 | Header section (column 1) | Any CAD pipeline attribute not covered by a named PCF keyword. The CAD export configuration maps e.g. "PipelineClass" → ATTRIBUTE5 | Any ATEXT switch (e.g. ATEXT-60 for ATTRIBUTE1, ATEXT-61 for ATTRIBUTE2 … ATEXT-159 for ATTRIBUTE100) | Range confirmed: ATTRIBUTE0 to ATTRIBUTE99. Values are free text. No format restriction. Empty value is valid. |
| ATTRIBUTE1 (typical use) | Header | Project name / project number | Title block "Project" field via ATEXT configuration | Most common use: project identification |
| ATTRIBUTE2 (typical) | Header | Drawing number | Title block "Drawing Number" field | Drives dynamic file naming in some ISOGEN configurations |
| ATTRIBUTE3 (typical) | Header | Fluid service description | Title block fluid service field | Also carried by MISC-SPEC1 in many systems |
| ATTRIBUTE4 to ATTRIBUTE10 | Header | CAD-system-specific; varies by implementation | User-defined ATEXT targets | Configured per-project in I-Configure. No universal standard mapping. |
| ATTRIBUTE0 to ATTRIBUTE99 (range) | Header | Full range available for any user data. Creo maps via ISOGEN Attribute Map File. CADWorx maps via Attribute Mappings panel in I-Configure. SmartPlant 3D uses dedicated PCF export configuration | Configured in I-Configure drawing style (ATEXT-60 to ATEXT-159) | VALIDATOR NOTE: Validator should accept ATTRIBUTE0 through ATTRIBUTE99 as valid header keywords without error. Unknown ATTRIBUTEn beyond 99 should trigger WARNING. |

Validator rule implication for ATTRIBUTE0-99:

# Rule H-ATTR: Valid ATTRIBUTE keyword range in header

FOR each header_line:

IF line.split()[0].startswith("ATTRIBUTE"):

suffix = line.split()[0][9:]      # e.g. "1", "99", "100"

IF NOT suffix.isdigit(): WARN

ELIF int(suffix) > 99: WARN

# ATTRIBUTE0 to ATTRIBUTE99: valid — no error

# ATTRIBUTE100 or higher: WARNING (beyond official range)

G.4  Component-Level Attributes: COMPONENT-ATTRIBUTE1 to COMPONENT-ATTRIBUTE10

Introduced in ISOGEN version 8.9, COMPONENT-ATTRIBUTE1 through COMPONENT-ATTRIBUTE10 are indented attributes (column 5+) that appear inside individual component blocks. They carry per-component data not covered by standard attributes like ITEM-CODE or MATERIAL. Common uses: paint code, insulation specification, P&ID reference tag, equipment number, service classification.

| Keyword | Position in PCF | Value Type | Maps To | Notes |
| --- | --- | --- | --- | --- |
| COMPONENT-ATTRIBUTE1 to COMPONENT-ATTRIBUTE10 | Inside component block — indented (column 5+) | Free text, any value | Component annotation on isometric; custom columns in material list; passed to downstream stress analysis (CAESAR II etc.) | One per attribute number, per component block. Not an error if absent — they are supplementary information. |

Example — VALVE block with component attributes:

VALVE

END-POINT    6000.00  10210.00  1403.00  3

END-POINT    6000.00  10210.00  1323.60  3

SKEY         VBFL

PIPING-SPEC  SP1

ITEM-CODE    VW4

FABRICATION-ITEM

COMPONENT-ATTRIBUTE1  P&ID-REF-H152

COMPONENT-ATTRIBUTE2  INSUL-TYPE-HOT

COMPONENT-ATTRIBUTE3  PAINT-CODE-CS3

Validator rule implications:

# Rule R-001 (attribute keyword list) must include COMPONENT-ATTRIBUTE1 to COMPONENT-ATTRIBUTE10

# Rule: COMPONENT-ATTRIBUTE keywords must be indented (column 5+)

# Rule: COMPONENT-ATTRIBUTE value must be non-empty — if empty, WARNING

# Range: COMPONENT-ATTRIBUTE11 or higher = WARNING (beyond official range of 1-10)

G.5  Material-Level Attributes and the MATERIALS Section

Material data flows to the isometric BOM (Bill of Materials / material list) through two mechanisms: (1) attributes embedded in each component block, and (2) a standalone MATERIALS section at the end of the PCF. Both are valid. The MATERIALS section provides DESCRIPTION data for ITEM-CODEs referenced throughout the file — ISOGEN uses it to populate the material list description column without repeating data in every component block.

G.5.1  Material Attributes Inside Component Blocks

| Attribute Keyword | Value | BOM Column Mapped | Notes |
| --- | --- | --- | --- |
| ('ITEM-CODE', True, True, '1A5276') | Catalogue code (e.g. FAM150WN40) | Item Code / Part Number column | Primary material identifier. Used as the key to look up DESCRIPTION in MATERIALS section |
| ('DESCRIPTION', True, True, '1A5276') | Free text | Description column | May appear in component block OR in MATERIALS section lookup. In component: applies to that component only |
| ('MATERIAL', True, True, '1A5276') | Material designation (e.g. ASTM A105) | Material column | Material of construction for BOM |
| ('GRADE', True, True, '1A5276') | Grade string | Grade column in BOM | Material grade detail |
| ('HEAT-NUMBER', True, True, '1A5276') | Heat/batch number | Heat/Cert column | Traceability data |
| ('SCHEDTHK', True, True, '1A5276') | Schedule code (e.g. SCH40, SCH80) | Schedule column | Pipe schedule or wall thickness specification code |
| ('WALL-THICKNESS', True, True, '1A5276') | Numeric value | Wall thickness column | Actual wall thickness in project units |
| ('WEIGHT', True, True, '1A5276') | Numeric (kg or lb) | Weight column | Component weight for piping weight reports and lifting studies |
| ('LENGTH', True, True, '1A5276') | Numeric | Length column | Component length for pipe cut lists. For PIPE component overrides calculated length from co-ords |
| ('QUANTITY', True, True, '1A5276') | Integer or decimal | Quantity column | Explicit quantity override — normally ISOGEN counts components, but QUANTITY allows manual override |
| ('SIZE', True, True, '1A5276') | Size string (e.g. 6\" x 4\") | Size column | Size designation for reducers, olets, and other multi-size components |

G.5.2  Standalone MATERIALS Section at End of PCF

The MATERIALS section (keyword: MATERIALS at column 1) appears after all component blocks. It provides a master lookup of ITEM-CODE → DESCRIPTION pairs. When ISOGEN encounters an ITEM-CODE in a component block, it looks up the DESCRIPTION here to populate the BOM. This avoids repeating long description strings in every component block that uses the same material.

MATERIALS

ITEM-CODE    PAW-40

DESCRIPTION  PIPE -API 5L GR B SCH 40

ITEM-CODE    JFA150R2

DESCRIPTION  GASKET -CAF GR A 1/16 THK 150# RF

ITEM-CODE    FAM150WN40

DESCRIPTION  FLANGE -WN 150# RF ASTM A105 SCH 40

ITEM-CODE    BAA5/8X3.75

DESCRIPTION  STUD BOLT -CS 5/8 DIA X 3.3/4 LG WITH NUTS

Note: MATERIALS is a standalone block at column 1 — it is NOT a component type. The validator must recognise MATERIALS as a valid column-1 keyword that starts a lookup section rather than a piping component block.

# Validator rule for MATERIALS section:

STANDALONE_HEADER_KEYWORDS = {..., "MATERIALS"}

# MATERIALS is valid at column 1 — NOT in component keyword list

# Content: repeated ITEM-CODE + DESCRIPTION pairs, both indented

# No END-POINT, SKEY, or coordinate data inside MATERIALS block

# If ITEM-CODE appears without following DESCRIPTION: WARNING

G.6  MATERIAL-ATTRIBUTE Mapping (I-Configure / CADWorx Extension)

For advanced material list customisation, Hexagon's I-Configure and CADWorx support MATERIAL-ATTRIBUTE1 through MATERIAL-ATTRIBUTE-n as mapped PCF attributes. These are configured in the I-Configure Attribute Mappings panel under "Material Attribute Mapping" and allow custom CAD system data fields to flow into named columns of the ISOGEN material list. This is an I-Configure/CADWorx extension; it is not in the base PCF format specification.

| Keyword | Configuration | Example Use Case | PCF Appearance |
| --- | --- | --- | --- |
| MATERIAL-ATTRIBUTE1 to n | Configured per-project in I-Configure > Attribute Mappings > Material Attribute Mapping panel. Each numbered slot is mapped to a custom data field in the CAD specification. | Map CADWorx custom field "PipeSize" to MATERIAL-ATTRIBUTE3 → value appears as custom column in ISOGEN material list | Appears indented in component blocks: MATERIAL-ATTRIBUTE3 50. The value (50 in this example) is the custom data value from the CAD specification. |

G.7  Complete Attribute Mapping Flow (How PCF Data Reaches the Drawing)

Attribute data flows through three stages from CAD model to isometric drawing:

┌─────────────────────────────────────────────────────────────────────┐

│                    STAGE 1: CAD MODEL                               │

│  Pipeline properties:  Line Tag, Class, Area, Date, Revision,       │

│                        Fluid Service, Test Pressure, Insulation ...  │

│  Component properties: Part number, Material, Schedule, Weight       │

└────────────────────────┬────────────────────────────────────────────┘

│  CAD Export / PCF Writer maps:

│  Pipeline props → Header keywords

│  Component props → COMPONENT-ATTRIBUTE1-10

│  Material data  → ITEM-CODE, DESCRIPTION, etc.

▼

┌─────────────────────────────────────────────────────────────────────┐

│                    STAGE 2: PCF FILE                                 │

│  Header:  PIPELINE-REFERENCE, REVISION, AREA, DATE-DMY,             │

│           MISC-SPEC1-5, ATTRIBUTE0-99                                │

│  Components: END-POINT, SKEY, ITEM-CODE, COMPONENT-ATTRIBUTE1-10    │

│  Materials:  MATERIALS section with ITEM-CODE/DESCRIPTION lookups    │

└────────────────────────┬────────────────────────────────────────────┘

│  ISOGEN reads PCF and applies:

│  I-Configure ATEXT switches → title block positions

│  Material list columns → BOM layout

▼

┌─────────────────────────────────────────────────────────────────────┐

│                    STAGE 3: ISOMETRIC DRAWING                        │

│  Title block:   Line tag, revision, area, date, specification        │

│  Pipe runs:     Flow arrows, bore annotations, co-ordinates          │

│  Components:    Symbol, item tag, weld numbers, spool breaks         │

│  BOM:           Item codes, descriptions, quantities, weights        │

└─────────────────────────────────────────────────────────────────────┘

G.8  New Validator Rules Arising From Mapping Attribute Research

The mapping attribute research identified several gaps in the v2.0 rule set. The following rules are added as NEW based on this appendix:

| Rule ID | Category | Severity | Trigger | Error Message Template |
| --- | --- | --- | --- | --- |
| H-010 | Header | WARNING | UNITS-BOLT-DIA present but value is not MM or INCH | H-010: Line {lineNo} — UNITS-BOLT-DIA VALUE "{val}" INVALID. Permitted: MM, INCH only. |
| H-011 | Header | WARNING | UNITS-BOLT-LENGTH present but value is not MM or INCH | H-011: Line {lineNo} — UNITS-BOLT-LENGTH VALUE "{val}" INVALID. Permitted: MM, INCH only. |
| H-012 | Header | INFO | REVISION keyword present but has no value token | H-012: Line {lineNo} — REVISION HAS NO VALUE. Provide revision identifier (e.g. REVISION 2 or REVISION A). |
| H-013 | Header | INFO | AREA keyword present but has no value token | H-013: Line {lineNo} — AREA HAS NO VALUE. Provide plant area code (e.g. AREA AR-26A). |
| H-014 | Header | WARNING | DATE-DMY value does not match DD/MM/YY or DD/MM/YYYY pattern; OR DATE-MDY value does not match MM/DD/YY pattern | H-014: Line {lineNo} — DATE VALUE "{val}" does not match expected format. DATE-DMY: DD/MM/YY. DATE-MDY: MM/DD/YY. |
| H-015 | Header | WARNING | Both DATE-DMY and DATE-MDY appear in the same header | H-015: DATE-DMY (line {l1}) and DATE-MDY (line {l2}) BOTH PRESENT. Use only one date format. ISOGEN will use the last occurrence. |
| H-016 | Header | INFO | FLOW keyword present but value is not 1 or 2 | H-016: Line {lineNo} — FLOW VALUE "{val}" INVALID. Permitted: 1 (left-to-right) or 2 (right-to-left). |
| H-017 | Header | WARNING | ATTRIBUTE keyword present with suffix > 99 (e.g. ATTRIBUTE100) | H-017: Line {lineNo} — ATTRIBUTE{n} EXCEEDS VALID RANGE. ATTRIBUTE0 to ATTRIBUTE99 are valid (100 slots). ATTRIBUTE{n} is outside the documented range. |
| H-018 | Header | INFO | COMPONENT-ATTRIBUTE keyword appears at column 1 (should be indented inside a component block) | H-018: Line {lineNo} — COMPONENT-ATTRIBUTE{n} AT COLUMN 1. This is a component-level attribute; it must appear indented (4+ spaces) inside a component block. |
| H-019 | Header | INFO | MATERIALS keyword appears anywhere other than after all component blocks | H-019: Line {lineNo} — MATERIALS SECTION APPEARS BEFORE COMPONENT BLOCKS. MATERIALS lookup section should be the last section in the PCF. |
| R-009 | Attribute | WARNING | COMPONENT-ATTRIBUTE with number outside 1–10 inside component block | R-009: Line {lineNo} — COMPONENT-ATTRIBUTE{n} OUTSIDE VALID RANGE 1-10. Only COMPONENT-ATTRIBUTE1 to COMPONENT-ATTRIBUTE10 are officially supported (ISOGEN 8.9+). |
| R-010 | Attribute | INFO | ITEM-CODE appears in MATERIALS section without a following DESCRIPTION line | R-010: Line {lineNo} — ITEM-CODE "{code}" IN MATERIALS SECTION HAS NO DESCRIPTION. Add DESCRIPTION line immediately after this ITEM-CODE. |

G.9  Important: U-001 Correction — Mixed Units Are Valid and Common

⚠ CORRECTION TO RULE U-001: The real-world PCF sample from the Hexagon PCF Reference Guide uses UNITS-BORE INCH  +  UNITS-CO-ORDS MM simultaneously — metric coordinates with imperial bore sizes. This is a documented and valid configuration, not an error.

Rule U-001 in v2.0 flags mixed metric/imperial as an ERROR. This is incorrect. The actual ISOGEN constraint is more specific: UNITS-BORE and UNITS-CO-ORDS must each be individually valid, but they do not need to be in the same system. ISOGEN handles the conversion internally. The real source of ISOGEN error 2001 is inconsistent numeric scale within coordinates — not the unit declaration mismatch itself. Rule U-001 should be revised as follows:

# REVISED U-001 logic:

# VALID: UNITS-BORE INCH + UNITS-CO-ORDS MM  (imperial bore, metric coords)

# VALID: UNITS-BORE MM + UNITS-CO-ORDS MM     (all metric)

# VALID: UNITS-BORE INCH + UNITS-CO-ORDS INCH (all imperial)

# ERROR: UNITS-BORE value not in {MM, INCH, MM-HUNDREDTHS, METER} → H-004

# ERROR: UNITS-CO-ORDS value not in {MM, INCH, MM-HUNDREDTHS, METER} → H-006

# WARNING only (not ERROR): if UNITS-BORE and UNITS-CO-ORDS use

#   different sub-systems within the same measurement system

#   e.g. UNITS-BORE MM + UNITS-CO-ORDS METER → W (both metric, unusual scale mix)

# The true source of ISOGEN 2001 is scale mismatch in actual coordinate

# values vs declared units — caught by U-002 (OD magnitude sanity check)

Updated U-001 rule status: DOWNGRADE from ERROR to WARNING. Only warn when UNITS-BORE and UNITS-CO-ORDS use different sub-categories (e.g. MM-HUNDREDTHS vs INCH). Cross-system mixing of INCH bore + MM coords is explicitly valid per Hexagon PCF Reference Guide sample data.

G.10  Quick Lookup: Attribute Keyword → Location → Validator Rule

| Keyword(s) | Location | Col | Rule | Sev |
| --- | --- | --- | --- | --- |
| ISOGEN-FILES | Header (must be line 1) | 1 | H-001, H-002 | FATAL |
| UNITS-BORE | Header | 1 | H-003, H-004 | FATAL |
| UNITS-CO-ORDS | Header | 1 | H-005, H-006 | FATAL |
| UNITS-BOLT-DIA | Header | 1 | H-010 (NEW) | WARN |
| UNITS-BOLT-LENGTH | Header | 1 | H-011 (NEW) | WARN |
| PIPELINE-REFERENCE | Header | 1 | H-007, H-008 | FATAL/ERR |
| REVISION | Header | 1 | H-009, H-012 (NEW) | WARN/INFO |
| AREA | Header | 1 | H-013 (NEW) | INFO |
| DATE-DMY / DATE-MDY | Header | 1 | H-014, H-015 (NEW) | WARN |
| FLOW | Header | 1 | H-016 (NEW) | INFO |
| WASTE | Header | 1 | INFO only (no rule) | INFO |
| PLANT-AREA | Header | 1 | INFO only (no rule) | INFO |
| PIPELINE-TEMP | Header | 1 | INFO only (no rule) | INFO |
| NOMINAL-CLASS | Header | 1 | INFO only (no rule) | INFO |
| PIPING-SPEC (header default) | Header | 1 | INFO only (no rule) | INFO |
| SPOOL-PREFIX | Header | 1 | INFO only (no rule) | INFO |
| WELD-PREFIX-FABRICATION | Header | 1 | INFO only (no rule) | INFO |
| WELD-PREFIX-ERECTION | Header | 1 | INFO only (no rule) | INFO |
| MISC-SPEC1 to MISC-SPEC5 | Header | 1 | Valid — no error | INFO |
| ATTRIBUTE0 to ATTRIBUTE99 | Header | 1 | H-017 if > 99 (NEW) | WARN if >99 |
| ATTRIBUTE100 and above | Header | 1 | H-017 (NEW) | WARN |
| COMPONENT-ATTRIBUTE1 to COMPONENT-ATTRIBUTE10 | Component block | 5+ | R-001, R-009 (NEW) | WARN if >10 |
| ITEM-CODE | Component block | 5+ | R-001 (valid attr) | INFO |
| DESCRIPTION | Component block or MATERIALS | 5+ | R-010 in MATERIALS | INFO |
| MATERIAL, GRADE, HEAT-NUMBER | Component block | 5+ | R-001 (valid attrs) | INFO |
| SCHEDTHK, WALL-THICKNESS | Component block | 5+ | R-001 (valid attrs) | INFO |
| WEIGHT, LENGTH, QUANTITY | Component block | 5+ | R-001 (valid attrs) | INFO |
| MATERIALS | End of file (standalone) | 1 | H-019 if before components | INFO |
| PIPING-SPEC (inside component) | Component block | 5+ | R-001 (valid attr) | INFO |

Reference: Hexagon PCF Reference Guide v15 (docs.hexagonppm.com) | Alias PCF Component Reference Guide v2 (Alias Limited, 2002) | PTC Creo ISOGEN Attributes documentation | CADWorx I-Configure Attribute Mappings documentation | Real-world PCF sample from Far East Piper Reference (3"-LSA-3250)

Appendix F:  CSV-to-PCF Column Mapping Reference

Source: Actual export_sys-1.xlsx (A1:Z115, 26 columns, 114 data rows). This appendix maps every column in the piping stress analysis CSV export to its corresponding PCF attribute, defines the transformation required, and proposes new PCF attributes for data that has no native PCF equivalent.

F.0  Legend — Column Mapping Status Codes

DIRECT  = CSV value maps directly to PCF attribute with no modification     TRANSFORM  = CSV value requires processing (strip units, format, convert) before writing to PCF     LOGIC  = Complex routing: requires conditional logic, row grouping, or multi-column decision     PROPOSED  = CSV column has NO native PCF attribute — mapping proposed using COMPONENT-ATTRIBUTE or MISC-SPEC     NOT_MAPPED  = Column is not written to PCF; used for processing control or informational only     PIPELINE  = Pipeline-level header attribute (written to PCF header, not inside component blocks)

F.1  Component Type Mapping  (CSV "Type" Column → PCF Identifier)

The "Type" column is the primary component selector. Each 4-character code must be substituted with the correct PCF component keyword. The transformation ranges from simple expansion to full substitution:

| CSV Type | PCF Keyword | Transformation | Notes |
| --- | --- | --- | --- |
| BRAN | PIPE | Truncation: remove last char ("BRAN" → use lookup → PIPE). Direct substitute. | Standard pipe segment with 2 END-POINTs. |
| FLAN | FLANGE | Expansion: "FLAN" → "FLANGE". No truncation needed — direct lookup. | Default SKEY = FLWN (Weld Neck RF). Confirm from spec if alternate face required. |
| GASK | GASKET | Expansion: "GASK" → "GASKET". | Two END-POINTs. ERECTION-ITEM. |
| ANCI | SUPPORT | Expansion: "ANCI" → "SUPPORT". (ANCI = anchor/support in stress analysis terminology) | Single CO-ORDS row (no END-POINTs). ERECTION-ITEM. NAME from NodeName. |
| TEE | TEE or TEE-SET-ON | No truncation/expansion needed — "TEE" = "TEE". Branch bore decision: if branch Bore (Point=3) ≠ run Bore (Point=1/2) → TEE-SET-ON; else → TEE. | Centre from Point=0. Branch from Point=3. Run ends (Point=1,2) belong to adjacent PIPE blocks. |
| VALV | VALVE | Expansion: "VALV" → "VALVE". Truncation of last char removed + add "E": VALV → VALVE. | componentName column often holds valve tag (e.g. "05-VBL-21475"). Use as ITEM-CODE. ERECTION-ITEM. |
| ELBO | BEND | Full substitution: "ELBO" → "BEND". (PCF uses BEND not ELBOW as component identifier) | Radius column provides bend radius. ANGLE derived from coordinate geometry or = 9000 (90°) default. FABRICATION-ITEM. |
| OLET | OLET | Direct: "OLET" = "OLET". No transformation required. | Weld-o-let / soc-o-let branch fitting. Two END-POINTs: run attachment + branch outlet. Run attachment bore = header pipe bore; branch bore from Bore column. |
| PCOM | PIPE-FIXED (or PIPE) | Proposed substitute: "PCOM" → "PIPE-FIXED". (PCOM = piping component / in-line component in stress model) Fallback: treat as PIPE if PIPE-FIXED not available in project ISOGEN library. | PCOM appears to represent inline piping components not classified as standard fittings. If no ISOGEN symbol matches, use PIPE with custom SKEY. FABRICATION-ITEM. Confirm with project ISOGEN symbol library. |

⚠  Note on VALV → VALVE transformation: "VALV" is a truncation of "VALVE". The reverse transformation (expansion) truncates and appends: VALV[4 chars] + "E" = VALVE. In the mapping lookup table this is simply a direct key-value substitute: {"VALV": "VALVE"}.

F.2  Complete Column-to-PCF Attribute Mapping  (All 26 Columns)

Every column in the CSV export (A–Z) is mapped below. Columns appear in the same order as in the source spreadsheet. Rows shaded in purple indicate proposed new mappings with no native PCF equivalent.

| CSV Column (A–Z) | PCF Attribute | Transformation Required | Status | Notes / Special Exceptions |
| --- | --- | --- | --- | --- |
| Sequence | Not written to PCF | Sort rows by Sequence (ascending) before grouping by RefNo. Sequence determines processing order only. | NOT_MAPPED | Row order control only. Does not appear in PCF output. |
| NodeNo | Not written to PCF directly | Cross-reference / traceability only. May be used in logging / error reporting. | NOT_MAPPED | Informational node identifier from the stress analysis model. Not a PCF attribute. |
| NodeName | NAME (on SUPPORT blocks) | Direct copy when non-empty. Maps to SUPPORT NAME attribute (col 5, indented). Also used as NodeName in continuation references. | DIRECT | Only populated for ANCI components (e.g. "PS00178.1"). Empty for most components. Do not write NAME if NodeName is empty. |
| componentName | ITEM-CODE (when ≠ "unset") / NAME (ANCI) | "unset" → skip (no ITEM-CODE generated from this field). Non-"unset" value → use as ITEM-CODE or component tag. For ANCI: value matches NodeName; use as NAME attribute. | LOGIC | "componentName" holds the equipment/instrument tag or valve tag (e.g. "05-VBL-21475"). When set, this IS the item tag — map to ITEM-CODE. When "unset", ITEM-CODE must be derived from material spec lookup. |
| Type | PCF Component Keyword (col 1) | Apply component type substitution table (see F.2). 4-char code → full PCF identifier. e.g. BRAN → PIPE, FLAN → FLANGE, VALV → VALVE. The Type column IS the primary component selector. | TRANSFORM | Critical routing field. All rows in a RefNo group share the same Type. OLET and PCOM require special handling — see F.3. |
| RefNo | Component block grouper (not written to PCF) | Rows with identical RefNo belong to one PCF component block. RefNo appears as an Excel formula (=67130482/xxxx). Extract the divisor (xxxx) as the component instance ID for grouping. | LOGIC | The formula evaluates to a floating-point number. Do NOT use the evaluated float as the group key — it may have precision drift. Use the raw formula string or extract the integer divisor (e.g. "1664") for reliable grouping. |
| Point | Determines PCF positional attribute: END-POINT / CENTRE-POINT / BRANCH1-POINT / CO-ORDS | Routing logic: • 1 → first END-POINT (most components) • 2 → second END-POINT (most components) • 0 → CENTRE-POINT (BEND, TEE) or CO-ORDS (SUPPORT) • 3 → BRANCH1-POINT (TEE, TEE-SET-ON) | LOGIC | Combined with PPoint for disambiguation. For ANCI (SUPPORT): Point is absent/0 — single CO-ORDS row. For ELBO (BEND): Point 1 and 2 are END-POINTs; Point 0 (if present) is CENTRE-POINT. |
| PPoint | Secondary routing qualifier | Mirrors Point in most cases. Used when a component has multiple rows with the same Point value. Process as secondary sort key within a RefNo group. | LOGIC | Example: VALV row 23 has Point=1, PPoint=2 (reversed from standard). Validator/converter should use PPoint to determine actual IN/OUT endpoint assignment when Point alone is ambiguous. |
| Bore | Bore value on END-POINT, BRANCH1-POINT (4th token) | Strip "mm" suffix → integer. "400mm" → 400. Append as 4th space-separated value on END-POINT and BRANCH1-POINT lines. DO NOT append on CENTRE-POINT or CO-ORDS lines. | TRANSFORM | All sample data uses "mm" suffix. For INCH projects the suffix would be absent or use " in". Always strip any trailing unit label. Integer for MM units (400, not 400.00). |
| O/D | No native PCF attribute. Proposed: COMPONENT-ATTRIBUTE1 | Numeric value (mm). Do not strip units — field is already numeric in Excel. Proposed mapping: write as COMPONENT-ATTRIBUTE1 <value> (col 5, indented inside component block). | PROPOSED | PCF has no OD field. OD is needed for schedule validation and downstream stress checks. Proposed new column header: "OD_mm (Proposed New Column: COMPONENT-ATTRIBUTE1)". Example: COMPONENT-ATTRIBUTE1 406.4 |
| Wall Thickness | WALL-THICKNESS (component attribute) | Numeric value. Zero (0.000) means "use specification default" — do NOT write WALL-THICKNESS if value is 0. Write only when > 0. | LOGIC | PCF supports WALL-THICKNESS as a component-level attribute (col 5). Most CSV rows have 0.000 indicating spec-driven thickness. Only override spec when CSV has an explicit non-zero value. |
| Corrosion Allowance | No native PCF attribute. Proposed: COMPONENT-ATTRIBUTE2 | Numeric value (mm). Write as COMPONENT-ATTRIBUTE2 <value> when non-zero. | PROPOSED | PCF has no CA field. Corrosion allowance is a stress/material parameter. Proposed mapping: COMPONENT-ATTRIBUTE2. Example: COMPONENT-ATTRIBUTE2 3.0 |
| Radius | Used to compute CENTRE-POINT on BEND/ELBO components | Not written directly to PCF. For ELBO: Radius value (e.g. 533.000mm) = bend radius. Use to compute CENTRE-POINT geometrically when no Point=0 row is present. Also validates LR vs SR elbow classification. | LOGIC | LR elbow: radius = 1.5 × nominal bore. SR elbow: radius = 1.0 × bore. Example: 400NB LR = 600mm radius, SR = 400mm. The Radius field removes the need to compute the bend radius from coordinate geometry. |
| SIF | No native PCF attribute. Proposed: COMPONENT-ATTRIBUTE3 | Numeric stress intensification factor. Write as COMPONENT-ATTRIBUTE3 <value> when non-zero (SIF > 1.0 is significant). | PROPOSED | SIF (Stress Intensification Factor) is used downstream by stress analysis tools that consume PCF (e.g. CAESAR II Caesar/PCF import). Proposed: COMPONENT-ATTRIBUTE3. Example: COMPONENT-ATTRIBUTE3 2.1 |
| Weight | WEIGHT (component attribute) | Numeric value. Write as WEIGHT <value> (col 5) when value > 0. Zero = not provided / use calculated weight. | LOGIC | PCF supports WEIGHT natively as a component attribute. Weight is in KGS when UNITS-WEIGHT KGS is declared in header. Zero values (0.000) should be suppressed — omit WEIGHT attribute rather than write WEIGHT 0. |
| Material | MATERIAL (component attribute) | Direct text value (e.g. "CS", "ASTM A106 Gr B"). Write as MATERIAL <value> when non-empty. | DIRECT | PCF supports MATERIAL as a component-level attribute. Maps directly. Empty string → omit. In sample data most are empty ("0") — suggests material comes from spec; write only when explicitly populated. |
| East | 1st coordinate token on END-POINT / CENTRE-POINT / CO-ORDS / BRANCH1-POINT | Strip "mm" suffix. Format to 2 decimal places. "96400.000mm" → 96400.00 | TRANSFORM | Corresponds to the X / Easting coordinate in the plant coordinate system. The PCF attribute line format is: END-POINT <East> <North> <Up> <bore> |
| North | 2nd coordinate token on END-POINT / CENTRE-POINT / CO-ORDS / BRANCH1-POINT | Strip "mm" suffix. Format to 2 decimal places. "17989.400mm" → 17989.40 | TRANSFORM | Corresponds to the Y / Northing coordinate. Same processing as East. |
| Up | 3rd coordinate token on END-POINT / CENTRE-POINT / CO-ORDS / BRANCH1-POINT | Strip "mm" suffix. Format to 2 decimal places. "101968.000mm" → 101968.00 | TRANSFORM | Corresponds to the Z / Elevation coordinate. Same processing as East. |
| Rigid | Pipeline boundary indicator. No direct PCF attribute — drives END-CONNECTION logic | "START" → marks the upstream termination of the pipeline. "END" → marks downstream termination. Generate END-CONNECTION-NULL or END-CONNECTION-EQUIPMENT block at the corresponding co-ordinate. Note: "Rigid" column name is misleading — it actually carries pipeline boundary flags in this export. | LOGIC | Confirmed by sample data: Row 1 (BRAN) Rigid=START, Row 5 (FLAN) Rigid=END. The "Rigid" column is repurposed by this CAD export to carry START/END pipeline flags. It does NOT indicate structural rigidity. This is a known quirk of this particular export format. |
| Status | No direct PCF attribute. Informational / filter control. | Not written to PCF. May be used for batch filtering (e.g. exclude "DETAIL" status rows). | NOT_MAPPED | Empty in most sample rows. Potential values are project-specific status codes. No PCF equivalent. |
| Pressure | No direct standard PCF component attribute. Proposed: Header MISC-SPEC1 or PIPELINE-TEMP equivalent | If non-zero, write as MISC-SPEC1 <value> in pipeline header (or as PIPELINE-TEMP if project-specific mapping exists). Component-level pressure → COMPONENT-ATTRIBUTE4. | PROPOSED | PCF does not have a standard PRESSURE component attribute. Pipeline-level design pressure may map to MISC-SPEC fields in the header. Per-component pressure (e.g. for equipment nozzles) → COMPONENT-ATTRIBUTE4. Proposed header: "PRESSURE (Proposed New Column: MISC-SPEC1 or COMPONENT-ATTRIBUTE4)". |
| Restraint Type | No native PCF attribute. Proposed: COMPONENT-ATTRIBUTE5 (ANCI/SUPPORT only) | Write as COMPONENT-ATTRIBUTE5 <value> inside SUPPORT component blocks when non-empty. Only applicable to ANCI rows. | PROPOSED | Restraint type classifies the support behaviour (e.g. guide, anchor, spring hanger). PCF has no native field. Proposed mapping: COMPONENT-ATTRIBUTE5 on SUPPORT blocks. Passed downstream to stress analysis tools that can read COMPONENT-ATTRIBUTE fields. |
| Restraint Stiffness | No native PCF attribute. Proposed: COMPONENT-ATTRIBUTE6 (ANCI/SUPPORT only) | Numeric stiffness value (N/mm or lb/in). Write as COMPONENT-ATTRIBUTE6 <value> when non-zero. | PROPOSED | Spring/variable support stiffness. Only meaningful for ANCI/SUPPORT blocks. Proposed: COMPONENT-ATTRIBUTE6. |
| Restraint Friction | No native PCF attribute. Proposed: COMPONENT-ATTRIBUTE7 (ANCI/SUPPORT only) | Numeric friction coefficient. Write as COMPONENT-ATTRIBUTE7 <value> when non-zero. | PROPOSED | Friction coefficient at sliding supports. Only for ANCI blocks. Proposed: COMPONENT-ATTRIBUTE7. |
| Restraint Gap | No native PCF attribute. Proposed: COMPONENT-ATTRIBUTE8 (ANCI/SUPPORT only) | Numeric gap value (mm). Write as COMPONENT-ATTRIBUTE8 <value> when non-zero. | PROPOSED | Restraint gap (cold clearance). Only for ANCI blocks. Proposed: COMPONENT-ATTRIBUTE8. |

F.3  Special Component Handling: OLET and PCOM

F.3.1  OLET (Weld-O-Let / Branch Fitting)

OLET is a branch fitting welded onto the run pipe. In PCF it uses the OLET keyword (col 1) with two END-POINTs and a CENTRE-POINT:

OLET                                      ← Col 1

END-POINT  <E_run>  <N_run>  <Up_run>  <run_bore>   ← run attachment (Point=1)

END-POINT  <E_brn>  <N_brn>  <Up_brn>  <brn_bore>   ← branch outlet (Point=2)

CENTRE-POINT  <Ec>  <Nc>  <Upc>         ← centre (Point=0, if present)

SKEY  OL                                ← standard OLET skey

ITEM-CODE  <code>

FABRICATION-ITEM

In sample data: OLET at Seq 29, RefNo =67130482/1558, Bore=350mm, O/D=355.6. Run bore from adjacent PIPE (400mm). Branch bore from OLET Bore column (350mm). Point=1 is run attachment; Point=2 is branch outlet.

F.3.2  PCOM (Pipeline Component — Inline Fitting)

PCOM represents generic inline piping components not classifiable as standard fittings (e.g. pipe nipples, instrument tapping pieces, in-line instruments). PCF does not have a universal "PCOM" keyword. Two options:

Option A: Map PCOM → PIPE-FIXED (for short rigid sections with specific end-to-end dimensions). PIPE-FIXED uses the same syntax as PIPE but signals ISOGEN to not calculate a cut length.

Option B: Map PCOM → PIPE with custom SKEY and COMPONENT-ATTRIBUTE1 to identify the component type for downstream processing.

Recommendation: Use PIPE-FIXED as primary mapping. If ISOGEN symbol PIPE-FIXED is not in the project library, fall back to PIPE. Document the decision in project configuration.

F.4  RefNo Column — Excel Formula Extraction

The RefNo column contains Excel formulas of the form =67130482/xxxx where xxxx is the component instance ID. Two reliable extraction methods:

# Method 1: Extract integer divisor from formula string

refno_raw = "=67130482/1664"          # raw cell formula

comp_id = refno_raw.split("/")[-1]    # → "1664" (use as group key)

# Method 2: Round evaluated float to nearest integer

evaluated = 40342.83774              # openpyxl returns float

comp_id = str(round(67130482 / evaluated))  # → "1664"

# WARNING: Do NOT use the float directly as group key:

# 40342.837... ≠ 40342.838... due to floating-point precision

# Different rows of the same component MAY evaluate to slightly different floats

F.5  Coordinate and Bore Unit Stripping Rules

| CSV Column | Sample Value | Strip Rule | PCF Output | Usage Context |
| --- | --- | --- | --- | --- |
| East / North / Up | 96400.000mm | Remove trailing "mm". Parse float. Format f"{v:.2f}" | 96400.00 | END-POINT, CENTRE-POINT, CO-ORDS, BRANCH1-POINT (1st-3rd tokens) |
| Bore | 400mm | Remove trailing "mm". Parse float. Convert to int. | 400 | END-POINT, BRANCH1-POINT (4th token). NOT on CENTRE-POINT/CO-ORDS. |
| O/D | 406.4 | Already numeric. No strip needed. | 406.4 | Proposed: COMPONENT-ATTRIBUTE1 value |
| Wall Thickness | 0.000 | Numeric. Write only if > 0. | 12.7 (example) | WALL-THICKNESS attribute. Suppress zeros. |
| Radius | 533.000 | Numeric. Use for CENTRE-POINT computation on ELBO. Do not write to PCF. | N/A (used in calc only) | BEND centre-point geometry. Radius in mm. |
| Weight | 0.000 | Numeric. Write WEIGHT <value> only if > 0. | 45.2 (example) | WEIGHT attribute. Suppress zeros. |

F.6  Summary — Proposed New CSV Columns for Complete PCF Mapping

The following CSV columns currently have no native PCF attribute. If the project requires this data in the PCF (e.g. for downstream stress analysis, insulation scheduling, or BOM enrichment), the recommended approach is to add corresponding COMPONENT-ATTRIBUTE or MISC-SPEC fields to the PCF output. These should also be added as new named columns to the CSV template.

| Existing CSV Column | Proposed PCF Attribute | Attribute Type | Applies To | Proposed New CSV Column Header |
| --- | --- | --- | --- | --- |
| O/D | COMPONENT-ATTRIBUTE1 | Component-level | All components | OD_mm (Proposed: COMPONENT-ATTRIBUTE1) |
| Corrosion Allowance | COMPONENT-ATTRIBUTE2 | Component-level | All components | CorrosionAllowance_mm (Proposed: COMPONENT-ATTRIBUTE2) |
| SIF | COMPONENT-ATTRIBUTE3 | Component-level | Fittings, bends, tees | SIF_Factor (Proposed: COMPONENT-ATTRIBUTE3) |
| Pressure | MISC-SPEC1 or COMPONENT-ATTRIBUTE4 | Pipeline header or component | Pipeline header / nozzles | DesignPressure_bar (Proposed: MISC-SPEC1) |
| Restraint Type | COMPONENT-ATTRIBUTE5 | Component-level | ANCI / SUPPORT only | RestraintType (Proposed: COMPONENT-ATTRIBUTE5) |
| Restraint Stiffness | COMPONENT-ATTRIBUTE6 | Component-level | ANCI / SUPPORT only | RestraintStiffness_N_mm (Proposed: COMPONENT-ATTRIBUTE6) |
| Restraint Friction | COMPONENT-ATTRIBUTE7 | Component-level | ANCI / SUPPORT only | RestraintFriction_coeff (Proposed: COMPONENT-ATTRIBUTE7) |
| Restraint Gap | COMPONENT-ATTRIBUTE8 | Component-level | ANCI / SUPPORT only | RestraintGap_mm (Proposed: COMPONENT-ATTRIBUTE8) |

Source data: export_sys-1.xlsx (26 columns, A1:Z115). Component types identified: BRAN, FLAN, GASK, ANCI, TEE, VALV, ELBO, OLET, PCOM. Mapping validated against Hexagon PCF Reference Guide v15 and Alias PCF Component Reference Guide v2.

Appendix F — Correction Sheet  (v2.2 → v2.3)

Legend — Change Colour Convention

BLUE / Blue background  = Content DELETED or found conceptually wrong in v2.2       RED / Red background  = Content ADDED or corrected in v2.3

BOLD  = Confirmed from PCF samples or Hexagon documentation (high-confidence)       Normal weight  = Inferred / proposed (lower confidence)

F-C.1  Summary of Conceptual Corrections

Three root misconceptions were identified in Appendix F (v2.2). Each is corrected below.

Correction 1 — O/D (OD) is NOT a new attribute  [F.2 Row "O/D"]

DELETED: Proposed COMPONENT-ATTRIBUTE1 = OD_mm

REASON: OD, NPS, NB, Nominal Bore, Pipe Size — these are all representations of pipe diameter in different conventions/units. The END-POINT line already carries pipe size as its 4th token:

END-POINT  96400.00  17989.40  101968.00  400   ← "400" IS the pipe size (NB 400 = DN400)

END-POINT  E  N  Up  <bore>                     ← bore token = OD/NPS/NB equivalent

CORRECTED ACTION: Remove O/D from F.6 proposed-new-column table entirely. Mark O/D row in F.2 as NOT_REQUIRED — already encoded in END-POINT bore token.

Correction 2 — Restraint attributes do NOT belong in COMPONENT-ATTRIBUTEs  [F.2 Rows 23–26]

DELETED: Restraint Type → CA5, Restraint Stiffness → CA6, Restraint Friction → CA7, Restraint Gap → CA8

REASON (Restraint Type): PCF handles restraint direction natively using SUPPORT-DIRECTION (UP, DOWN, NORTH, SOUTH, EAST, WEST). This is a core PCF attribute, not an extension field. Example of correct PCF for a downward rest:

SUPPORT

CO-ORDS  96400.00  17186.40  101968.00

SKEY  01FS

SUPPORT-DIRECTION  DOWN         ← direction IS the "Restraint Type" natively

ERECTION-ITEM

REASON (Stiffness, Friction, Gap): These are parameters of the support behaviour model in CAESAR II, controlled via PCF_RES_MAP.TXT — the support mapping file. They are NOT written into the PCF file as COMPONENT-ATTRIBUTEs. The mapping file entry carries MU= (friction) and GAP= (gap) directly:

# PCF_RES_MAP.TXT syntax:

REST_SLIDE   1  +Y  MU=0.3          ← friction coefficient from Restraint Friction

STOP_X       1  X   GAP=0.5         ← gap from Restraint Gap

GUIDE_SOFT   2  +Y  MU=0.3  GUIDE MU=0.3

↑

Support name must match the NAME attribute in the PCF SUPPORT block

CORRECTED ACTION: Restraint Stiffness, Friction, and Gap → populate PCF_RES_MAP.TXT (MU= / GAP= entries). Restraint Type → already native via SUPPORT-DIRECTION. Remove CA5–CA8 restraint entries from F.6.

Correction 3 — SIF is NOT a COMPONENT-ATTRIBUTE  [F.2 Row "SIF"]

DELETED: SIF → COMPONENT-ATTRIBUTE3

REASON: SIF (Stress Intensification Factor) in CAESAR II is controlled by PCF_SIF_MAP.TXT, which maps ISOGEN SKEY codes directly to C2 intersection types. The SIF is derived automatically from the component geometry — it is NOT manually injected per-component via COMPONENT-ATTRIBUTE. The SIF CSV column value is used to VALIDATE the PCF_SIF_MAP.TXT configuration, not to write a CA field.

# PCF_SIF_MAP.TXT syntax (SKEY → C2 SIF type):

TEEB   2                     ← Unreinforced Tee (SIF type 2)

TESO   3                     ← Weldolet/Set-on (SIF type 3)

TERF   1  PAD=10 MM          ← Reinforced Tee with 10mm pad

# The SKEY is already in the PCF component block:

SKEY  TESO               ← CAESAR II looks this up in PCF_SIF_MAP.TXT automatically

CORRECTED ACTION: SIF column → used for PCF_SIF_MAP.TXT validation / cross-check only. Remove CA3=SIF from F.6. CSV SIF column status → NOT_MAPPED (cross-check use only).

Correction 4 — COMPONENT-ATTRIBUTE numbering must follow CAESAR II standard

The CAESAR II "Stress ISO" import configuration defines the following standard CA slots. Any deviation would cause CAESAR II to misinterpret the imported data (e.g. treating pressure values as temperature). The v2.2 numbering was arbitrary — v2.3 aligns to the CAESAR II standard mapping:

| CA Slot | CAESAR II Field | v2.2 (WRONG — DELETED) | v2.3 (CORRECT — ADDED) |
| --- | --- | --- | --- |
| CA1 | Pressure 1 (P1) | OD_mm | Pressure (existing CSV column) |
| CA2 | Temperature 1 (T1) | Corrosion Allowance | DesignTemperature_degC (Proposed NEW column) |
| CA3 | Material (→ PCF_MAT_MAP lookup) | SIF Factor | Material (existing CSV column) |
| CA4 | Wall Thickness override | Design Pressure (was at wrong slot) | Wall Thickness (existing CSV column) |
| CA5 | Insulation Thickness | Restraint Type (wrong — use SUPPORT-DIR) | InsulationThickness_mm (Proposed NEW column) |
| CA6 | Insulation Density | Restraint Stiffness (wrong — use RES_MAP) | InsulationDensity_kg_m3 (Proposed NEW column) |
| CA7 | Corrosion Allowance | Restraint Friction (wrong — use RES_MAP MU=) | Corrosion Allowance (existing CSV column — CORRECT slot) |
| CA8 | (not defined in C2 standard) | Restraint Gap (wrong — use RES_MAP GAP=) | — REMOVED — CA8 is not a standard C2 slot |
| CA9 | Fluid Density | (not mapped in v2.2) | FluidDensity_kg_m3 (Proposed NEW column) |
| CA10 | Hydrotest Pressure | (not mapped in v2.2) | HydrotestPressure_bar (Proposed NEW column) |

F-C.2  Corrected F.6 — Proposed New CSV Columns  (Replaces v2.2 F.6)

⚠  This table REPLACES F.6 from v2.2. Red rows = new/corrected. Blue rows = removed from v2.2.

| CSV Column | CA Slot | CAESAR II Field | Attr Type | Applies To | Proposed Column Header / Action |
| --- | --- | --- | --- | --- | --- |
| Pressure | CA1 | Pressure 1 (P1) | Component-level | All components | DesignPressure_bar (existing column → correct slot) |
| (NEW COLUMN) | CA2 | Temperature 1 (T1) | Component-level | All components | DesignTemperature_degC (Proposed New Column) |
| Material | CA3 | Material (PCF_MAT_MAP lookup key) | Component-level | All components | Material (existing column → confirmed) |
| Wall Thickness | CA4 | Wall Thickness override | Component-level | All components | WallThickness_mm (existing column → confirmed) |
| (NEW COLUMN) | CA5 | Insulation Thickness | Component-level | All components | InsulationThickness_mm (Proposed New Column) |
| (NEW COLUMN) | CA6 | Insulation Density | Component-level | All components | InsulationDensity_kgm3 (Proposed New Column) |
| Corrosion Allowance | CA7 | Corrosion Allowance | Component-level | All components | CorrosionAllowance_mm (existing column — moved from CA2 to correct slot CA7) |
| (NEW COLUMN) | CA9 | Fluid Density | Component-level | All components | FluidDensity_kgm3 (Proposed New Column) |
| (NEW COLUMN) | CA10 | Hydrotest Pressure | Component-level | All components | HydrotestPressure_bar (Proposed New Column) |
| O/D | CA1 (was) | DELETED — redundant. Bore already in END-POINT 4th token. OD = NPS = NB = bore. Not a new field. | — | — | → REMOVED FROM F.6 |
| SIF | CA3 (was) | DELETED — SIF controlled by PCF_SIF_MAP.TXT keyed on SKEY. Not a COMPONENT-ATTRIBUTE. | — | — | → REMOVED FROM F.6 |
| Restraint Type | CA5 (was) | DELETED — handled natively by SUPPORT-DIRECTION in PCF block. | — | — | → REMOVED FROM F.6 |
| Restraint Stiffness | CA6 (was) | DELETED — maps to PCF_RES_MAP.TXT. Not a COMPONENT-ATTRIBUTE. | — | — | → REMOVED FROM F.6 |
| Restraint Friction | CA7 (was) | DELETED — maps to PCF_RES_MAP.TXT MU= parameter. | — | — | → REMOVED FROM F.6 |
| Restraint Gap | CA8 (was) | DELETED — maps to PCF_RES_MAP.TXT GAP= parameter. | — | — | → REMOVED FROM F.6 |

F-C.3  Corrected Mapping — Restraint CSV Columns → PCF_RES_MAP.TXT

Restraint Stiffness, Friction, and Gap from the CSV are NOT written into the PCF file. They inform the PCF_RES_MAP.TXT configuration file which CAESAR II reads at import time. The support NAME in the PCF block links the PCF component to the mapping entry.

| CSV Column | v2.2 (DELETED) | v2.3 Correct Target | PCF_RES_MAP.TXT Parameter | Example Entry |
| --- | --- | --- | --- | --- |
| Restraint Friction | COMPONENT-ATTRIBUTE7 | PCF_RES_MAP.TXT | MU=<value> | REST_SLIDE 1 +Y MU=0.30 |
| Restraint Gap | COMPONENT-ATTRIBUTE8 | PCF_RES_MAP.TXT | GAP=<value> | STOP_X 1 X GAP=0.50 |
| Restraint Stiffness | COMPONENT-ATTRIBUTE6 | PCF_RES_MAP.TXT (type definition) | Encoded in restraint type selection | SPRING_VAR 1 Y (spring type implies variable stiffness) |
| Restraint Type | COMPONENT-ATTRIBUTE5 | SUPPORT-DIRECTION (native PCF attribute) | SUPPORT-DIRECTION UP / DOWN / NORTH… | SUPPORT-DIRECTION DOWN |

F-C.4  Corrected Mapping — SIF Column → PCF_SIF_MAP.TXT  (Cross-Check Only)

The CSV SIF column value is used as a VALIDATOR cross-check against PCF_SIF_MAP.TXT, NOT as a COMPONENT-ATTRIBUTE written into the PCF. The PCF SKEY code drives the CAESAR II SIF automatically via the mapping file.

| CSV Column | v2.2 (DELETED) | v2.3 Correct Usage | How SIF Actually Works in PCF→C2 |
| --- | --- | --- | --- |
| SIF | COMPONENT-ATTRIBUTE3 | Cross-check only. Not written to PCF. | PCF SKEY (e.g. TESO) → PCF_SIF_MAP.TXT lookup → CAESAR II SIF Type 3 (Weldolet). CSV SIF value used to VERIFY mapping table is configured correctly. |

F-C.5  Summary — F.2 Row Status Changes  (Full 26-Column Table)

The following rows in the F.2 master mapping table are affected by the corrections above. F.2 itself is not reproduced here — refer to the delta below and apply when regenerating the master table.

| F.2 CSV Column | v2.2 PCF Attribute (DELETED — Blue) | v2.2 Status (Old) | v2.3 PCF Attribute (ADDED — Red) | v2.3 Status (New) |
| --- | --- | --- | --- | --- |
| O/D | COMPONENT-ATTRIBUTE1 | PROPOSED | NOT REQUIRED (bore in END-POINT token) | NOT_MAPPED |
| SIF | COMPONENT-ATTRIBUTE3 | PROPOSED | PCF_SIF_MAP.TXT (cross-check only) | NOT_MAPPED |
| Corrosion Allowance | COMPONENT-ATTRIBUTE2 | PROPOSED | COMPONENT-ATTRIBUTE7 (CA7 = correct C2 slot) | TRANSFORM |
| Pressure | MISC-SPEC1 | PROPOSED | COMPONENT-ATTRIBUTE1 (CA1 = P1 in CAESAR II) | TRANSFORM |
| Restraint Type | COMPONENT-ATTRIBUTE5 | PROPOSED | SUPPORT-DIRECTION (native PCF attribute) | DIRECT |
| Restraint Stiffness | COMPONENT-ATTRIBUTE6 | PROPOSED | PCF_RES_MAP.TXT (restraint type stiffness) | NOT_MAPPED → RES_MAP |
| Restraint Friction | COMPONENT-ATTRIBUTE7 | PROPOSED | PCF_RES_MAP.TXT MU= | NOT_MAPPED → RES_MAP MU= |
| Restraint Gap | COMPONENT-ATTRIBUTE8 | PROPOSED | PCF_RES_MAP.TXT GAP= | NOT_MAPPED → RES_MAP GAP= |

Version v2.3 Correction Source: Hexagon PCF Reference Guide v15, CAESAR II PCF Import Configuration (PCF_RES_MAP.TXT / PCF_SIF_MAP.TXT / PCF_MAT_MAP.TXT / PCF_UNITS_MAP.TXT), Section 4 & 5 of PCF Configuration and Mapping Architecture document.

Appendix H:  CAESAR II PCF Import Configuration

This appendix documents the four plain-text mapping files that CAESAR II uses to translate PCF geometry into a stress analysis model. Content is sourced from "PCF Configuration and Mapping Architecture" (Sections 4 & 5). Bold text = confirmed from documentation. Normal weight = inferred/proposed.

H.1  The Four-File Mapping Ecosystem

PCF is a geometric format, not an analytical one. CAESAR II uses four plain-text configuration files in its System directory to translate geometry into analysis. A syntax error in any of these files causes CAESAR II to revert to default ("dumb") behaviour — rigid anchors, default material.

⚠  CRITICAL: Syntax errors in mapping files are SILENT. CAESAR II does not abort — it silently falls back to rigid anchors and default Carbon Steel material. Always review the import log after every PCF import.

| File Name | Function | Failure Consequence (if syntax error or missing entry) |
| --- | --- | --- |
| PCF_RES_MAP.TXT | Restraint Mapping Maps support NAME tags → CAESAR II restraint types | Unmatched support → RIGID ANCHOR (ANC) Overstiff model → high thermal stresses |
| PCF_MAT_MAP.TXT | Material Mapping Maps CAD description strings → C2 Material IDs | Unmatched material → defaults to Material #1 (Low Carbon Steel). Critical for alloy projects. |
| PCF_UNITS_MAP.TXT | Unit Conversion Defines factors to convert PCF units → C2 English units | Missing entry → C2 reads raw values as English units (e.g. 100mm read as 100 inches = 8.3 ft). Catastrophic. |
| PCF_SIF_MAP.TXT | SIF Mapping Maps ISOGEN SKEY codes → C2 intersection types | Unmatched SKEY → default SIF = 1.0 (unconservative for Tees, Olets, Bends). |

H.2  PCF_RES_MAP.TXT — Restraint Mapping

H.2.1  Syntax Definition  (Confirmed — Hexagon Documentation)

The file uses a strict space-delimited columnar format. Each line maps one PCF support NAME tag to one or more CAESAR II restraint functions:

<SupportName>  <NumFunctions>  <Type1> [MU=<val>] [GAP=<val>]  [<Type2> [MU=<val>]] ...

SupportName   — string CAESAR II searches for in PCF SUPPORT > NAME attribute

Matching is SUBSTRING-BASED (partial match is sufficient)

NumFunctions  — integer: how many C2 restraint rows to generate at this node

Type          — C2 Restraint Code: ANC  Y  +Y  -Y  Guide  X  +X  -X  Z  +Z  -Z

MU=           — Friction coefficient (dimensionless, e.g. MU=0.3)

GAP=          — Gap in C2 internal units (inches, e.g. GAP=0.5 = half inch each side)

H.2.2  Worked Examples  (Confirmed — Hexagon Documentation)

Example A: Resting Support with Friction

REST_SLIDE   1   +Y   MU=0.3

↑   ↑     ↑

│   │     └── Friction coefficient = 0.3

│   └── +Y = upward only (allows liftoff)

└── 1 restraint function generated

# CSV Source: Restraint Type = REST, Restraint Friction = 0.3

Example B: Guided Support with Weight Rest  (Complex — 2 Functions)

GUIDE_W_SHOE   2   +Y MU=0.3   GUIDE MU=0.3

↑   ──────────  ────────────

│   Function 1  Function 2

└── 2 restraint functions generated at same node

# Function 1: +Y with friction (weight rest)

# Function 2: GUIDE with friction (lateral restraint)

# CSV Source: Restraint Type = GUIDE_W, Friction = 0.3 (both functions)

Example C: X-Stop with Gap

STOP_X   1   X   GAP=0.5

↑    ↑

│    └── 0.5" gap EACH direction (+X and -X), total play = 1.0"

└── X = double-acting (both +X and -X)

# CSV Source: Restraint Type = STOP, Restraint Gap = 12.7mm → 0.5" after unit conversion

H.2.3  The "Blind Import" Risk — Validator Rule  [CRITICAL]

BLIND IMPORT RULE (Validator Rule H-001): If a support NAME in the PCF does not match ANY entry in PCF_RES_MAP.TXT, CAESAR II silently creates a RIGID ANCHOR (ANC) at that node. This makes the system artificially stiff and produces unrealistically high thermal expansion stresses. The validator MUST cross-check every SUPPORT NAME against PCF_RES_MAP.TXT and flag any unmatched entry as a CRITICAL error.

Detection: Parse the import log file for lines containing "No match found" or "Default restraint applied". Also validate directly by comparing the set of unique NAME values in all SUPPORT blocks against the keys in PCF_RES_MAP.TXT before import.

H.3  PCF_MAT_MAP.TXT — Material Mapping

H.3.1  Syntax  (Confirmed — Hexagon Documentation)

<MaterialID>   <SearchString>

MaterialID    — CAESAR II internal material database ID (integer or short code)

SearchString  — string that CAESAR II searches for in the PCF COMPONENT-ATTRIBUTE3

(Mode 1) or MATERIAL / ITEM-DESCRIPTION field (Mode 2)

# Examples:

106    A106 Gr.B

304    ASTM A312 TP304

316L   ASTM A312 TP316L

DSS    DUPLEX 2205

INC    INCONEL 625

H.3.2  Matching Algorithm  (Confirmed — Hexagon Documentation)

Two-stage matching:

Stage 1 — Exact Match: Scans PCF_MAT_MAP.TXT for the exact string found in the PCF COMPONENT-ATTRIBUTE3 (Mode 1) or MATERIAL attribute (Mode 2).

Stage 2 — Intelligent Fallback: If no exact match, CAESAR II attempts to parse for keywords such as "A106", "304", "Carbon Steel". This works for common grades but FAILS for exotic alloys (Duplex, Inconel, Super Duplex, Titanium).

⚠  ALLOY PROJECTS: Explicit mapping entries are MANDATORY for Duplex, Super Duplex, Inconel, Titanium, and other non-standard alloys. Fallback defaults to Material #1 (Low Carbon Steel) — wrong E-modulus, wrong expansion coefficient → invalid stress results.

H.4  PCF_UNITS_MAP.TXT — Unit Conversion

H.4.1  Syntax and Conversion Logic  (Confirmed — Hexagon Documentation)

CAESAR II converts all incoming PCF data to internal English units before applying display units. The conversion formula is:

Internal_Value = Input_Value / Factor

Syntax:  <C2_InternalUnit>  <PCF_UnitLabel>  <Factor>

| C2 Internal Unit | PCF Unit Label | Factor | Example Calculation | Source Confidence |
| --- | --- | --- | --- | --- |
| IN | MM | 25.4 | 100mm ÷ 25.4 = 3.937 inches | CONFIRMED (Hexagon doc) |
| PSI | BAR | 0.0689476 | 10 bar ÷ 0.0689476 = 145.04 PSI | CONFIRMED (Hexagon doc) |
| F | C | 0.55555555 | 100°C ÷ 0.555 = 180°F scale (32° offset handled automatically) | CONFIRMED (Hexagon doc) |
| IN | IN | 1.0 | Pass-through for inch projects | INFERRED |
| KGS/M3 | KGS/M3 | 1.0 | Fluid density: no conversion needed if source is kg/m³ | PROPOSED |

Note on Temperature: The file requires only the SCALE factor (0.5555 ≈ 1/1.8). CAESAR II automatically handles the 32° offset for the affine transformation °F = (°C × 1.8) + 32. Do not attempt to encode the offset in the factor.

H.5  PCF_SIF_MAP.TXT — SIF Mapping

H.5.1  SKEY Mapping Syntax  (Confirmed — Hexagon Documentation)

<SKEY>   <SIFType>   [PAD=<thickness> <unit>]   [Ii=<val>]   [Io=<val>]

SKEY     — ISOGEN Symbol Key from PCF SKEY attribute (e.g. TEEB, TESO, WOL)

SIFType  — C2 intersection type:

1 = Reinforced Tee

2 = Unreinforced Tee

3 = Weldolet

4 = Sweepolet

PAD=     — Reinforcement pad thickness with unit (e.g. PAD=10 MM)

Ii=      — Explicit in-plane SIF override (overrides calculated value)

Io=      — Explicit out-of-plane SIF override

H.5.2  Standard SKEY Mapping Reference  (Confirmed — Hexagon Documentation)

| SKEY | PCF Component | SIF Type | C2 Intersection Type | Notes |
| --- | --- | --- | --- | --- |
| TEEB | TEE (equal bore butt-weld) | 2 | Unreinforced Tee | Standard butt-weld equal-bore tee. SIF calculated from bore/thickness geometry. |
| TESO | TEE-SET-ON (set-on branch) | 3 | Weldolet | Unequal bore branch. CSV SIF column used for cross-check. |
| TERF | TEE (reinforced) | 1 | Reinforced Tee | PAD= thickness required. Ii=1.0 Io=1.0 forces unity SIF. |
| WOL | Weld-O-Let / OLET | 3 | Weldolet | Same type as TESO. Branch bore ratio determines SIF magnitude. |
| ELBW | BEND (long-radius elbow) | — | Not an intersection — C2 uses Radius column for bend SIF | Bend SIF is computed geometrically by C2. No SIF type entry needed. |

Validator Cross-Check (Rule H-002): For each TEE, TEE-SET-ON, and OLET block in the PCF, verify the SKEY has a corresponding entry in PCF_SIF_MAP.TXT. An unmatched SKEY results in SIF = 1.0 (unconservative). Flag as WARNING.

H.6  COMPONENT-ATTRIBUTE Mapping — Mode 1 vs Mode 2

Two modes exist for mapping Wall Thickness and Material from CSV into PCF. The correct mode depends on whether the CAESAR II project uses native PCF attributes or the COMPONENT-ATTRIBUTE injection mechanism. All other CA slots (CA1, CA2, CA5–CA7, CA9, CA10) are unambiguous and apply identically in both modes.

MODE 1:  COMPONENT-ATTRIBUTE (CA3 / CA4)  —  CAESAR II Standard Import

Use when: PCF is imported directly into CAESAR II via the standard "Stress ISO" PCF import. CAESAR II reads CA3 for material lookup and CA4 for wall thickness override.

# Mode 1 — PCF output (component block example):

PIPE

END-POINT  96400.00  17989.40  101968.00  400

END-POINT  96400.00  17840.40  101968.00  400

COMPONENT-ATTRIBUTE1  12.07       ← Pressure (bar)         → C2 P1

COMPONENT-ATTRIBUTE2  120         ← Temperature (°C)       → C2 T1

COMPONENT-ATTRIBUTE3  A106 Gr.B   ← Material string        → PCF_MAT_MAP lookup

COMPONENT-ATTRIBUTE4  12.7        ← Wall thickness (mm)    → C2 wall thickness override

COMPONENT-ATTRIBUTE5  50          ← Insulation thick (mm)  → C2 insulation thickness

COMPONENT-ATTRIBUTE6  130         ← Insulation density (kg/m³) → C2 insulation density

COMPONENT-ATTRIBUTE7  3.0         ← Corrosion allowance (mm) → C2 corrosion

COMPONENT-ATTRIBUTE9  850         ← Fluid density (kg/m³)  → C2 fluid density

COMPONENT-ATTRIBUTE10 18.5        ← Hydrotest pressure (bar) → C2 hydrotest P

FABRICATION-ITEM

Confidence: CA3 and CA4 confirmed from Hexagon documentation. All other slots confirmed from "PCF Configuration and Mapping Architecture" Section 5.

MODE 2:  Native PCF Attributes (MATERIAL / WALL-THICKNESS)  —  PCF-Native

Use when: PCF is consumed by non-CAESAR-II tools (e.g. ROHR2, AutoPIPE, or PCF viewers) that read native PCF attributes. MATERIAL and WALL-THICKNESS are standard PCF attributes recognised by multiple applications.

# Mode 2 — PCF output (same component, native attributes):

PIPE

END-POINT  96400.00  17989.40  101968.00  400

END-POINT  96400.00  17840.40  101968.00  400

MATERIAL  ASTM A106 GR B         ← Native PCF attribute (col 5)

WALL-THICKNESS  12.7             ← Native PCF attribute (col 5)

COMPONENT-ATTRIBUTE1  12.07      ← Pressure — same in both modes

COMPONENT-ATTRIBUTE2  120        ← Temperature — same in both modes

FABRICATION-ITEM

Confidence: MATERIAL and WALL-THICKNESS attribute names seen in PCF samples from this project. Syntax (col 5, value as 4th token) is consistent with all other PCF attributes. However, HEXAGON GUIDE SYNTAX NOT YET CONFIRMED — treat as LOW confidence until verified against Hexagon PCF Reference Guide v15.

H.6.1  Mode Comparison Summary

| CSV Column | CA Slot | Mode 1 PCF Attribute (CAESAR II Standard) | Mode 1 Confidence | Mode 2 PCF Attribute (Native PCF) | Mode 2 Confidence |
| --- | --- | --- | --- | --- | --- |
| Material | CA3 | COMPONENT-ATTRIBUTE3 <value> | HIGH (Hexagon doc confirmed) | MATERIAL <value> | LOW (PCF samples only, Hexagon guide not confirmed) |
| Wall Thickness | CA4 | COMPONENT-ATTRIBUTE4 <value> | HIGH (Hexagon doc confirmed) | WALL-THICKNESS <value> | LOW (PCF samples only, Hexagon guide not confirmed) |
| Pressure (all others) | CA1 (+others) | COMPONENT-ATTRIBUTE1 <value> (same in both modes) | HIGH | COMPONENT-ATTRIBUTE1 <value> (same in both modes) | HIGH |

Recommendation: Use MODE 1 (COMPONENT-ATTRIBUTE) as the primary output mode for CAESAR II projects. Add MODE 2 native attributes as supplementary lines in the same component block — this satisfies both CAESAR II and any PCF viewer simultaneously (duplication is harmless as CAESAR II prioritises CA slots).

H.7  Complete COMPONENT-ATTRIBUTE Reference  CA1–CA10  (CAESAR II Standard)

The following table applies to BOTH modes. CA1–CA10 are the standard CAESAR II "Stress ISO" import slots. Bold = confirmed from Hexagon documentation.

| CA Slot | CAESAR II Field | CSV Source Column | Units (PCF) | Condition to Write | Confidence |
| --- | --- | --- | --- | --- | --- |
| CA1 | Pressure 1 (P1) | Pressure | bar | Always if non-zero | HIGH |
| CA2 | Temperature 1 (T1) | DesignTemperature_degC (Proposed new column) | °C | Always if non-zero | HIGH |
| CA3 | Material (PCF_MAT_MAP key) | Material | text string | Write when non-empty (not "0") | HIGH |
| CA4 | Wall Thickness override | Wall Thickness | mm | Write ONLY if > 0 (0 = use spec default) | HIGH |
| CA5 | Insulation Thickness | InsulationThickness_mm (Proposed new column) | mm | Write if non-zero | HIGH |
| CA6 | Insulation Density | InsulationDensity_kgm3 (Proposed new column) | kg/m³ | Write if CA5 > 0 | HIGH |
| CA7 | Corrosion Allowance | Corrosion Allowance | mm | Write if non-zero | HIGH |
| CA8 | (not defined in C2 standard) | — | — | Do not use — skip CA8 | N/A |
| CA9 | Fluid Density | FluidDensity_kgm3 (Proposed new column) | kg/m³ | Write if non-zero (critical for hydrotest weight) | HIGH |
| CA10 | Hydrotest Pressure | HydrotestPressure_bar (Proposed new column) | bar | Write if non-zero (generates separate hydrotest case) | HIGH |

H.8  Advanced Custom Mapping  CA11–CA100  (Confirmed — Hexagon Documentation)

Advanced users can map CA11–CA100 via PCF_MAP.XML or the CAESAR II interface. This enables multi-case analysis and specialised pipe modelling to be driven entirely from CAD model data:

| CA Slot | Proposed CAESAR II Field | Application | Confidence |
| --- | --- | --- | --- |
| CA20 | Temperature 2 (T2 — Operating) | Multi-case: CA20 drives T2 load case automatically from CAD model | CONFIRMED (Hexagon doc) |
| CA21 | Temperature 3 (T3 — Upset) | Multi-case: CA21 drives T3 upset/emergency temperature case | CONFIRMED (Hexagon doc) |
| CA30+ | Refractory / Tracing Density | Jacketed pipe modelling — trace density for steam-jacketed lines | CONFIRMED (Hexagon doc) |
| CA11–CA19 CA22–CA29 | User-defined / project-specific | Available for project custom fields. Map via PCF_MAP.XML. | INFERRED |

Appendix H sources: PCF Configuration and Mapping Architecture (Sections 4 & 5), Hexagon CAESAR II PCF Import Reference. Bold text = confirmed from documentation. Normal = inferred/proposed. Mode 2 (MATERIAL / WALL-THICKNESS) requires verification against Hexagon PCF Reference Guide v15.

Appendix H:  CAESAR II PCF Import Configuration  (v2.5 — Complete Rewrite)

This appendix supersedes Appendix H from v2.4. All content has been verified against Hexagon CAESAR II documentation and confirmed against working PCF samples NG-151841-102-03-TEST.pcf and NG-151843-102-02-TEST.pcf. Bold = confirmed from Hexagon documentation or PCF samples. Normal = inferred/proposed.

H.1  The Five-File Mapping Ecosystem

PCF is a geometric format. CAESAR II uses five configuration files stored in C:\ProgramData\Intergraph CAS\CAESAR II\<version>\System to translate PCF geometry into a stress analysis model. Syntax errors cause silent fallback to defaults — no abort, no warning dialog.

⚠  CRITICAL: All failures are SILENT. CAESAR II does not abort on mapping errors — it falls back to rigid anchors, default Carbon Steel, and SIF=1.0. Always review the import log after every PCF import.

| File | Type | Confirmed Purpose | Failure Consequence |
| --- | --- | --- | --- |
| PCF_MAP.xml | XML | PCF Property Mapping Template. Dictionary: PCF keyword ↔ CAESAR II attribute. Configurable via GUI (Utilities → PCF Mapping). | CA slots misread → wrong P/T/Material assigned to wrong fields. |
| PCF_MAT_MAP.TXT | TXT | Material Mapping. Maps CA3 string values → CAESAR II material numbers. Multiple alias rows per material ID. | Unmatched string → Material #1 (Low Carbon Steel). Wrong E-modulus, wrong expansion coefficient. |
| PCF_RES_MAP.TXT | TXT | Restraint Mapping. Two-section file: KEYWORD MAPPING + SUPPORT MAPPING. Matches on ITEM-DESCRIPTION of SUPPORT blocks. | Unmatched ITEM-DESCRIPTION → RIGID ANCHOR (ANC). Overstiff model → high thermal stresses. |
| PCF_SIF_MAP.TXT | TXT | SIF Mapping. Maps ISOGEN SKEY codes → CAESAR II intersection types (1-17). Standard BW tees (TEBW) not mapped — C2 computes geometrically. | Unmatched SKEY → SIF=1.0. Unconservative for fabricated tees, olets. |
| PCF_Units_Map.txt | TXT | Unit Mapping. Conversion factors: C2 internal (English) ↔ PCF units. Formula: C2_value × factor = PCF_value. | Missing entry → raw value read as English units. (e.g. 1000 KPA read as 1000 PSI). Catastrophic. |

H.2  PCF_MAP.xml — Property Mapping Template

Acts as a dictionary matching PCF keyword names to CAESAR II Classic Piping Input fields. Configurable via GUI: Utilities → PCF Mapping. Saved as PCF_MAP.xml in System folder. This is the PRIMARY file controlling CA slot assignments — the TXT files handle material/restraint/SIF/unit lookup WITHIN those slots.

H.2.1  Confirmed Project Configuration  (from PCF Mapping GUI)

The following is the confirmed mapping for this project. Only populated rows shown — blank rows (Pressure 2–9, Temperature 2–9, Cladding, Refractory) are not used.

| CAESAR II Attribute | PCF Keyword (CA Slot) | Sample Value | Units (PCF) | Write Condition |
| --- | --- | --- | --- | --- |
| Pressure 1 | COMPONENT-ATTRIBUTE1 | XXXX KPA | KPA | Always. Use XXXX if unknown. |
| Temperature 1 | COMPONENT-ATTRIBUTE2 | xxxx C | C | Always. Use xxxx if unknown. |
| Material | COMPONENT-ATTRIBUTE3 | A106-B | text | Always. Use Undefined if unknown. |
| Wall Thickness | COMPONENT-ATTRIBUTE4 | Undefined MM or 12.7 MM | MM | Always. Never omit. Undefined MM = spec-driven. |
| Insulation Thickness | COMPONENT-ATTRIBUTE5 | 0 MM | MM | Always. 0 MM = uninsulated. |
| Insulation Density | COMPONENT-ATTRIBUTE6 | (not in samples) | KG/M3 | Write when insulation present (CA5 > 0). |
| Corrosion Allowance | COMPONENT-ATTRIBUTE7 | (not in samples) | MM | Write when non-zero. |
| Weight (Pipe Density) | COMPONENT-ATTRIBUTE8 | (not in samples) | KG/M3 | Write when non-zero. CA8 = Weight confirmed from GUI. |
| Fluid Density | COMPONENT-ATTRIBUTE9 | (not in samples) | KG/M3 | Write when non-zero. Critical for hydrotest weight. |
| Hydro Test Pressure | COMPONENT-ATTRIBUTE10 | xxxx KPA | KPA | Always. Use xxxx if unknown. |
| Line Number | PIPELINE-REFERENCE | TMP-2 | text | Direct — no CA slot. Passes without mapping. |

H.2.2  Value Population — Mode 1 vs Mode 2

The CA slot assignments are identical in both modes. The distinction is whether design values are populated at PCF export time or deferred as placeholders.

MODE 1:  Design Values Known at Export Time

Use when P&T are confirmed from process datasheets before PCF export.

PIPE

COMPONENT-IDENTIFIER  1

END-POINT  14073.67  11597.3  528.6  152.4

END-POINT  14073.67  11597.3  2588.85  152.4

ITEM-DESCRIPTION  6" STD Pipe BE ASME B36.10M smls ASTM A106 GR B

PIPING-SPEC  1C1P

COMPONENT-ATTRIBUTE1  1200 KPA       ← actual design pressure

COMPONENT-ATTRIBUTE2  120 C          ← actual design temperature

COMPONENT-ATTRIBUTE3  A106-B         ← material code for MAT_MAP lookup

COMPONENT-ATTRIBUTE4  7.11 MM        ← actual wall thickness

COMPONENT-ATTRIBUTE5  50 MM          ← actual insulation thickness

COMPONENT-ATTRIBUTE10  1800 KPA      ← actual hydrotest pressure

FABRICATION-ITEM

MODE 2:  Placeholder Values  (pending stress analysis input)

Use when PCF is exported before process conditions are finalised. Matches exactly what is seen in project samples NG-151841 and NG-151843.

PIPE

COMPONENT-IDENTIFIER  1

END-POINT  14073.67  11597.3  528.6  152.4

END-POINT  14073.67  11597.3  2588.85  152.4

ITEM-DESCRIPTION  6" STD Pipe BE ASME B36.10M smls ASTM A106 GR B

PIPING-SPEC  1C1P

COMPONENT-ATTRIBUTE1  XXXX KPA       ← pressure TBD

COMPONENT-ATTRIBUTE2  xxxx C         ← temperature TBD

COMPONENT-ATTRIBUTE3  A106-B         ← material always known

COMPONENT-ATTRIBUTE4  Undefined MM   ← spec-driven / TBD

COMPONENT-ATTRIBUTE5  0 MM           ← no insulation (or TBD)

COMPONENT-ATTRIBUTE10  xxxx KPA      ← hydrotest TBD

FABRICATION-ITEM

H.3  PCF_MAT_MAP.TXT — Material Mapping

H.3.1  Confirmed Syntax  (Hexagon documentation + samples)

Format: MaterialID first, search string second. Multiple alias rows per ID. First line is a year/version comment (informational only).

2019                          ← year/version comment (informational)

<MaterialID>  <SearchString>  ← one alias per line

* Example: Material 106 = ASTM A106 Grade B

* All of the following map to the same C2 material number:

106  A106-B

106  A-106 B

106  ASTM A106 grade B

106  ASME SA106

106  ASTM A106-B

106  ASTM A106 Gr.B

106  ALABAMA-Mat-101

* Project sample CA3 value "A106-B" matches first alias above

H.3.2  Matching Algorithm  (Confirmed — Hexagon documentation)

CAESAR II scans every row of PCF_MAT_MAP.TXT and performs substring matching against the CA3 value. If no match found → defaults to Material #1 (Low Carbon Steel). No intelligent fallback — all aliases must be explicitly listed.

⚠  Alloy projects: Explicit entries MANDATORY for Duplex, Super Duplex, Inconel, Titanium. Fallback to Material #1 gives wrong E-modulus and thermal expansion coefficient → invalid stress results.

H.4  PCF_RES_MAP.TXT — Restraint Mapping

H.4.1  Two-Section File Structure  (Confirmed — Hexagon documentation)

The file has two distinct sections:

* ======================================================

* SECTION 1: KEYWORD MAPPING SECTION

* Defines which PCF attribute is used as the lookup key

* ======================================================

<SUPPORT_NAME>   ITEM-DESCRIPTION   ← C2 searches ITEM-DESCRIPTION for match

<SUPPORT_TAG>    SUPPORT-TYPE       ← secondary tag attribute

<SUPPORT_GUID>   UCI                ← unique component identifier

* ======================================================

* SECTION 2: SUPPORT MAPPING SECTION

* Maps each support ITEM-DESCRIPTION → C2 restraint functions

* ======================================================

<SupportName>  <NumFunctions>

<RestraintCode>  [MU=<val>]  [GAP=<val>]

...

H.4.2  Lookup Key — ITEM-DESCRIPTION  (Confirmed)

CAESAR II matches on the ITEM-DESCRIPTION attribute of the SUPPORT block in PCF. This is configured in Section 1 via <SUPPORT_NAME> → ITEM-DESCRIPTION. NOT matched on SKEY, NAME, or any other attribute.

Validator Rule H-001 (BLIND IMPORT RISK): Every unique ITEM-DESCRIPTION value in SUPPORT blocks must have a matching entry in PCF_RES_MAP.TXT Section 2. Unmatched → silent RIGID ANCHOR. Cross-check before every import.

H.4.3  Valid Restraint Codes  (Confirmed — Hexagon documentation)

| Code | Type | Description | Notes |
| --- | --- | --- | --- |
| ANC | Anchor | Full 6-DOF rigid restraint | Default fallback for unmatched supports. Avoid. |
| VERT | Vertical | Global vertical direction restraint | Equivalent to Y in global coords |
| NS | North-South | Global horizontal North-South |  |
| EW | East-West | Global horizontal East-West |  |
| A B C | Local axes | Local pipe axis directions | A = axial, B/C = lateral. Double-acting. |
| +A -A +B -B +C -C | Local axes (single-acting) | Single-direction local restraints | + = positive direction only. Gap/friction applicable. |
| GUI | Guide | Lateral guide restraint | Allows axial movement, restrains lateral. |
| LIM | Limit stop | Displacement-limited restraint | Used with GAP= |
| VHGR | Variable hanger | Variable spring hanger |  |
| CHGR | Constant hanger | Constant effort spring hanger |  |

H.4.4  Worked Examples  (Confirmed — Hexagon documentation)

* Example 1: Resting support with friction

S1 - PIPE SHOE WELDED RESTING  1

VERT  MU=0.3

* Example 2: Guide on pipe shoe (lateral + vertical)

SG1 - GUIDE ON PIPE SHOE  2

VERT  MU=0.3

GUI  MU=0.3

* Example 3: Limit stop with gap

STOP_X  1

EW  GAP=12.7

* Example 4: Complex 3-function support

TestA01_Name_B0047  3

A    MU=0.04    GAP=0.04

B    MU=0.05    GAP=0.05

C    MU=0.06    GAP=0.06

H.4.5  SUPPORT-DIRECTION Syntax  (Confirmed — PCF samples)

Standard orthogonal directions (all confirmed from project samples):

| PCF Syntax | Meaning | Confirmed From |
| --- | --- | --- |
| SUPPORT-DIRECTION UP | Support acts upward (resting on structure below) | NG-151841 sample |
| SUPPORT-DIRECTION DOWN | Support acts downward (hanging from structure above) | Hexagon documentation |
| SUPPORT-DIRECTION NORTH | Lateral restraint in North direction | NG-151841 sample |
| SUPPORT-DIRECTION SOUTH | Lateral restraint in South direction | Hexagon documentation |
| SUPPORT-DIRECTION EAST | Lateral restraint in East direction | Hexagon documentation |
| SUPPORT-DIRECTION WEST | Lateral restraint in West direction | Hexagon documentation |
| SUPPORT-DIRECTION S 89.4271 U | Extended syntax for sloped lines. South component at slope angle, Up component. EDGE CASE — do not plan for in converter. | NG-151843 sample (sloped pipeline only) |

H.5  PCF_SIF_MAP.TXT — SIF Mapping

H.5.1  Confirmed Syntax  (Hexagon documentation)

* FORMAT: <SKEY> <SIF_TYPE> [PAD=X.X UNITS] [Ii=X.XX] [Io=X.XX]

* Lines starting with * are comments

* PAD thickness, Ii (in-plane), Io (out-plane) are OPTIONAL

* Example:

TESO  1  PAD=10 MM  Ii=1.23  Io=2.56

H.5.2  Confirmed SIF Type Numbers  (Hexagon documentation)

| Type # | CAESAR II Intersection Type |
| --- | --- |
| 1 | Reinforced Fabricated Tee |
| 2 | Unreinforced Fabricated Tee |
| 3 | Welding Tee |
| 4 | Sweepolet |
| 5 | Weldolet |
| 6 | Extruded Welding Tee |
| 7 | Girth Butt Weld |
| 8 | Socket Weld (No Undercut) |
| 9 | Socket Weld (As Welded) |
| 10 | Tapered Transition |
| 11 | Threaded Joint |
| 12 | Double Welded Slip-On |
| 13 | Lap Joint Flange (B16.9) |
| 14 | Bonney Forge Sweepolet |
| 15 | Bonney Forge Latrolet |
| 16 | Bonney Forge Insert Weldolet |
| 17 | Full Encirclement Tee |

H.5.3  Key SKEY Entries  (Confirmed — Hexagon documentation + PCF samples)

| SKEY | Component | SIF Type | Notes |
| --- | --- | --- | --- |
| TESO | Set-On Tee (fabricated) | 1 | Reinforced Fabricated. PAD= optional. |
| TEBW | Tee Equal Butt Weld | NOT MAPPED | C2 computes SIF geometrically from bore/thickness. No entry needed. |
| BTFL | Butt-weld Tee Flanged | 2 | Unreinforced Fabricated Tee. |
| CEBW CESC CESW | Olet types (butt-weld, S/C) | 5 | Weldolet type. |
| HCSC HCSW | Half-coupling olets | 5 | Weldolet type. |
| BEBW | Bend Elbow Butt Weld | NOT MAPPED | Confirmed from samples. Bend SIF computed geometrically by C2. |

H.6  PCF_Units_Map.txt — Unit Mapping

H.6.1  Confirmed Syntax and Formula  (Hexagon documentation)

File header: "Intergraph CAS  PCF / CAESAR II Units Mapping Designations"

* Formula: (C2 internal unit) × constant = PCF unit

* Therefore to convert PCF → C2:  C2_value = PCF_value / constant

* CAESAR II   PCF Unit      Factor      Category

PSI         [PSI]         1.0         *PRESSURE

PSI         [BARg]        0.068946    *PRESSURE

PSI         [BAR]         0.68947     *PRESSURE

PSI         [KPAg]        6.89475     *PRESSURE

PSI         [KPa]         6.89475     *PRESSURE  ← project uses KPA

PSI         [MPa]         0.0068947   *PRESSURE

PSI         [N/MM^2]      0.0068947   *PRESSURE

F           [C]           0.55560     *TEMPERATURE

IN          [MM]          25.4        *THICKNESS

IN          [N]           4.4480      *WEIGHT

LB          [KG]          0.453592    *WEIGHT

LB./CU.IN.  KG./CU.M.    27680.0     *DENSITY

LB./CU.IN.  KG./CM^3     27680.0     *DENSITY

LB./CU.IN.  KG./DM^3     27.68       *DENSITY

IN          [CM]          2.54        *THICKNESS

H.6.2  CAESAR II Internal Units  (Confirmed)

| Quantity | C2 Internal Unit | PCF Project Unit | Factor |
| --- | --- | --- | --- |
| PRESSURE | PSI | KPA | 6.89475 |
| TEMPERATURE | F (Fahrenheit) | C (Celsius) | 0.55560 |
| THICKNESS / LENGTH | IN (inches) | MM | 25.4 |
| WEIGHT | LB | KG | 0.453592 |
| DENSITY | LB./CU.IN. | KG/M3 or KG/CM^3 | 27680.0 or 27.68 |

Note on Temperature: The factor (0.55560 ≈ 1/1.8) is the SCALE only. CAESAR II automatically handles the 32° offset. Do not encode the offset in the factor.

H.7  PCF File Header — Confirmed Structure  (PCF Samples)

Both project samples confirm this exact header structure. UNITS-WEIGHT KGS was missing from v2.4 documentation.

ISOGEN-FILES  ISOGEN.FLS      ← mandatory first line, col 1

UNITS-BORE  MM                ← bore units

UNITS-CO-ORDS  MM             ← coordinate units

UNITS-WEIGHT  KGS             ← weight units (missing from v2.4)

UNITS-BOLT-DIA  MM            ← bolt diameter units

UNITS-BOLT-LENGTH  MM         ← bolt length units

PIPELINE-REFERENCE  TMP-2     ← line number / pipeline ID

PROJECT-IDENTIFIER            ← project tag (blank in samples)

AREA                          ← plant area (blank in samples)

ATTRIBUTE5                    ← pipeline-level slot (see Appendix I)

ATTRIBUTE7                    ← pipeline-level slot (see Appendix I)

ATTRIBUTE10                   ← pipeline-level slot (see Appendix I)

ATTRIBUTE11                   ← pipeline-level slot (see Appendix I)

Appendix H v2.5 sources: Hexagon CAESAR II PCF Import documentation (slides 41–53), PCF samples NG-151841-102-03-TEST.pcf and NG-151843-102-02-TEST.pcf, PCF Mapping GUI screenshot (confirmed project configuration). Bold = confirmed. Normal = inferred.

Appendix I:  Known Gaps & Future Improvements

This appendix captures items that are partially understood or require further project-specific information before they can be fully specified. Items here should NOT be implemented in code until resolved.

I.1  Pipeline-Level Header Attributes  (ATTRIBUTE5/7/10/11)

I.1.1  What is Known

Both project PCF samples contain these four lines in the file header, after PIPELINE-REFERENCE and before the first component block:

ATTRIBUTE5

ATTRIBUTE7

ATTRIBUTE10

ATTRIBUTE11

Observations from samples:

1. All four appear with NO value — keyword only, blank value.

2. They are pipeline-level (file header), NOT component-level attributes.

3. They use ATTRIBUTE (pipeline-level) not COMPONENT-ATTRIBUTE (component-level) — these are different namespaces in PCF.

4. Numbering is non-consecutive (5, 7, 10, 11) — suggesting a project-specific selection from a larger available set.

5. PCF_MAP.xml GUI only shows COMPONENT-ATTRIBUTE slots — pipeline ATTRIBUTEs are configured separately.

I.1.2  What is Unknown — Gap List

| Gap | Question | Impact if Unresolved | How to Resolve |
| --- | --- | --- | --- |
| G-I-01 | What do ATTRIBUTE5/7/10/11 map to in CAESAR II or ISOGEN? | Cannot populate values — must leave blank as in samples. | Check PCF_MAP.xml for pipeline ATTRIBUTE mapping section, or ask project piping lead. |
| G-I-02 | Are values expected to be blank, or should they carry data? | If values required: ISOGEN isometric drawing may be missing data. | Compare isometric output from working PCF vs blank PCF to see if these fields appear on drawing. |
| G-I-03 | Why non-consecutive numbering (5,7,10,11)? What happened to ATTRIBUTE1-4,6,8,9? | May indicate other ATTRIBUTEs are in use elsewhere in the project. | Review full project ISOGEN configuration (FLS file) for complete ATTRIBUTE usage. |
| G-I-04 | Are these ISOGEN drawing attributes (for isometric annotation) or CAESAR II analysis attributes? | If ISOGEN-only: no impact on stress analysis. If C2: may carry P/T/fluid data at pipeline level. | Check ISOGEN .FLS configuration file and compare with CAESAR II import results. |

I.1.3  Interim Implementation Rule

Until resolved: Write ATTRIBUTE5, ATTRIBUTE7, ATTRIBUTE10, ATTRIBUTE11 as blank keyword lines in the PCF header — exactly as seen in project samples. Do not attempt to populate values. This matches working PCF behaviour.

I.2  PCF_RES_MAP.TXT — Project-Specific Support Name Mapping

I.2.1  What is Known

From project samples, the following unique ITEM-DESCRIPTION values exist on SUPPORT blocks:

WP   - WEAR PAD

S1   - PIPE SHOE WELDED RESTING

SG1  - GUIDE ON PIPE SHOE

BSG1 - GUIDE FOR BASE SUPPORT

DT1  - DOUBLE TRUNNION WITHOUT PAD

S3   - PIPE SHOE FOR SLOPING LINES - RESTING

BS1  - TRUNION BASE SUPPORT

I.2.2  What is Unknown — Gap List

| Gap | Question | Impact | How to Resolve |
| --- | --- | --- | --- |
| G-I-05 | What C2 restraint codes (VERT, GUI, ANC etc.) and MU/GAP values are assigned to each ITEM-DESCRIPTION? | Without this, cannot build PCF_RES_MAP.TXT. All supports default to ANC (blind import risk). | Obtain project PCF_RES_MAP.TXT from stress engineer, or define support behaviour from support standard drawings. |
| G-I-06 | Is matching exact or substring? e.g. does "S1" match "BS1"? | Wrong match could assign incorrect restraint type. | Test with CAESAR II import log. Use most specific string possible. |

I.3  PCF Unit Label — Bracket Convention

I.3.1  What is Unknown

PCF_Units_Map.txt shows units with brackets: [KPa], [KPAg], [BAR]. Project samples use KPA without brackets: COMPONENT-ATTRIBUTE1  XXXX KPA. It is unconfirmed whether brackets are mandatory, optional, or irrelevant for the unit suffix written in the CA value string.

| Gap | Question | Impact | How to Resolve |
| --- | --- | --- | --- |
| G-I-07 | Are brackets required in PCF unit labels? [KPA] vs KPA — does CAESAR II parse both? | If brackets required and omitted → unit not recognised → value treated as dimensionless. | Samples work without brackets. Accept both. Document project convention as KPA (no brackets). |

Interim rule: Use project sample convention — KPA, C, MM without brackets. Samples confirmed working in CAESAR II.

I.4  Future Enhancement Candidates

The following items are out of scope for the current converter but should be tracked for future versions:

| ID | Enhancement | Priority | Dependency |
| --- | --- | --- | --- |
| FE-01 | Multi-case support: populate CA20=T2, CA21=T3 for upset/emergency temperature cases | Medium | Process datasheet with multiple temperature cases |
| FE-02 | Jacketed pipe: populate CA for refractory density and tracing density | Low | Project requirement for jacketed lines |
| FE-03 | SUPPORT-DIRECTION extended slope syntax: S <angle> U for non-orthogonal supports | Low | Only for sloped pipeline projects. Edge case. |
| FE-04 | Automatic PCF_RES_MAP.TXT generation from support standard library | High | Support standard drawings + stress engineer input (Gap G-I-05) |
| FE-05 | Resolve ATTRIBUTE5/7/10/11 pipeline header mapping | Medium | Project ISOGEN FLS file + C2 import log analysis (Gaps G-I-01 to G-I-04) |

Appendix I — Living document. Update as project information becomes available. Gaps G-I-01 through G-I-07 require input from stress engineer and project ISOGEN configuration.

Appendix J:  PCF Component Mapping Reference

This appendix defines the complete PCF syntax for each component type, showing CSV source columns, default values, units, and remarks. Based on PCF_Mapping.csv, confirmed PCF samples, and Hexagon ISOGEN documentation. Bold = confirmed. Normal = inferred/proposed.

J.1  COMPONENT-IDENTIFIER Reference  (ISOGEN IDF Record Numbers)

COMPONENT-IDENTIFIER links each PCF block to the ISOGEN symbol library. The number is the IDF (Intermediate Data Format) record type — it selects the correct isometric drawing symbol. Not a sequential counter.

| PCF Component Keyword | CSV Type Code | IDF Record # | ISOGEN PCF Identification | Confirmed From |
| --- | --- | --- | --- | --- |
| PIPE | BRAN | 1 | PIPE | PCF_Mapping.csv |
| TEE | TEE | 7 | TEE | PCF_Mapping.csv |
| BEND | ELBO | 13 | BEND / ELBOW | PCF_Mapping.csv |
| OLET | OLET | 2 | OLET | Hexagon PCF Reference Guide |
| FLANGE | FLAN | 105 | FLANGE | ISOGEN SKEY Guide v2016 |
| FLANGE-BLIND | FLAN | 107 | FLANGE-BLIND | ISOGEN SKEY Guide v2016 |
| VALVE | VALV | 70–75* | VALVE | * Varies by valve type/SKEY. Verify from ISOGEN SKEY Guide for project valve type. |
| SUPPORT | ANCI | 82* | SUPPORT | * Inferred. Verify from ISOGEN SKEY Guide. |

J.2  SKEY End-Type Suffix Reference  (Confirmed — ISOGEN SKEY Guide v2016)

All SKEYs are 2–4 characters. First 2 characters = component type. Last 2 characters = end condition. Replace ** with suffix from table below.

| Suffix | End Condition | Common Use |
| --- | --- | --- |
| BW | Butt Weld | Process piping standard end — BEBW, TEBW, VGBW |
| FL | Flanged | Flanged valves, flanged bends — VBFL, BEFL, FLWN |
| SW | Socket Weld | Small bore socket weld fittings — BESW, TESW |
| SC | Screwed | Screwed fittings, utility piping |
| PL | Plain End | Plain end pipe |
| CP | Compression | Instrumentation tubing |
| CL | Clamped | Hygienic/sanitary piping |
| GL | Glued | GRP/FRP piping |

J.3  SKEY Master List  (by Component Family)

| Family | SKEY | Description | SIF Map Entry? | Confirmed From |
| --- | --- | --- | --- | --- |
| PIPE | — | No SKEY for straight pipe | No | PCF standard |
| BEND | BEBW | Butt Weld elbow/bend (project standard) | No (geometric) | PCF samples confirmed |
| BEND | BEFL | Flanged elbow | No | ISOGEN SKEY Guide |
| BEND | BESW | Socket Weld elbow | No | ISOGEN SKEY Guide |
| TEE | TEBW | Equal bore butt weld tee (project standard) | No (geometric) | PCF samples confirmed |
| TEE | TESO | Set-on / fabricated tee (branch stub-in) | Yes Type 1 | PCF samples + SIF map confirmed |
| TEE-SET-ON | TESO | PCF keyword for set-on branch | Yes Type 1 | PCF reference |
| TEE | TEFL | Flanged tee | No | ISOGEN SKEY Guide |
| TEE | TESW | Socket weld tee | No | ISOGEN SKEY Guide |
| FLANGE | FLWN | Weld neck flange (most common process standard) | No | ISOGEN SKEY Guide |
| FLANGE | FLSO | Slip-on flange | No | ISOGEN SKEY Guide |
| FLANGE | FLSW | Socket weld flange | No | ISOGEN SKEY Guide |
| FLANGE | FLSC | Screwed flange | No | ISOGEN SKEY Guide |
| FLANGE-BLIND | FLBL | Blind flange (IDF 107) | No | ISOGEN SKEY Guide |
| VALVE | VBFL | Ball valve, flanged ends (project standard) | No | User confirmed |
| VALVE | VBBW | Ball valve, butt weld ends | No | ISOGEN SKEY Guide |
| VALVE | VGFL | Gate valve, flanged | No | ISOGEN SKEY Guide |
| VALVE | VGBW | Gate valve, butt weld | No | ISOGEN SKEY Guide |
| VALVE | VCFL | Check valve, flanged | No | ISOGEN SKEY Guide |
| VALVE | VCBW | Check valve, butt weld | No | ISOGEN SKEY Guide |
| VALVE | VBFL | Butterfly valve, flanged | No | ISOGEN SKEY Guide |
| OLET | CEBW | Weldolet / olet butt weld | Yes Type 5 | PCF SIF map confirmed |
| OLET | CESC | Sockolet | Yes Type 5 | PCF SIF map confirmed |
| OLET | CESW | Olet socket weld | Yes Type 5 | PCF SIF map confirmed |
| OLET | HCSC | Half-coupling / threadolet screwed | Yes Type 5 | PCF SIF map confirmed |
| SUPPORT | SKID | Generic support (project standard SKEY) | N/A | PCF samples confirmed |

J.4  PIPE  (CSV Type: BRAN  |  ISOGEN IDF: 1)

Straight pipe segment. Two END-POINTs. No SKEY. FABRICATION-ITEM marks as shop fabrication. CA1–CA10 define all stress analysis inputs.

PIPE

COMPONENT-IDENTIFIER  1

END-POINT  <East>  <North>  <Up>  <Bore>         ← Point=1 from CSV

END-POINT  <East>  <North>  <Up>  <Bore>         ← Point=2 from CSV

ITEM-DESCRIPTION  6" STD Pipe BE ASME B36.10M smls ASTM A106 GR B

PIPING-SPEC  1C1P

COMPONENT-ATTRIBUTE1  <Pressure> [KPA]           ← CA1: Pressure 1

COMPONENT-ATTRIBUTE2  <Temperature> [C]          ← CA2: Temperature 1

COMPONENT-ATTRIBUTE3  <Material>                 ← CA3: MAT_MAP key

COMPONENT-ATTRIBUTE4  <WallThickness> [MM]       ← CA4: Wall thickness

COMPONENT-ATTRIBUTE5  <InsulThickness> [MM]      ← CA5: Insulation thickness

COMPONENT-ATTRIBUTE6  <InsulDensity> [KG/M3]     ← CA6: Insulation density

COMPONENT-ATTRIBUTE7  <CorrAllow> [MM]           ← CA7: Corrosion allowance

COMPONENT-ATTRIBUTE8  <Weight> [KGS]             ← CA8: Pipe weight/density

COMPONENT-ATTRIBUTE9  <FluidDensity> [KG/M3]     ← CA9: Fluid density

COMPONENT-ATTRIBUTE10  <HydroPress> [KPA]        ← CA10: Hydrotest pressure

FABRICATION-ITEM

| PCF Attribute | CSV Source | Default Value | Unit (PCF) | Remarks |
| --- | --- | --- | --- | --- |
| PIPE | — | — | — | Component type keyword. Fixed. |
| COMPONENT-IDENTIFIER | — | 1 | — | Fixed IDF record = 1 for all PIPE blocks. |
| END-POINT (1) | East, North, Up, Bore (Point=1) | — | MM | Truncate mm suffix. Transform coordinates. |
| END-POINT (2) | East, North, Up, Bore (Point=2) | — | MM | Truncate mm suffix. Transform coordinates. |
| ITEM-DESCRIPTION | — | 6" STD Pipe BE ASME B36.10M smls ASTM A106 GR B | — | Use Default. For isometric annotation only. |
| PIPING-SPEC | — | 1C1P | — | Use Default. |
| COMPONENT-ATTRIBUTE1 | Pressure | 700 [KPA] | KPA | Design pressure. Default if CSV blank. |
| COMPONENT-ATTRIBUTE2 | Temperature | 120 [C] | C | Design temperature. Default if CSV blank. |
| COMPONENT-ATTRIBUTE3 | Material | 106 | — | MAT_MAP lookup key. No unit suffix. |
| COMPONENT-ATTRIBUTE4 | Wall Thickness | 9.53 [MM] | MM | Always write. Never omit. "Undefined [MM]" if spec-driven. |
| COMPONENT-ATTRIBUTE5 | Insulation Thickness | 0 [MM] | MM | 0 = uninsulated. Always write. |
| COMPONENT-ATTRIBUTE6 | Insulation Density | 0 [KG/M3] | KG/M3 | Write when CA5 > 0. |
| COMPONENT-ATTRIBUTE7 | Corrosion Allowance | 3 [MM] | MM | Write when non-zero. |
| COMPONENT-ATTRIBUTE8 | Weight | 100 [KGS] | KGS | Pipe element weight. |
| COMPONENT-ATTRIBUTE9 | Fluid Density | 900 [KG/M3] | KG/M3 | Critical for hydrotest weight calc. |
| COMPONENT-ATTRIBUTE10 | Hydro Test Pressure | 1500 [KPA] | KPA | Hydrotest pressure. Default if CSV blank. |
| FABRICATION-ITEM | — | (blank) | — | Fixed keyword. No value. Always present. |

J.5  BEND  (CSV Type: ELBO  |  ISOGEN IDF: 13)

Elbow/bend. Two END-POINTs + one CENTRE-POINT (no bore on centre). ANGLE in actual degrees. BEND-RADIUS from CSV. SKEY=BEBW for project standard.

BEND

COMPONENT-IDENTIFIER  13

END-POINT  <East>  <North>  <Up>  <Bore>         ← Point=1 from CSV

END-POINT  <East>  <North>  <Up>  <Bore>         ← Point=2 from CSV

CENTRE-POINT  <East>  <North>  <Up>              ← no bore token

Skey  BEBW

ANGLE  <degrees>                                  ← actual degrees (e.g. 90, 45, 89.43)

BEND-RADIUS  <radius>                             ← from CSV Radius column (MM)

ITEM-DESCRIPTION  6" STD Elbow 90 deg BW ASME B16.9 LR smls ASTM A234 GR WPB

PIPING-SPEC  1C1P

COMPONENT-ATTRIBUTE1  <Pressure> [KPA]

COMPONENT-ATTRIBUTE2  <Temperature> [C]

COMPONENT-ATTRIBUTE3  <Material>

COMPONENT-ATTRIBUTE4  <WallThickness> [MM]

COMPONENT-ATTRIBUTE5  <InsulThickness> [MM]

COMPONENT-ATTRIBUTE10  <HydroPress> [KPA]

FABRICATION-ITEM

| PCF Attribute | CSV Source | Default Value | Unit (PCF) | Remarks |
| --- | --- | --- | --- | --- |
| BEND | — | — | — | Component type keyword. |
| COMPONENT-IDENTIFIER | — | 13 | — | Fixed IDF record = 13 for all BEND blocks. |
| END-POINT (1) | East, North, Up, Bore (Point=1) | — | MM | Truncate mm suffix. |
| END-POINT (2) | East, North, Up, Bore (Point=2) | — | MM | Truncate mm suffix. |
| CENTRE-POINT | East, North, Up (Point=0) | — | MM | 3 tokens only — NO bore on CENTRE-POINT. |
| Skey | — | BEBW | — | Butt weld elbow. Fixed for project standard. |
| ANGLE | Angle (degrees) | 90 | degrees | Actual degrees. NOT 100ths. e.g. 90 or 45 or 89.43. |
| BEND-RADIUS | Radius | 228.6 | MM | Explicit from CSV. Not computed by C2. |
| COMPONENT-ATTRIBUTE1–10 | (same as PIPE) | (same defaults) | — | Same CA rules as PIPE block. |
| FABRICATION-ITEM | — | (blank) | — | Fixed keyword. |

J.6  TEE  (CSV Type: TEE  |  ISOGEN IDF: 7)

Equal or reducing tee. Two END-POINTs + CENTRE-POINT (no bore) + BRANCH1-POINT (with bore). SKEY=TEBW for standard butt weld equal tee (no SIF map entry needed — geometric). SKEY=TESO for fabricated set-on branch (SIF map Type 1 required).

TEE

COMPONENT-IDENTIFIER  7

END-POINT  <East>  <North>  <Up>  <Bore>         ← Point=1 from CSV (run start)

END-POINT  <East>  <North>  <Up>  <Bore>         ← Point=2 from CSV (run end)

CENTRE-POINT  <East>  <North>  <Up>              ← Point=0, no bore

BRANCH1-POINT  <East>  <North>  <Up>  <BranchBore> ← Point=3 from CSV

Skey  TEBW

ITEM-DESCRIPTION  6" STD Tee equal BW ASME B16.9 smls ASTM A234 GR WPB

PIPING-SPEC  1C1P

COMPONENT-ATTRIBUTE1  <Pressure> [KPA]

COMPONENT-ATTRIBUTE2  <Temperature> [C]

COMPONENT-ATTRIBUTE3  <Material>

COMPONENT-ATTRIBUTE4  <WallThickness> [MM]

COMPONENT-ATTRIBUTE5  <InsulThickness> [MM]

COMPONENT-ATTRIBUTE10  <HydroPress> [KPA]

FABRICATION-ITEM

| PCF Attribute | CSV Source | Default Value | Unit (PCF) | Remarks |
| --- | --- | --- | --- | --- |
| TEE | — | — | — | Component type keyword. |
| COMPONENT-IDENTIFIER | — | 7 | — | Fixed IDF record = 7 for all TEE blocks. |
| END-POINT (1) | Point=1 coords | — | MM | Run pipe start end. |
| END-POINT (2) | Point=2 coords | — | MM | Run pipe end. |
| CENTRE-POINT | Point=0 coords | — | MM | 3 tokens only — NO bore. |
| BRANCH1-POINT | Point=3 coords + branch bore | — | MM | 4 tokens. Branch bore from CSV. |
| Skey | — | TEBW | — | TEBW for equal BW tee. TESO for set-on/stub-in. |
| COMPONENT-ATTRIBUTE1–10 | (same as PIPE) | (same defaults) | — | Same CA rules. |
| FABRICATION-ITEM | — | (blank) | — | Fixed keyword. |

J.7  FLANGE  (CSV Type: FLAN  |  ISOGEN IDF: 105)

Standard weld neck flange assumed as project default (FLWN). Two END-POINTs. FABRICATION-ITEM. Same CA rules as PIPE.

FLANGE

COMPONENT-IDENTIFIER  105

END-POINT  <East>  <North>  <Up>  <Bore>         ← Point=1

END-POINT  <East>  <North>  <Up>  <Bore>         ← Point=2

Skey  FLWN

ITEM-DESCRIPTION  6" 150# WN Flange RF ASME B16.5 ASTM A105

PIPING-SPEC  1C1P

COMPONENT-ATTRIBUTE1  <Pressure> [KPA]

COMPONENT-ATTRIBUTE2  <Temperature> [C]

COMPONENT-ATTRIBUTE3  <Material>

COMPONENT-ATTRIBUTE4  <WallThickness> [MM]

COMPONENT-ATTRIBUTE5  <InsulThickness> [MM]

COMPONENT-ATTRIBUTE10  <HydroPress> [KPA]

FABRICATION-ITEM

| PCF Attribute | CSV Source | Default Value | Unit (PCF) | Remarks |
| --- | --- | --- | --- | --- |
| FLANGE | — | — | — | Component type keyword. |
| COMPONENT-IDENTIFIER | — | 105 | — | IDF 105 = FLANGE. IDF 107 = FLANGE-BLIND. |
| END-POINT (1 & 2) | Point=1, Point=2 coords | — | MM | Same as PIPE. |
| Skey | — | FLWN | — | Weld neck = FLWN (default). See J.3 for full list. |
| ITEM-DESCRIPTION | — | 6" 150# WN Flange RF ASME B16.5 ASTM A105 | — | Use Default. |
| COMPONENT-ATTRIBUTE1–10 | (same as PIPE) | (same defaults) | — | Same CA rules. |
| FABRICATION-ITEM | — | (blank) | — | Fixed keyword. |

J.8  VALVE  (CSV Type: VALV  |  ISOGEN IDF: varies)

Ball valve, flanged (VBFL) confirmed as project standard. ERECTION-ITEM (not FABRICATION-ITEM) — valve is site-erected, not shop-fabricated. ITEM-DESCRIPTION from CSV componentName column.

VALVE

COMPONENT-IDENTIFIER  <IDF for VBFL>             ← verify from ISOGEN SKEY Guide

END-POINT  <East>  <North>  <Up>  <Bore>         ← Point=1

END-POINT  <East>  <North>  <Up>  <Bore>         ← Point=2

Skey  VBFL

ITEM-DESCRIPTION  <componentName from CSV>        ← Use CSV data

PIPING-SPEC  1C1P

COMPONENT-ATTRIBUTE1  <Pressure> [KPA]

COMPONENT-ATTRIBUTE2  <Temperature> [C]

COMPONENT-ATTRIBUTE3  <Material>

COMPONENT-ATTRIBUTE4  <WallThickness> [MM]

COMPONENT-ATTRIBUTE5  <InsulThickness> [MM]

COMPONENT-ATTRIBUTE10  <HydroPress> [KPA]

ERECTION-ITEM                                     ← NOT FABRICATION-ITEM

| PCF Attribute | CSV Source | Default Value | Unit (PCF) | Remarks |
| --- | --- | --- | --- | --- |
| VALVE | — | — | — | Component type keyword. |
| COMPONENT-IDENTIFIER | — | TBD | — | Verify from ISOGEN SKEY Guide for VBFL. See Gap G-J-01. |
| END-POINT (1 & 2) | Point=1, Point=2 coords | — | MM | Same as PIPE. |
| Skey | — | VBFL | — | Ball valve flanged. User confirmed. |
| ITEM-DESCRIPTION | componentName (CSV column) | — | — | Use CSV data — valve tag/description. |
| COMPONENT-ATTRIBUTE1–10 | (same as PIPE) | (same defaults) | — | Same CA rules. |
| ERECTION-ITEM | — | (blank) | — | NOT FABRICATION-ITEM. Valve = site erected. |

Gap G-J-01: COMPONENT-IDENTIFIER for VALVE (VBFL) not yet confirmed from ISOGEN SKEY Guide. Verify IDF record number before implementation. Add to Appendix I gap list.

J.9  OLET  (CSV Type: OLET  |  ISOGEN IDF: 2)

Branch fitting welded to side of main pipe. Uses CENTRE-POINT (on main pipe centreline) + BRANCH1-POINT (branch end). No END-POINTs — differs from all other component types. SIF map entry REQUIRED (Type 5 Weldolet). FABRICATION-ITEM.

OLET

COMPONENT-IDENTIFIER  2

CENTRE-POINT  <East>  <North>  <Up>  <MainBore>  ← on main pipe CL

BRANCH1-POINT  <East>  <North>  <Up>  <BranchBore> ← branch end

Skey  CEBW                                        ← weldolet BW (project standard)

ITEM-DESCRIPTION  4" x 6" Weldolet BW ASTM A105

PIPING-SPEC  PIL-RPADSPEC

COMPONENT-ATTRIBUTE1  <Pressure> [KPA]

COMPONENT-ATTRIBUTE2  <Temperature> [C]

COMPONENT-ATTRIBUTE3  <Material>

COMPONENT-ATTRIBUTE4  <WallThickness> [MM]

COMPONENT-ATTRIBUTE5  <InsulThickness> [MM]

COMPONENT-ATTRIBUTE10  <HydroPress> [KPA]

FABRICATION-ITEM

| PCF Attribute | CSV Source | Default Value | Unit (PCF) | Remarks |
| --- | --- | --- | --- | --- |
| OLET | — | — | — | Component type keyword. |
| COMPONENT-IDENTIFIER | — | 2 | — | IDF 2 confirmed from Hexagon PCF Reference. |
| CENTRE-POINT | East, North, Up, MainBore | — | MM | On main pipe centreline. 4 tokens with bore. |
| BRANCH1-POINT | East, North, Up, BranchBore | — | MM | Branch pipe end. 4 tokens with branch bore. |
| Skey | — | CEBW | — | Weldolet BW default. See J.3 for variants. |
| ITEM-DESCRIPTION | — | 4" x 6" Weldolet BW ASTM A105 | — | Use Default. |
| PIPING-SPEC | — | PIL-RPADSPEC | — | Reinforcement pad spec. Project standard. |
| COMPONENT-ATTRIBUTE1–10 | (same as PIPE) | (same defaults) | — | Same CA rules. |
| FABRICATION-ITEM | — | (blank) | — | Fixed keyword. |

SIF Map Rule (H-002): OLET SKEY must have entry in PCF_SIF_MAP.TXT → Type 5 (Weldolet). Unmatched SKEY → SIF=1.0 unconservative. Validate before every import.

J.10  SUPPORT  (CSV Type: ANCI  |  ISOGEN IDF: 82)

Pipe support or restraint. CO-ORDS (not END-POINT). ITEM-DESCRIPTION is the PCF_RES_MAP.TXT lookup key — must match exactly. SKEY=SKID (blank in samples). FABRICATION-ITEM and MATERIAL-IDENTIFIER always present.

SUPPORT

COMPONENT-IDENTIFIER  82                         ← IDF for SUPPORT

CO-ORDS  <East>  <North>  <Up>  <Bore>          ← 4 tokens incl bore (confirmed)

SUPPORT-DIRECTION  UP                            ← default. See H.4.5 for all values

UNIQUE-COMPONENT-IDENTIFIER  1223456-3440-4FCA-9930-2921DE5F0047

SKEY                                             ← blank (SKID in samples, blank in MAP)

FABRICATION-ITEM                                 ← blank keyword

MATERIAL-IDENTIFIER  3

ITEM-DESCRIPTION  <NodeName from CSV>            ← PCF_RES_MAP.TXT lookup key

SUPPORT-TYPE  PS_TEST1                           ← default

UCI  PS_TEST2                                    ← default

| PCF Attribute | CSV Source | Default Value | Unit (PCF) | Remarks |
| --- | --- | --- | --- | --- |
| SUPPORT | — | — | — | Component type keyword. |
| COMPONENT-IDENTIFIER | — | 82 | — | IDF 82 for SUPPORT (inferred). Verify. |
| CO-ORDS | East, North, Up, Bore | — | MM | 4 tokens including bore. Confirmed from samples. |
| SUPPORT-DIRECTION | TBA from CSV | UP | — | Default UP. From CSV restraint direction when available. |
| UNIQUE-COMPONENT-IDENTIFIER | — | 1223456-3440-4FCA- 9930-2921DE5F0047 | — | Use Default GUID. |
| SKEY | — | (blank) | — | Leave blank — SKID observed but not written. |
| FABRICATION-ITEM | — | (blank) | — | Blank keyword. Always present. |
| MATERIAL-IDENTIFIER | — | 3 | — | Use Default = 3. |
| ITEM-DESCRIPTION | NodeName (CSV) | — | — | CRITICAL: PCF_RES_MAP.TXT lookup key. Must match entry in map file exactly. |
| SUPPORT-TYPE | — | PS_TEST1 | — | Use Default. |
| UCI | — | PS_TEST2 | — | Use Default. |

Validator Rule H-001 (BLIND IMPORT): ITEM-DESCRIPTION on every SUPPORT block MUST match an entry in PCF_RES_MAP.TXT. Unmatched → silent RIGID ANCHOR (ANC). Cross-check all NodeName values against PCF_RES_MAP.TXT before import.

Appendix J sources: PCF_Mapping.csv (project master), ISOGEN SKEY Definitions Reference Guide v2016 (Alias/Hexagon), Hexagon PCF Reference Guide v13, PCF samples NG-151841 and NG-151843. Bold = confirmed. Normal = inferred. Gap G-J-01: VALVE IDF record pending verification.